3GPP TSG-RAN4  #97-e																	     	R4-2014263

Chongqing, CN, 14-18 Oct. 2019

Source: 	Qualcomm Incorporated

Title: 	Discussion on PC3 EIRP and EIS in n262

Agenda item:	10.28.1

Work item:	NR_47GHz_Band -Core

Document for:	Discussion

# Introduction

We previously shared our view on the prevailing performance trends on RF components as operating bands increased in frequency [1]. In this contribution we elaborate further on our expected performance for n262 (47.2 to 48.2 GHz).

# Discussion

## Peak EIRP

Our previous survey of performance trends suggested both, PA capability as well as antenna gain are expected to degrade, compared to say, n258 [1], see figures 2.1-1. It is possible however to contain the degradation to some degree by refinements in antenna module design and making design choices to reduces process variation.

Figure 2.1-1: Performance trends expected for n262 [1]

For reference, n260 requirement is 20.6 dBm (37-40 GHz) and n259 requirement is 18.7 dBm (39.5-43.5 GHz). Our preliminary budget for n262 is below.

| Parameter | Worst Case Value | units | Notes |
| --- | --- | --- | --- |
| # of elements in array | 4 |  |  |
| Pout per element | 8.0 | dBm | PVT corners |
| Gain per element | 0.4 | dBi |  |
| # of chains | 2 |  | Arranged to transmit in orthogonal polarizations |
| Implementation Losses | 4.0 | dB | Tx specific |
| Packaging Loss | 4.6 | dB |  |
| Peak EIRP | 14.8 | dBm |  |

Table 2.1-1: n262 min peak EIRP budget

## REFSENS

Our proposal continues to be based on rel. 15 assumptions for REFSENS. Note that REFSENS estimate is agnostic of misalignment between UE and TE axes. These details are captured in [2], for convenient reference. For reference, REFSENS requirement for 100MHz CC and -1 dB target SNR for n260 is -82.7 dBm (37-40 GHz) and n259 requirement is -81.7 dBm (39.5-43.5 GHz). Our preliminary budget for n262 is below.

| Parameter | Worst Case Value | units | Notes |
| --- | --- | --- | --- |
| # of elements in array | 4 |  |  |
| Element NF | 12.1 | dB | PVT corners |
| Gain per element | 0.4 | dBi |  |
| # of chains | 2 |  | Arranged to receive orthogonal polarizations, MRC |
| Implementation Losses | 3.0 | dB | Rx specific |
| Packaging Loss | 4.6 | dB |  |
| REFSENS | -81.5 | dBm | SNR Target = -1dB, 100MHz channel |

Table 2.2-1: n262 REFSENS budget

# Conclusion

We presented updated min peak EIRP and REFSENS estimates for n262, based on rel. 15 assumptions:

- Min. peak EIRP = 14.8 dBm

- REFSENS @ -1dB SNR and 100M = -81.5 dBm

# Reference

- R4-2011455, ‘Discussion on 48G RF components’, Qualcomm, RAN4#96-e, Aug 2020

- R4-1900671, ‘On FR2 EIS’, Qualcomm, RAN4#90, Athens, GR, Feb-Mar 2019