3GPP TSG-RAN4 Meeting #97-e	R4-2014281

# Online, 2-13 Nov., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1150 | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on R16 IDLE/INACTIVE RRM requirement with SMTC2-LP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The SMTC2-LP is missing in the R16 RRM requirement in IDLE/INACTIVE mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarification for SMTC configuration in the R16 RRM requirement in IDLE/INACTIVE mode when SMTC2-LP is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SMTC2-LP is missing in the R16 RRM requirement in IDLE/INACTIVE mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 4.2.2.2; 4.2.2.3; 4.2.2.4; 4.2.2.6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### 4.2.2.2	Measurement and evaluation of serving cell

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where:

M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second,

otherwise M1=1.

If the high layer signalling of smtc2-LP is configured in TS 38.331 [2], the SMTC periodicity follows smtc2-LP; otherwise the SMTC periodicity follows smtc.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

If the UE has evaluated according to Table 4.2.2.2-1 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].

Table 4.2.2.2-1: Nserv

| DRX cycle length [s] | Scaling Factor (N1) |  | Nserv [number of DRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |
| 0.32 | 1 | 8 | M1*N1*4 |
| 0.64 |  | 5 | M1*N1*4 |
| 1.28 |  | 4 | N1*2 |
| 2.56 |  | 3 | N1*2 |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. |  |  |  |

End of Change 1

Start of Change 2

#### 4.2.2.3 	Measurements of intra-frequency NR cells

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [1] within Tdetect,NR_Intra when that Treselection= 0 . An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1 or table 4.2.2.3-2) for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1 or table 4.2.2.3-2 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2 if the current serving cell is among them.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non zero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [1], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For UE not configured with highSpeedMeasFlag-r16, Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra are specified in Table 4.2.2.3-1. For UE configured with highSpeedMeasFlag-r16, Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra are specified in Table 4.2.2.3-2.

Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If the high layer signalling of smtc2-LP is configured in TS 38.331 [2], the SMTC periodicity follows smtc2-LP; otherwise the SMTC periodicity follows smtc. |  |  |  |  |  |

End of Change 2

Start of Change 3

#### 4.2.2.4	Measurements of inter-frequency NR cells

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [1] within Kcarrier * Tdetect,NR_Inter  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 or 6.5dB in FR2 for reselections based on ranking or 6dB in FR1 or 7.5dB in FR2 for SS-RSRP reselections based on absolute priorities or 4dB in FR1 and 4dB in FR2 for SS-RSRQ reselections based on absolute priorities. The parameter Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell. An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.3 for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier * Tmeasure,NR_Inter (see table 4.2.2.4-1) for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [1] within Kcarrier * Tevaluate,NR_Inter when Treselection = 0 as specified in table 4.2.2.4-1 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

when rangeToBestCell is not configured:

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 or.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 if the current serving cell is among them. or

-	6dB in FR1 or 7.5dB in FR2 for SS-RSRP reselections based on absolute priorities or

-	4dB in FR1 or 4dB in FR2 for SS-RSRQ reselections based on absolute priorities.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2.2.4-1 under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively, and

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS38.304 [1].

-	If the high layer signalling of smtc2-LP is configured in TS 38.331 [2], the above SMTC periodicity follows smtc2-LP; otherwise the SMTC periodicity follows smtc.

Table 4.2.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
| 0.64 |  | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. |  |  |  |  |  |

End of Change 3

Start of Change 4

#### 4.2.2.6	Maximum interruption in paging reception

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the high layer signalling of smtc2-LP is configured in TS 38.331 [2], the SMTC periodicity follows smtc2-LP; otherwise the SMTC periodicity follows smtc.

At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms.

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell.

TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell.

These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

End of Change 4