3GPP TSG-RAN WG4 Meeting #97-e                                        	R4-2014315

Online, 02nd – 13th November 2020

Source:	LG Electronics

Title:	TP on update of summary of self-interference analysis for new LTE x bands (xDL/1UL, x=1,2,3,4) and NR 2 bands (2DL/1UL) DC band combinations in Rel-17

Agenda item:			10.7.2

Document for:	Approval

1. Introduction

In this contribution, we provided self- desense analysis results for the new DC band combos in summary Table 5.3-1 and Proposed the MSD test configurations in Table 5.3-5.

2. Text Proposal for TR 37.717-11-21

<Start of TP>

5.3	Summary of self-interference analysis

Table 5.3-1 summarizes the EN- DC band combinations with self-interference problems for 3DL/2UL DC operation.

Table 5.3-1: Summary of Self-interference analysis for LTE 1 band & NR 2 bands DL and 2 bands UL DC operation

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_8_n28-n78 | DC_8A-n28 | 4th harmonic into n78 5th harmonic into n78 | 4th & 5th IMDs into n78 | - | - 4th harmonic problem will be solved in DC_8A-n78A - 5th harmonic problem will be solved in DC_28A-n78A - Same as DC_8_n28-n77A with 2UL_DC_8A_n28A |
|  | DC_8A_n78A | - | 4th IMD into n28 | - | - Same as DC_8_n28-n77A with 2UL_DC_8A_n77A |
| DC_(n)41AA-n78 DC_(n)41CA-n78 DC_(n)41DA-n78 | DC_41A_n78A | - | 4th IMD into n41 | - | B41 and n41 will be operated by synchronous operation. So there is no IMD problem |
| DC_40A_n1-n78 DC_40C_n1-n78 | DC_40A_n1A |  | 4th IMD & 5th IMD into n78 | - | - Same as DC_1A_n40A-n78A with 2UL_DC_1A_n40A |
|  | DC_40A_n78A |  | 4th IMD into n1 | - | - FFS |
| DC_3C_n1-n78 | DC_3A_n1A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n78A | - | 5th IMD into n1 | - | - Same as DC_3A_n1A-n78A with 2UL_DC_3A_n78A |
| DC_1_n28-n41 | DC_1A_n28A | - | 2nd & 4th IMDs into n41 |  | - Same as DC_1A-41A_n28A with 2UL_DC_1A_n28A |
|  | DC_1A_n41A |  | 2nd & 5th IMDs into n28 |  | - 29.3 dB by IMD2 - 4.5dB by IMD5 |
| DC_3_n28-n41 | DC_3A_n28A |  | 2nd & 3rd IMDs into n41 |  | - Same as DC_3A-41A_n28A with 2UL_DC_3A_n28A |
|  | DC_3A_n41A |  | 2nd & 3rd IMDs into n28 |  | - Same as DC_3A-28A_n41A with 2UL_DC_3A_n41A |
| DC_28_n1-n40 | DC_28A_n1A | - | 4th IMD into n40 | - | - Same as DC_1A_n28A-n40A with 2UL_DC_1A_n28A |
|  | DC_28A_n40A | 3rd harmonic from B28 into n1 | - | - | 3rd harmonic problem was covered in lower DC or CA band combinations |
| DC_7_n40-n78 | DC_7A_n40A | - | - | - | No issue |
|  | DC_7A_n78A |  | 4th IMD into n40 |  | - FFS |
| DC_8_n40-n78 | DC_8A_n40A | 4th harmonic from B8 into n78 | 2nd & 3rd IMDs into n78 |  | - FFS |
|  | DC_8A_n78A |  | 2nd IMD into n40 |  | - FFS |
| DC_1_n3-n77A DC_1_n3-n77(2A) | DC_1A_n3A | 2nd harmonic from B1 or n3 into n77 | 2nd & 4th IMDs into n77 | Yes | - Harmonic problems already covered in DC_n3A-n78A. - Same as IMD issues in DC_1A_n3A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_1A_n77A | - | 2nd , 4th & 5th IMDs into n3 | - | - Same as IMD issues in DC_1A_n3A-n78A |
| DC_8_n3-n77A DC_8_n3-n77(2A) | DC_8A_n3A | 2nd harmonic from n3 into n77 4th harmonic from B8 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_3A_n77A and DC_8A_n77A - Same as DC_3A_n8A-n78A with 2UL_DC_3A_n8A |
|  | DC_8A_n77A | 2nd harmonic from B8 into n3 | 3rd IMD into n3 |  | - Same as DC_3A-8A-n77A with 2UL_DC_8A_n77A |
| DC_11_n28-n77A DC_11_n28-n77(2A) | DC_11A_n28A | 5th harmonic from n28 into n77 | 3rd & 4th IMDs into n77 |  | - Harmonic problems already covered in CA_n28A_n77A - FFS |
|  | DC_11A_n77A | - | 3rd & 4th IMDs into n28 |  | - FFS |
| DC_42A_n3-n28 DC_42C_n3-n28 | DC_42A_n3A DC_42C_n3A | - | - |  | - No issue |
|  | DC_42A_n28A DC_42C_n28A | - | - |  | - No issue |
| DC_42A_n3-n77A DC_42A_n3-n77(2A) DC_42C_n3-n77A DC_42C_n3-n77(2A) | DC_42A_n3A DC_42C_n3A | 2nd harmonic from n3 into n77 | 4th & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_n3A_n77A - These IMD problem was not impact to n77 due to synchronous TDD operation |
| DC_2_n5-n77 or DC_2A-2A_n5A-n77A | DC_2A_n5A | 2nd harmonic from B2 into n77 5th harmonic from n5 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems already covered in DC_2A_n77A or CA_n5A-n77A. - FFS |
|  | DC_2A_n77A | - | 5th IMD into n5 |  | - FFS |
| DC_13_n2-n77 | DC_13A_n2A | 2nd harmonic from n2 into n77 5th harmonic from B13 into n77 | 3rd, 4th & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_n2A-n77A or DC_13A_n77A. - FFS |
|  | DC_13A_n77A | - | 3rd IMD into n2 |  | - FFS |
| DC_66_n2-n77 | DC_66A_n2A | 2nd harmonic from B66 or n2 into n77 | 2nd & 4th IMDs into n77 | - | - Same as DC_2A_n66A-n78A with 2UL_DC_2A_n66A |
|  | DC_66A_n77A | - | 2nd , 4th & 5th IMDs into n2 | - | - Same as DC_2A-66A_n78A with 2UL_DC_66A_n78A |
| DC_66_n5-n77 DC_66-66_n5-n77 | DC_66A_n5A | 2nd harmonic from B66 into n77 4th & 5th harmonics from n5 into n77 | 3rd, 4th & 5th IMDs into n77 | - | - These harmonic issue already solved in DC_66A_n77A or DC_5A_n77A - FFS |
|  | DC_66A_n77A | - | 2nd & 5th IMDs into B66 | - | - These IMD problem already solved in DC_66A_n77A |
| DC_28_n1-n78 | DC_28A_n1A | 5th harmonic from 28 into n78 | 3rd IMD into n78 | - | - Harmonic problems already solved in CA_28A-n78A - Same as DC_1A_n28A-n78A with 2UL_DC_1A_n28A |
|  | DC_28A_n78A | 3rd harmonic from B28 into n1 | 3rd IMD into n1 | - | - 3rd harmonic will be covered in CA_1A-28A - Same as DC_1A-28A_n78A with 2UL_DC_28A_n78A |
| DC_2_n48-n66 | DC_2A_n48A | - | 4th IMD into n66 | - | - Same as DC_2A-66A_n48A with 2UL_DC_2A_n48A |
|  | DC_2A_n66A | 2nd harmonic from n66 into n48 | 2nd IMD into n48 | - | - Same as DC_2A-48A_n66A with 2UL_DC_2A_n66A |
| DC_48_n25-n48 | DC_48A_n25A | - | 4th IMD into n48 |  | There was no IMD problem due to TDD n48 |
| DC_48_n48-n66 | DC_48A_n66A | 2nd harmonic from n66 into n48 | 4th IMD into n48 |  | There was no IMD problem due to TDD n48 |
| DC_66_n25-n48 | DC_66A_n25A | 2nd harmonic from n66 into n48 | 2nd IMD into n48 | - | - Same as DC_2A-48A_n66A with 2UL_DC_2A_n66A |
|  | DC_66A_n48A | - | 2nd IMD into n25 | - | - Same as DC_2A-48A_n66A with 2UL_DC_48A_n66A |
| DC_3_n40-n41C | DC_3A_n40A | 3rd harmonic from B3 into n41 | - | - | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_3A_n41A | - | - | Yes | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_3_n40-n79C | DC_3A_n40A | 2nd harmonic form n40 into n79 | 5th IMD into n79 | - | - Same as DC_3A_n40A-n79A with 2UL_DC_3A_n40A |
|  | DC_3A_n79A | - | 5th IMD into n40 | - | - Same as DC_3A_n40A-n79A with 2UL_DC_3A_n79A |
| DC_3_n41A-n79C DC_3_n41C-n79A DC_3_n41C-n79C | DC_3A_n41A | 2nd harmonic form n41 into n79 | 2nd & 5th IMDs into n79 | Yes | - The 2nd harmonic problem will be studied in CA_n41-n79. - Same as DC_3A_n41A-n79A with 2UL_DC_3A_n41A - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
|  | DC_3A_n79A | - | 2nd & 5th IMDs into n41 | - | - Same as DC_3A_n41A-n79A with 2UL_DC_3A_n79A |
| DC_8_n40-n41C | DC_8A_n40A | 3rd harmonic from B8 into n41 | - | - | - Harmonic problem will be studied in DC_8A_n41A. - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_8A_n41A | - | 5th IMD into n40 | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_8_n40-n79C | DC_8A_n40A | 2nd harmonic from n40 into n79 5th harmonic from B8 into n79 | 4th IMD into n79 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n40A-n79A with 2UL_DC_8A_n40A |
|  | DC_8A_n79A | - | 4th IMD into n40 | - | - Same as DC_8A_n40A-n79A with 2UL_DC_8A_n79A |
| DC_8_n41A-n79C DC_8_n41C-n79A DC_8_n41C-n79C | DC_8A_n41A | 2nd harmonic from n41 into n79 5th harmonic from B8 into n79 | 3rd IMD into n79 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n41A-n79A with 2UL_DC_8A_n41A |
|  | DC_8A_n79A | 3rd harmonic from B8 into n41 | 3rd IMD into n41 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n41A-n79A with 2UL_DC_8A_n79A |
| DC_39_n40-n41C | DC_39A_n40A | - | 3rd IMD into n41 | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_39A_n41A | - | - | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_39_n40-n79C | DC_39A_n40A | 2nd harmonic from n40 into n79 | 4th IMD into n79 | - | - The harmonic problem will be treated CA_n40-n79 band combos. - Same as DC_39A_n40A-n79A with 2UL_DC_39A_n40A |
|  | DC_39A_n79A | - | - | - | No issue |
| DC_39_n41A-n79C DC_39_n41C-n79A DC_39_n41C-n79C | DC_39A_n41A | 2nd harmonic from n41 into n79 | 2nd & 5th IMDs into n79 | - | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - Same as DC_39A_n41A-n79A with 2UL_DC_39A_n41A |
|  | DC_39A_n79A | - | 2nd & 5th IMDs into n41 | - | - Same as DC_39A_n41A-n79A with 2UL_DC_39A_n79A |
| DC_13_n5-n48 | DC_13A_n48A | - | - | - | No issue |
| DC_13_n48-n66 | DC_13A_n48A | - | 3rd IMD into n66 | - | Follow 17.1dB MSD for CA_13A-48A-66A in TS36.101 |
|  | DC_13A_n66A | 2nd harmonic from n66 into n48 | 5th IMD into n48 | - | - 2nd harmonic issue will be solved in CA_n48-n66 - In rel-16 TR, define as 2.8 dB |
| DC_66_n5-n48 | DC_66A_n5A | 2nd harmonic from n66 into n48 | 5th IMD into n48 | - | - 2nd harmonic issue will be solved in CA_n48-n66 - In rel-16 TR, define as 3.3 dB |
|  | DC_66A_n48A | - | - | - | No issue |
| DC_11_n3-n28 | DC_11A_n3A | - | 5th IMD into n28 |  | - In rel-16 TR, define as 2.5 dB |
|  | DC_11A_n28A | - | - | - | No issue |
| DC_3_n1-n257 DC_3-3_n1-n257 | DC_3A_n1A | - | - | Yes | No issue |
|  | DC_3A_n257A | - | - | - | No issue |
| DC_7_n1-n257 DC_7-7_n1-n257 | DC_7A_n1A | High order harmonics from B7 or n1 | - | - | No issue |
|  | DC_7A_n257A | - | - | - | No issue |
| DC_3_n78-n258 | DC_3A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_3A_n258A | 2nd harmonic from B3 into B78 | - | - | - Harmonic problem already covered in DC_3A_n78A |
| DC_7A_n78-n258 DC_7C_n78-n258 | DC_7A_n78A DC_7C_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_7A_n258A DC_7C_n258A | - | - | - | - No issue |
| DC_28_n78-n258 | DC_28A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_28A_n258A | 5th harmonic from B28 into n78 | - | - | - Harmonic problem already covered in DC_28A_n78A |
| DC_8_n78-n258 | DC_8A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n258A | 4th harmonic from B8 into n78 | - | - | - Harmonic problem already covered in DC_8A_n78A |
| DC_8_n40-n258 | DC_8A_n40A | 11th harmonics from n40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | - | - | - | - No issue |
| DC_3_n34-n258 | DC_3A_n34A | 14th & 15th harmonics from B3 into n258 12th & 13th harmonics from n34 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_3A_n258A | - | - | - | - No issue |
| DC_3_n41-n258 | DC_3A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_3A_n258A | - | - | - | - No issue |
| DC_8_n34-n258 | DC_8A_n34A | 12th & 13th harmonics from n34 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | - | - | - | - No issue |
| DC_8_n41-n258 | DC_8A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | 3rd harmonic from B8 into n41 | - | - | - Harmonic problem already covered in DC_8A_n41A |
| DC_39_n40-n258 | DC_39A_n40A | 11th harmonics from n40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_39_n41A-n258 DC_39_n41C-n258 | DC_39A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_8_n79-n258 | DC_8A_n79A | 5th & 6th harmonic from n79 into n258 | - |  | - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_8A_n258A | 5th harmonic from B8 into n79 | - | - | - Harmonic problem already covered in DC_8A_n79A |
| DC_39_n79A-n258 DC_39_n79C-n258 | DC_39A_n79A | 5th & 6th harmonic from n79 into n258 | - |  | - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_40_n41A-n258 DC_40_n41C-n258 | DC_40A_n41A | 10th harmonic from n41 into n258 11th harmonics from B40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_40A_n258A | - | - | - | - No issue |
| DC_40_n79A-n258 DC_40_n79C-n258 | DC_40A_n79A | 11th harmonics from B40 into n258 5th & 6th harmonic from n79 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_40A_n258A | 2nd harmonic from B40 into n79 |  |  | - These harmonic problem will be covered in DC_40A_n79A |
| DC_41_n79A-n258 DC_41_n79C-n258 | DC_41A_n79A | 10th harmonic from n41 into n258 5th & 6th harmonic from n79 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_41A_n258A | 2nd harmonic from n41 into n79 |  |  | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used |
| DC_8_n77(2A)-n257 | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I | 4th harmonic from B8 into n77 | - | - | - Harmonic problem already covered in DC_8A_n77A |
| DC_11_n77(2A)-n257 | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | No harmonic problem to n257 |
|  | DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I | - | - | - | No issue |
| DC_28_n77(2A)-n257D | DC_28A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | No harmonic problem to n257 |
|  | DC_28A_n257D | 5th harmonic from B28 into n77 | - | - | 5th harmonic problem was solved in DC_28A_n77A |
| DC_2_n7A-n66A DC_2_n7(2A)-n66A | DC_2A_n7A | - | - | - | No issue |
|  | DC_2A_n66A | - | - | - | No issue |
| DC_12_n7A-n66A DC_12_n7(2A)-n66A | DC_12A_n7A | 3rd harmonic into n66 from B12 | - | - | The 3rd harmonic issue already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | - | - | No issue |
| DC_2_n66A-n78(2A) DC_2_n66(2A)-n78A DC_2_n66(2A)-n78(2A) | DC_2A_n66A | 2nd harmonics into n78 from B2 and n66 | 2nd & 4th IMDs into n78 | - | The 2nd harmonic issue already covered in DC_66A_n78A - Follow same as DC_2A_n66A-n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_2A_n78A | - | 4th IMD into n66 | - | - For 4th IMD problem, follow same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_12_n66A-n78A DC_12_n66A-n78(2A) DC_12_n66(2A)-n78A DC_12_n66(2A)-n78(2A) | DC_12A_n66A | 2nd harmonic into n78 from n66 5th harmonic into n78 from B12 | 5th IMD into n78 | - | These harmonic issues are already covered in DC_66A_n78A or DC_12A_n78A - 4.1dB by 5th IMD in TR37.716-21-21 |
|  | DC_12A_n78A | 3rd harmonic into n66 from B12 | 3rd IMD into n66 | - | 3rd harmonic issue already covered in DC_12A_n66A - 16.5dB by 3rd IMD in TR37.716-21-21 |
| DC_13_n7-n78 | DC_13A_n7A | - | 2nd & 4th IMDs into n78 | - | - 29.0 dB by 2nd IMD already analyzed in TR37.716-21-21 |
|  | DC_13A_n78A |  | 2nd IMD into n7 | - | - 27.9 dB by 2nd IMD already analyzed in TR37.716-21-21 |
| DC_2_n38-n66 | DC_2A_n38A | - | - | - | No issue |
|  | DC_2A_n66A | - | - | - | No issue |
| DC_66_n38-n66 | DC_66A_n38A | - | - | - | No issue |
|  | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
| DC_1_n3-n77 | DC_1A_n3A | 2nd harmonic from n3 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  | DC_1A_n77A |  | 2nd, 4th & 5th IMDs into B3 | Yes | These MSD issues are same as DC_1A-3A_n77A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
| DC_1-3_n3-n77 or DC_1-3_n3-n78 | DC_1A_n3A | 2nd harmonic from n3 into n77/n78 | 2nd & 4th IMDs into n77/n78 |  | - Harmonic problems was already covered in DC_3A_n77A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  | DC_1A_n77A or DC_1A_n78A |  | 2nd, 4th & 5th IMDs into B3 | Yes | These MSD issues are same as DC_1A-3A_n77A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n3A | 2nd harmonic from n3 into n77/n78 | 2nd & 4th IMD into n77/n78 | - | - Harmonic problems was already covered in DC_3A_n77A. - Due to apply MPR, no need to define MSD |
|  | DC_3A_n77A or DC_3A_n78A | - | 2nd & 5th IMD into B1 2nd, 4th &5th IMD into n3 | - | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. - MSD problems in n3 were already solved in DC_3A_n77A |
| DC_18_n28-n77 or DC_18_n28-n78 | DC_18A_n28A | 4th & 5th harmonic from B18 and n28 fall into n77/n78 | 5th IMD into n77/n78 | - | - Harmonic problems was already covered in DC_18A_n77A or CA_n28-n77 - FFS |
|  | DC_18A_n77A or DC_18A_n78A | - | 5th IMD into n77 | - | - FFS |
| DC_1_n3-n41 | DC_1A_n3A | - | 5th IMD into n41 | - | - 5.0 dB by 5th IMD that follow DC_1A-41A_n3A in TS38.101-3 |
|  | DC_1A_n41A | - | - | Yes | Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
| DC_3_n3-n41 | DC_3A_n3A | - | - | - | No issue |
|  | DC_3A_n41A | - | 4th IMD into n3 | - | 4th IMD issue already covered by DC_3_n41 in Table 7.3B.2.3.5.1-1 in TS38.101-13 |
| DC_18_n3-n41 | DC_18A_n3A | - | 2nd & 3rd IMDs into B41 | - | 3rd IMD problem already covered in DC_18A-41A_n3A - FFS for 2nd IMD |
|  | DC_18A_n41A | - | 2nd IMD into n3 |  | - FFS |
| DC_1-3_n3-n41 | DC_1A_n3A | - | 5th IMD into n41 |  | 5th IMD problem already covered in CA_1A-3A-41A in TS36.101 |
|  | DC_1A_n41A | - | - | - | No issue |
|  | DC_3A_n3A | - | - | - | No issue |
|  | DC_3A_n41A | - | - | - | No issue |
| DC_3_n28-n41 | DC_3A_n28A | - | 2nd & 3rd IMDs into n41 | - | These IMD problem already covered in DC_3A-41A_n28A |
|  | DC_3A_n41A | - | 2nd & 3rd IMDs into n28 |  | The IMD problem already covered in DC_3A-28A_n41A |
| DC_18_n28-n41 | DC_18A_n28A |  | 5th IMD into n41 | - | - FFS |
|  | DC_18A_n41A |  | 5th IMD into n28 | - | - FFS |
| DC_1_n41-n77 | DC_1A_n41A | 2nd harmonic from B1 into n77 | 3rd , 4th & 5th IMDs into n77 | - | - The harmonic problem already covered in DC_1_n77 in TS38.101-3 - These IMDs problem follow DC_1_n41-n78 in TS38.101-3 |
|  | DC_1A_n77A | - | 4th & 5th IMDs into n41 | - | - The IMD problem follow DC_1-41_n77 in TS38.101-3 |
| DC_3_n41-n77 | DC_3A_n41A | 2nd harmonic from B3 into n77 | 3rd & 5th IMDs into n77 | - | - The harmonic problem already covered in DC_3_n77 in TS38.101-3 - These IMDs problem follow DC_3_n41-n78 in TS38.101-3 |
|  | DC_3A_n77A |  | 5th IMD into n41 |  | - The IMD problem follow DC_3-41_n77 in TS38.101-3 |
| DC_18_n41-n77 or DC_18_n41-n78 | DC_18A_n41A | 4th harmonic from B18 into n78 | 2nd & 4th IMDs into n77/n78 | - | - FFS |
|  | DC_18A_n77A or DC_18A_n78A |  | 2nd IMD into n4177/n78 3rd IMD into n41 in DC_18A_n77A | - | - FFS |
| DC_41_n3-n41 | DC_41A_n3A | - | 4th IMD into n41 | - | For TDD band, the IMD problem is not impact to the 3rd Rx band |
|  | DC_41A_n41A | - | - | - | No issue |
| DC_41_n28-n41 | DC_41A_n28A | - | - | - | No issue |
|  | DC_41A_n41A | - | - | - | No issue |
| DC_41_n41-n78 | DC_41A_n41A | - | - | - | No issue |
|  | DC_41A_n78A | - | 4th IMD into n41 |  | For TDD band, the IMD problem is not impact to the 3rd Rx band |
| DC_(n)5AA-n77A | DC_(n)5AA | 4th & 5th harmonics from n5 into n77 | 4th & 5th IMD into n77 |  | 4th & 5th harmonic issues were covered in DC_5A_n77A The 4th & 5th IMD problem will not be impacted due to only single UL switching operation. |
|  | DC_5A_n77A |  | 4th & 5th IMDs |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A |
| DC_12_n5-n77 | DC_12A_n5A | 4th & 5th harmonics from n5 into n77 | 5th IMD |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A - FFS |
|  | DC_12A_n77A |  | 5th IMD |  | - FFS |
| DC_14_n5-n77 | DC_14A_n5A | 4th & 5th harmonics from n5 into n77 | 4th & 5th IMD |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A - FFS |
|  | DC_14A_n77A |  | 5th IMD |  | - FFS |
| DC_2_n71-n78 | DC_2A_n71A | 2nd harmonic from B2 into n78 5th harmonic from n71 into n78 | 3rd IMD into n78 | - | These harmonics issues were already solved in DC_2_n78 or CA_n71_n78 - FFS |
|  | DC_2A_n78A | - | - | - | No issue |
| DC_7_n71-n78 | DC_7A_n71A | 5th harmonic from n71 into n78 | 4th IMD into n78 | - | - The harmonic issue was already solved in CA_n71_n78 - FFS |
|  | DC_7A_n78A | - | 5th IMD into n71 | - | - FFS |
| DC_66_n71-n78 | DC_66A_n71A | -2nd harmonic from B66 into n78 5th harmonic from n71 into n78 | 4th & 5th IMDs into n78 | - | Harmonic issues were already solved in DC_66_n78 or CA_n71_n78 - MSD of 4th IMD can be reused in DC_48A-66A_n71A in TS38.101-3 |
|  | DC_66A_n78A | - | - | - | No issue |
| DC_2_n38-n71 | DC_2A_n38A | - | - | - | No issue |
|  | DC_2A_n71A | - | 2nd IMD into n38 | - | 29.2 dB of DC_2A_n41A-n71A can be reused in TS38.101-3 |
| DC_66_n38-n71 | DC_66A_n38A | - | - | - | No issue |
|  | DC_66A_n71A | - | - | - | No issue |
| DC_2_n38-n78 | DC_2A_n38A | 2nd harmonic from B2 | 3rd IMD into n78 |  | Harmonic issue were covered in DC_2A_n78A - 14.8 dB was captured in TR37.716-21-21 in Rel-16 |
|  | DC_2A_n78A | - | - | - | No issue |
| DC_66_n38-n78 | DC_66A_n38A | 2nd harmonic from B66 | 3rd IMD into n78 | - | Harmonic issue were covered in DC_66A_n78A - 15.0 dB was captured in TR37.716-21-21 in Rel-16 |
|  | DC_66A_n78A | - | - | - | No issue |
| DC_71_n38-n78 | DC_71A_n38A | 5th harmonic from B71 into n78 | 2nd & 4th IMDs into n78 |  | Harmonic issue were covered in DC_71A_n78A - FFS |
|  | DC_71A_n78A |  | 2nd IMD into n38 |  | - FFS |
| DC_5_n66-n78 | DC_5A_n66A | -2nd harmonic from B66 into n78 4th harmonic from B5 into n78 | 3rd & 5th IMDs into n78 |  | Harmonic issue were covered in DC_66A_n78A or DC_5A_n78A - Same MSD levels can be reused in DC_66_n5-n77 in this TR |
|  | DC_5A_n78A | - | 3rd IMD into n66 |  | 13.2 dB by DC_5A-66A_n78A can be reused in TS38.101-3 |
| DC_71_n66-n78 | DC_71A_n66A | -2nd harmonic from n66 into n78 5th harmonic from B71 into n78 | 4th & 5th IMDs into n78 | - | Harmonic issues were already solved in DC_2_n78 or CA_n71_n78 - MSD of 4th IMD can be reused in DC_48A-66A_n71A in TS38.101-3 |
|  | DC_71A_n78A |  | 3rd IMD into n66 | - | - FFS |
| DC_2_n38-n66 | DC_2A_n38A | - | - | - | No issue |
|  | DC_2A_n66A | - | - | - | No issue |
| DC_5_n38-n66 | DC_5A_n38A | - | - | - | No issue |
|  | DC_5A_n66A |  | 2nd & 3rd IMD into n38 |  | - FFS |
| DC_71_n38-n66 | DC_71A_n38A | - | - | - | No issue |
|  | DC_71A_n66A | - | - | - | No issue |
| DC_2_n2-n38 or DC_2_n2-n41 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n38A or DC_2A_n41A | - | - | - | No issue |
| DC_66_n2-n38 | DC_66A_n2A | - | - | - | No issue |
|  | DC_66A_n38A | - | - | - | No issue |
| DC_12_n2-n38 or DC_12_n2-n41 | DC_12A_n2A |  | 2nd IMD into n38/n41 | - | - FFS |
|  | DC_12A_n38A or DC_12A_n41A | - | - | - | No issue |
| DC_71_n2-n41 | DC_71A_n2A | 4th harmonic from B71 into n41 | 2nd IMD into n41 | - | Harmonic issue already covered in DC_71_n41 - 29.2dB MSD by DC_2A_n41A-n71A can be reused in TS38.101-3 |
|  | DC_71A_n41A | 3rd harmonic from B71 into n2 | 2nd IMD into n2 |  | Harmonic issue already covered in DC_71_n2 - 26.0dB MSD by DC_2A-71A_n38A can be reused in TS38.101-3 |
| DC_2_n2-n78 | DC_2A_n2A | 2nd harmonic from B2 into n78 | 2nd & 4th IMDs into n78 |  | DC_2A_n2A only allowed single UL switched mode. |
|  | DC_2A_n78A |  | 2nd,4th & 5th IMDs into n2 |  | The IMD problem already solved in DC_2A_n78A in TS38.101-3 |
| DC_7_n2-n78 | DC_7A_n2A | 2nd harmonic from n2 into n78 | 5th IMD into n78 |  | Harmonic issue already covered in DC_2_n78 4.2dB MSD of DC_2A_n7A-n78A can be reused in TS38.101-3 |
|  | DC_7A_n78A | - | 4th IMD into n2 |  | 8.6dB MSD of DC_2A-7A_n78A can be reused in TS38.101-3 |
| DC_71_n2-n78 | DC_71A_n2A | 2nd harmonic from n2 into n78 5th harmonic from B71 into n78 | 3rd IMD into n78 |  | Harmonic issue already covered in DC_2_n78 or DC_71_n78 - 3rd IMD can be reused in DC_2_n71-n78 in this TRFFS |
|  | DC_71A_n78A | 3rd harmonic from B71 into n2 | 3rd IMD into n2 |  | Harmonic issue already covered in DC_71_n2 - 16.5dB MSD of DC_2A-71A_n78A can be reused in TS38.101-3 |
| DC_2_n66-n71 | DC_2A_n66A | - | - | - | No issue |
|  | DC_2A_n71A | - | - | - | No issue |
| DC_7_n66-n71 | DC_7A_n66A | - | - | - | No issue |
|  | DC_7A_n71A | - | - | - | No issue |
| DC_66_n66-n71 | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
|  | DC_66A_n71A |  | 4th IMD into n66 |  | 5.0dB MSD for DC_66A_n71A can be reused in TS38.101-3 |
| DC_2_n2-n71 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n71A | 3rd harmonic from n71 into B2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_7_n2-n71 | DC_7A_n2A |  | 2nd & 5th IMDs into n71 |  | 28.7dB MSD for DC_2A_n41A-n71A can be reused in TS38.101-3 |
|  | DC_7A_n71A | 3rd harmonic from n71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_66_n2-n71 | DC_66A_n2A | - | - | - | No issue |
|  | DC_66A_n71A | 3rd harmonic from n71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_2_n2-n66 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n66A |  | 3rd & 5th IMDs into n2 |  | 20.0dB MSD of DC_2A_n66A can be reused in TS38.101-3 |
| DC_7_n2-n66 | DC_7A_n2A | - | - | - | No issue |
|  | DC_7A_n66A | - | - | - | No issue |
| DC_66_n2-n66 | DC_66A_n2A |  | 5th IMD into n66 |  | 4.0dB MSD of DC_2A_n66A can be reused in TS38.101-3 |
|  | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
| DC_71_n2-n66 | DC_71A_n2A | - | - | - | No issue |
|  | DC_71A_n66A | 3rd harmonic from B71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_2_n66-n77 or DC_2-2_n66-n77 | DC_2A_n77A | - | 2nd , 4th & 5th IMDs into n66 | - | - FFS for 2nd IMD - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_13_n66-n77 | DC_13A_n77A |  | 3rd IMD into n66 |  | - 17.1dB MSD for DC_13-66A_n48A can be reused in TS38.101-3 |
|  | DC_13A_n66A | 2nd harmonic from n66 into n77 5th harmonic from B13 into n77 | 3rd, 4th & 5th IMDs into n77 | - | -Harmonic issue will be solved in CA_n66-n77 or DC_13_n77 - FFS |
| DC_66_n66-n77 | DC_66A_n77A | - | 2nd & 5th IMDs into B66 | - | - These IMDs problems already covered in DC_66A_n77A |
| DC_11_n3-n77 or DC_11_n3-n77(2A) | DC_11A_n3A | 2nd harmonic from n3 into n77 | 4th & 5th IMDs into n77 | - | Harmonic issues already solved in DC_3_n77 - FFS |
|  | DC_11A_n77A |  | 2nd & 5th IMDs into n3 |  | - FFS |

For the high order xDL/2UL (x=4,5,6) DCcombinations, the self-interference problems will be covered in 3DL/2UL or lower order DC operation. Hence, RAN4 do not make summary tables for high order xDL/2UL DC band combinations.

Based on these assumptions, we proposed the MSD test configuration and required MSD levels by self-interference as below.

Table 5.3-2: Proposed MSD levels and MSD test configuration for 3DL/2UL DC UE

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_XA_nYA-nZA | X | IMD order | ¦fBX -fnY¦ |  |  |  |  |  |  | N/A |
|  | nY |  |  |  |  |  |  |  |  |  |
|  | nZ |  |  |  |  |  |  |  |  | x.x dB |

<End of TP>