3GPP TSG-RAN4 Meeting #97-e												     			R4-2014320

Electric Meeting, 02nd - 13th November, 2020

Source: 	LG Electronics

Title: 	Enhanced beam correspondence test applicability rules in rel-16

Agenda item:	7.12.1.1

Document for:	Approval

Introduction

In last RAN4 #96-e meeting, one WF [1] on enhanced BC approved for SSB based eBC and CSI-RS based BC. However, RAN4 do not make consensus on the test applicability rule when UE support both eBC capability in Rel-16. The initial CR on eBC was agreed the test applicability rule is still TBD [2].

- Issue 1. SSB-Based BC enhancement

    - Issue 1-1-1: Whether and how much performance relaxation, ∆p, relative to the condition which assumes both SSB and CSI-RS are present, is necessary

      - Alt 1-1-1-1: ∆p = 0 dB (compared to R15 bit 1 or bit 0 UE?)

      - Alt 1-1-1-2: 0 < ∆p ≤ 3 dB

      - Alt 1-1-1-3: ∆p = 0 dB is optional to support, and 0 < ∆p ≤ 3 dB is the baseline to support

    - Agreements:

      - Alt 1-1-1-1: ∆p = 0 dB, an optional capability

      - If a R16 UE indicates it doesn’t support BC based on SSB, network can expect the UE to fulfill BC based on R15 BC requirements.

      - R16 will continue to have bit 1 and bit 0 UEs

- Issue 2. CSI-RS Based BC enhancement

    - Issue 2-1-1: The method to achieve CSI-RS based condition which will use SSB for P1 procedure

      - Alt 2-1-1-1: SSB and CSI-RS are present, but SSB’s PSD is backed-off by X dB from CSI-RS

      - Alt 2-1-1-2: decrease SSB power until UE SSB based SS-SINR measurement reporting is ≤ [-3] dB

      - Alt 2-1-1-3: as proposed in R4-2010119 (Qualcomm)

      - Alt 2-1-1-4: as proposed in R4-2010198 (Samsung)

      - Alt 2-1-1-5: Update side conditions as proposed in R4-2011479 (Huawei) and take Alt 2-1-1-1

      - Alt 2-1-1-6: Further study is needed

    - Agreements:

      - Continue to focus on Alt 2-1-1-1 with all relevant details including the value of X and other P1 CSI-RS configurations to be worked out

- Issue 3. Capability aspects

    - Issue 3-1-1: How to introduce capability signaling for enhanced BC requirements

      - Alt 3-1-1-1: A single unified capability for both BC based on SSB and BC based on CSI-RS (and introduce requirements for both)

      - Alt 3-1-1-2: Separate capabilities for BC based on SSB and BC based on CSI-RS (and introduce requirements for both)

      - Alt 3-1-1-3: A single capability only for BC based on SSB (and introduce requirements for BC based on SSB only)

    - Agreements:

      - Alt 3-1-1-2: Separate capabilities for BC based on SSB and BC based on CSI-RS (and introduce requirements for both)

      - R16 will continue to have bit 1 and bit 0 UEs for BC based on CSI-RS

      - BC based on CSI-RS is optional

- Issue 4. Test applicability rules for both eBC supporting UE

    - Agreements:

      - If a UE meets beam correspondence requirements either based on SSB or based on CSI-RS, it is considered to have met the beam correspondence requirements based on SSB and CSI-RS.

      - Additional applicability rules TBD

In this contribution, we share our views to complete the test applicability rule when UE support both eBC capability in rel-16.

Discussion

For the test applicability rule, RAN4 discussed at last RAN4 meeting with two alternative solutions as follow

- Option1: If a UE meets beam correspondence requirements based on SSB, it is considered to have met the beam correspondence requirements based on CSI-RS.

- Option2: If the UE supports both types of BC, one UL representative test (e.g. min peak EIRP testing) using the second side-condition set is additionally conducted

These two options for test applicability rule for both eBC supporting UE in Rel-16.

For the RRM measurements accuracy aspect, we analyse each SSB or CSI-RS side conditions in TS38.331 as follow

In RRM specification, RAN4 already agreed to same side condition and almost same number of RE for SSB (127 REs) or CSI-RS (144 REs) measurements to specify same measurements accuracy with 6.5 dB for SSB or CSI-RS based beam management when SSB Es/Iot and CSI-RS Es/Iot with -3dB as following Table 10.1.20.1.1-1 and Table 10.1.20.2.1-1 in TS38.331 v16.5.0.

Observation 1: In RRM measurement accuracy requirements, they specified same measurement accuracy requirements as 6.5 dB for each SSB based or CSI-RS based beam management with same side conditions.

In RF session, RAN4 updated the side condition for eBC requirements with single RS configuration in Rel-16 as follow.

These side condition can guarantee quite good signal quality compare to RRM measurement accuracy test condition even though only configure the single RS for eBC requirements in Rel-16.

When the UE satisfy same measurement accuracy for each SSB or CSI-RS based beam management, then RAN4 can consider the UE can guarantee same beam quality between SSB-based eBC or CSI-RS based eBC with same side conditions.

Also, RAN4 still remain the X dB back-off of SSB’ PSD for CSI-RS based eBC requirements. If RAN4 decide X dB is 3~6dB, then the SSB signalling quality is still useful to choose fine CSI-RS beam quality since 0~3 dB SSB Es/Iot is sufficient until -3dB signal quality as shown in RRM measurement accuracy test conditions.

Observation 2: In RF session, RAN4 decide the X = 3~6dB, then the SSB signalling quality is still useful to choose fine CSI-RS beam detection.

Therefore, we realize the worst scenarios for both eBC capability in Rel-16 is SSB-based eBC test case if RAN4 decide X=3~6dB back off of SSB’PSD for CSI-RS eBC side condition.

Based on this observations, we propose as follow

Proposal 1: RAN4 specify X=3~6dB back off of SSB’ PSD for CSI-RS based eBC requirements.

Proposal 2: Based on Proposal 1, RAN4 allow test applicability rule with option1.

- Option1: If a UE meets beam correspondence requirements based on SSB, it is considered to have met the beam correspondence requirements based on CSI-RS.

Conclusion

In this paper, we provide our view on test applicability rule and side conditions for CSI-RS based eBC requirements in rel-16 as follow

Observation 1: In RRM measurement accuracy requirements, they specified same measurement accuracy requirements as 6.5 dB for each SSB based or CSI-RS based beam management with same side conditions.

Observation 2: In RF session, RAN4 decide the X = 3~6dB, then the SSB signalling quality is still useful to choose fine CSI-RS beam detection.

Proposal 1: RAN4 specify X=3~6dB back off of SSB’ PSD for CSI-RS based eBC requirements.

Proposal 2: Based on Proposal 1, RAN4 allow test applicability rule with option1.

- Option1: If a UE meets beam correspondence requirements based on SSB, it is considered to have met the beam correspondence requirements based on CSI-RS.

Reference

- R4-2011678, ‘WF on Beam Correspondence’, Apple

- R4-2011928, “CR on Beam correspondence enhancement,” Nokia

- R4-2011856, ‘Email discussion summary for [96e][116] NR_RF_FR2_req_enh_Part_2, Moderator (Apple)