3GPP TSG-RAN WG4 Meeting #97-e                 	R4-2014361

# Electronic Meeting, 2nd – 13th Nov., 2020

Agenda item:		7.5.2.1

Source:	MediaTek Inc.

Title:	Discussion on LTE CRS based and NR SSB based measurement in NR IDLE/INACTIVE mode

Document for:		Discussion

# 1	Introduction

In RAN4 #96 meeting, companies achieved following agreements and open issues for early measurement reporting (EMR) related requirements [1]:

Agreements - Issue 1-2-2-1: Number of non-overlapping EMR carriers: - The number of non-overlapping inter-frequency EMR carriers is ≤7 - Issue 1-2-2-2: Number of non-overlapping EMR carrier when the total number limit is exceeded: - RAN4 will not define requirements for number of non-overlapping EMR carrier when the total number limit is exceeded - Issue 1-2-3-1: NR inter-frequency beam-level measurement capability: - RAN4 agree following related to NR inter-frequency beam-level measurement capability: - FR1: 7 SSBs with different SSB index and/or PCI per inter-frequency layer - FR2: 10 SSBs with different SSB index and/or PCI per inter-frequency layer - Issue 1-2-5-1: For overlapping EMR carriers: - For overlapping EMR carriers, the UE measurement accuracy requirements for carriers configured for EMR: - RAN4 to define relaxed NR measurement requirements for overlapping carrier compared to existing NR inter-frequency requirements in terms of SNR and accuracy - LTE inter-RAT measurement requirements for overlapping carrier follows existing LTE inter-frequency requirements for CA Idle mode measurements for overlapping carrier - Issue 1-2-3-3: beam-level measurement requirements: - RAN4 agree to allow UE additional time period in measurement requirement for UE who supports the beam level EMR reporting: - TtimeIndex,NR_Interf = [X]* Tmeasure,NR_Interf, where X is [3, 5]. Way Forward - Topic #1: s-NonIntraSearch thresholds and EMR carriers (38.133) - Option 1: - UE applies EMR carrier measurements disregards whether the signal exceeds the configured s-NonIntraSearch thresholds - UE will follow conventional measurement interval for both overlapping and non-overlapping EMR carrier measurements when s-NonIntraSearch thresholds are configured - Note: conventional measurement interval is the measurement period used for cell reselection purpose - Option 2: - No requirements defined for the case s-NonIntraSearch thresholds are configured - Option 3 (Moderator compromise proposal): - RAN4 does not capture in RAN4 Rel-16 specifications anything specific related to search thresholds when UE is configured to perform EMR measurements. Following is proposed: - UE shall be able to support EMR measurement both when search thresholds are configured and when search thresholds are not configured. - UE has to fulfil the accuracy requirements for overlapping EMR measurements as defined 38.133 (to be defined) - Other options are under email discussion - Sub-topic#1-6 Conditions for ‘actively measured’ - Conditions for ‘actively measured’ is FFS - Sub-topic#1-7 Other (38.133) - Option 1: Follow the agreed definitions of overlapping and non-overlapping EMR carriers - Option 2: Based on search thresholds and carrier priority - Option 3: While T331 is running: - overlapping carrier: a carrier configured for EMR and mobility. - non-overlapping carrier: a carrier configured for EMR only. - Option 5: No need for differentiation (added by Moderator)

In this paper, we will discuss all the related issues for NR IDLE/INACTIVE CA and DC measurement.

# 2	Discussion on scenarios of the early measurement reporting

Below we list the existing scenarios for the early measurement reporting as shown in Table 1.

Table 1: Measurements of inter-frequency CA candidate cells

| scenarios | EMR requirement | Purpose | Spec | FR difference | Capability in ASN.1 |
| --- | --- | --- | --- | --- | --- |
| 1 | LTE CRS based measurement in LTE IDLE/INACTIVE | CA | 36.133 | No | ca-IdleModeMeasurements-r15 |  |
| 2 | NR SSB based measurement in LTE IDLE/INACTIVE | DC (EN-DC) | 36.133 | Yes | endc-IdleInactiveMeasurementsFR1-r16 endc-IdleInactiveMeasurementsFR2-r16 |  |
| 5 | NR SSB based measurement in LTE IDLE/INACTIVE | DC (EN-DC) | 36.133 | Yes | Beam level reporting for NR early measurement in LTE IDLE/INACTIVE. |  |
| 3 | LTE CRS based measurement in NR IDLE/INACTIVE | DC (NE-DC) | 38.133 | No | idleInactiveEUTRA-MeasReport-r16 |  |
| 4 | NR SSB based measurement in NR IDLE/INACTIVE | DC ,CA (NR-DC, NR-SA) | 38.133 | Yes | idleInactiveNR-MeasReport-r16 |  |
| 6 | NR SSB based measurement in NR IDLE/INACTIVE | DC ,CA (NR-DC, NR-SA) | 38.133 | Yes | idleInactiveNR-MeasBeamReport-r16 |  |

In this paper, we will focus on the scenario 3, 4 and 6.

# 3	Discussion on measurement capability of NR IDLE/INACTIVE DC and CA measurement

As shown in the introduction part, RAN4 already agreed that “the same principle assumptions which currently applies concerning UE measurement capability would also apply to UE measurement capability of carriers configured for EMR.” According to this agreement, the measurement capabilities for UE supporting inter-freq. or inter-RAT EMR measurement in NR IDLE/INACTIVE mode are listed in Table 2.

Table 2: Measurement capability for UE supporting 
inter-freq. or inter-RAT EMR measurement in NR IDLE/INACTIVE mode

| UE support inter-freq. or inter-RAT EMR measurement | Intra-frequency carriers 1 | NR inter-frequency carriers X≤7, (X= xR + xO + xN) | FDD E-UTRA inter-RAT carriers Y≤7, (Y= yR + yO + yN) | TDD E-UTRA inter-RAT carriers Z≤7, (Z= zR + zO + zN) | Total carriers N≤14, |
| --- | --- | --- | --- | --- | --- |
| Re-selection |  | xR | yR | zR |  |
| Overlapping EMR |  | xO Overlapping inter-freq. EMR xO≤7 | yO | zO | Overlapping inter-RAT EMR kO≤7 (kO= yO+ zO) |
| Non-overlapping EMR |  | xN | yN | zN | Non-overlapping inter-RAT EMR kN≤1 (kN= yN+ zN) |
|  |  | Total inter-freq. EMR carriers xO + xN ≤7 | Total inter-RAT EMR carriers yO + yN+ zO + zN ≤7 |  | Total EMR carriers K≤13 (K=xO + xN + kO+ kN) |

However, the descriptions in the current CR [2] still have ambiguities.

Therefore, we provide a corresponding CR to modify the aforementioned issues [3]

Proposal 1: RAN4 to modify the EMR measurement capabilities specified in TS38.133 according to R4-2014362

# 4	Discussion on measurement requirement of NR IDLE/INACTIVE DC and CA measurement

In this section, we will discuss the open issues listed in [1].

- s-NonIntraSearch thresholds and EMR carriers:

In agreed CR [2], it is already agreed and specified in Section 4.3.2.2 that “for overlapping carriers, the inter-frequency measurement requirements in section 4.2.2.4 apply. For overlapping carriers, the inter-RAT measurement requirements in section 4.2.2.5 apply.” According to the current descriptions in these 2 sections, both measurement principles and measurement requirements are already specified. It is very clear that:

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: UE shall search for inter-frequency layers of higher priority and inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search

- When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ: UE shall search for and measure inter-frequency layers of higher, equal or lower priority layers and follow the requirement in Table 4.2.2.4-1. UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority layers and follow the requirement in Table 4.2.2.5-1.

#### 4.2.2.4	Measurements of inter-frequency NR cells

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

#### 4.2.2.5	Measurements of inter-RAT E-UTRAN cells

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs.

Therefore, based on current spec, s-NonIntraSearch thresholds do affect the EMR measurement requirements

Proposal 2: According to R4-2009264, different EMR measurement requirements are specified for Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and for Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

- Conditions for ‘actively measured’:
As mentioned above, based on current measurement rules, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ

  - UE shall search for inter-frequency layers of higher priority and inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search

  - UE does not have to search for and measure the inter-frequency layers and inter-RAT E-UTRAN layers of equal or lower priority

If a carrier is configured as the inter-frequency layers of higher priority or inter-RAT E-UTRAN layers of higher priority, UE is required to search for and measure that carrier no matter serving cell quality is higher or lower than the s-NonIntraSearch thresholds. Therefore, if this carrier is also configured as an EMR carrier, it is an overlapping carrier. In contrast, if a carrier is configured as the inter-frequency layers of equal/lower priority or inter-RAT E-UTRAN layers of equal/lower priority, UE is required to search for and measure that carrier only when serving cell quality is lower than the s-NonIntraSearch thresholds. Therefore, if this carrier is also configured as an EMR carrier, it is an overlapping carrier when serving cell quality is lower than the s-NonIntraSearch thresholds and becomes a non-overlapping carrier when serving cell quality is higher than the s-NonIntraSearch thresholds

Proposal 3: Ran4 to follow the measurement rules of cell re-selection and update the overlapping and non-overlapping EMR carrier definitions as following table

Table 3: Definition of overlapping and non-overlapping EMR carrier

| Inter-frequency/Inter-RAT E-UTRAN layers configured for both re-selection and EMR | higher priority layers | equal/lower priority layers |
| --- | --- | --- |
| Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Overlapping | Non-overlapping |
| Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Overlapping | Overlapping |

- UE measurement requirements for non-overlapping EMR carriers:

- When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

We first consider the measurement requirements for overlapping EMR carriers when serving cell quality is lower than the s-NonIntraSearch thresholds. As clarified in previous item 1, the measurement requirements of overlapping EMR carriers should follow that of the inter-freq. and inter-RAT measurement as shown in Table 4. Here, the definitions of Kcarrier and NEUTRA_carrier are the number of NR inter-frequency carriers indicated by the serving cell and total number of configured E-UTRA carriers in the neighbour frequency list, respectively.

Table 4: Measurement requirements for cell re-selection purpose

|  | Un-detected cell | detected cell |
| --- | --- | --- |
| Inter-freq. | Kcarrier * Tdetect,NR_Inter | Kcarrier * Tevaluate,NR_Inter |
| Inter-RAT | NEUTRA_carrier * Tdetect,EUTRAN | NEUTRA_carrier* Tevaluate,EUTRAN |

To allow UE to conduct only 1 measurement during each DRX cycle, the evaluating time is multiplied by the carrier number to be monitored. That concept should also be applied when UE is requested to conduct the measurement on both overlapping and non-overlapping EMR carriers. As shown in below Table 5.

Proposal 4: Introduce measurement periodicity requirements for non-overlapping EMR carriers

Assuming that Tdetect,NR_Inter_nonOverEMR is the evaluating time for undetected cell and Tevaluate,NR_Inter_nonOverEMR is the evaluating time for detected cell on the non-overlapping inter-freq. EMR carrier, the carrier number to be monitored Kcarrier’ should be the summation of the number of overlapping inter-freq. EMR carriers, the number of non-overlapping inter-freq. EMR carriers and the number of reselection carriers without EMR configured. Assuming that Tdetect,EUTRAN nonOverEMR is the evaluating time for undetected cell and Tevaluate,EUTRAN_nonOverEMR is the evaluating time for detected cell on the non-overlapping inter-RAT EMR carrier, the carrier number to be monitored NEUTRA_carrier’ should be the summation of the number of overlapping inter-RAT EMR carriers and the number of non-overlapping inter-RAT EMR carriers and the number of reselection carriers without EMR configured. So we propose that

Proposal 5: For measurement requirement of EMR carriers, evaluating time should be multiplied by the total carrier number to be monitored, i.e., the summation of total overlapping EMR carrier number, total non-overlapping EMR carrier number, and total reselection carriers without EMR configured .

As for the evaluating time for non-overlapping inter-freq. and inter-RAT EMR carriers, we can reuse the current evaluating time of overlapping inter-freq. and inter-RAT EMR carriers

Proposal 6: RAN4 to reuse the evaluating time for overlapping inter-freq. and inter-RAT EMR carriers as the evaluating time for non-overlapping inter-freq. and inter-RAT EMR carriers

Proposal 7: RAN4 to specify the measurement requirements for non-overlapping EMR carriers when serving cell quality is lower than the s-NonIntraSearch thresholds as below table

Table 5: Measurement requirements for cell re-selection purpose and EMR purpose 
(Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ)

|  | Un-detected cell | detected cell |
| --- | --- | --- |
| Inter-freq. overlapping | Kcarrier’ * Tdetect,NR_Inter | Kcarrier’ * Tevaluate,NR_Inter |
| Inter-freq. non-overlapping | Kcarrier’ * Tdetect,NR_Inter_nonOverEMR | Kcarrier’ * Tevaluate,NR_Inter_nonOverEMR |
| Inter-RAT overlapping | NEUTRA_carrier’ * Tdetect,EUTRAN | NEUTRA_carrier’ * Tevaluate,EUTRAN |
| Inter-RAT non-overlapping | NEUTRA_carrier’ * Tdetect,EUTRA_nonOverEMR | NEUTRA_carrier’ * Tevaluate,EUTRAN_nonOverEMR |

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

Apply similar principles, the measurement requirements for non-overlapping EMR carriers when serving cell quality is higher than the s-NonIntraSearch thresholds are listed in below

Table 6.

Proposal 8: RAN4 to specify the measurement requirements for non-overlapping EMR carriers when serving cell quality is higher than the s-NonIntraSearch thresholds as below table

Table 6: Measurement requirements for cell re-selection purpose and EMR purpose 
(Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ)

|  | Un-detected cell | detected cell |
| --- | --- | --- |
| Inter-freq. overlapping | Kcarrier’ * (Thigher_priority_search + Tevaluate,NR_Inter) | Kcarrier’ * Tevaluate,NR_Inter |
| Inter-freq. non-overlapping | Kcarrier’ * (Thigher_priority_search + Tevaluate,NR_Inter_nonOverEMR) | Kcarrier’ * Tevaluate,NR_Inter_nonOverEMR |
| Inter-RAT overlapping | NEUTRA_carrier’ * (Thigher_priority_search + Tevaluate,EUTRAN) | NEUTRA_carrier’ * Tevaluate,EUTRAN |
| Inter-RAT non-overlapping | NEUTRA_carrier’ * (Thigher_priority_search + Tevaluate,EUTRAN_nonOverEMR) | NEUTRA_carrier’ * Tevaluate,EUTRAN_nonOverEMR |

- UE requirement related to beam-level reporting:

In last meeting, RAN4 agreed to introduce the beam level measurement requirements for EMR under the assumption that UE will be allowed additional time for such measurement. Based on previous agreement in CONNECTED mode, 3 samples in FR1 and 5 samples in FR2 should be sufficient for UE to complete the time index detection. Therefore, we suggest to introduce additional TtimeIndex,NR_Inter period for time index decoding. As shown in below Table 7, TtimeIndex,NR_Inter is 3* Tmeasure,NR_Inter in FR1 and 5* Tmeasure,NR_Inter in FR2

Proposal 9: RAN4 to introduce additional time period TtimeIndex,NR_Inter in measurement requirement for UE who supports the beam level EMR reporting, where TtimeIndex,NR_Inter equals to 3* Tmeasure,NR_Inter in FR1 and 5* Tmeasure,NR_Inter in FR2

Table 7: Beam level measurement requirements for EMR purpose

|  | Un-detected cell | detected cell |
| --- | --- | --- |
| Inter-freq. overlapping (Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ) | Kcarrier’ * (Tdetect,NR_Inter + TtimeIndex,NR_Inter) | Kcarrier’ * (Tevaluate,NR_Inter+ TtimeIndex,NR_Inter) |
| Inter-freq. non-overlapping (Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ) | Kcarrier’ * (Tdetect,NR_Inter_nonOverEMR+ TtimeIndex,NR_Inter) | Kcarrier’ * (Tevaluate,NR_Inter_nonOverEMR+ TtimeIndex,NR_Inter) |
| Inter-freq. overlapping (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) | Kcarrier’ * (Thigher_priority_search + TtimeIndex,NR_Inter + Tevaluate,NR_Inter) | Kcarrier’ * (Tevaluate,NR_Inter+ TtimeIndex,NR_Inter) |
| Inter-freq. non-overlapping (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) | Kcarrier’ * (Thigher_priority_search + TtimeIndex,NR_Inter + Tevaluate,NR_Inter_nonOverEMR) | Kcarrier’ * (Tevaluate,NR_Inter_nonOverEMR+ TtimeIndex,NR_Inter) |

# 5	Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 to modify the EMR measurement capabilities specified in TS38.133

Proposal 2: According to R4-2009264, different EMR measurement requirements are specified for Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and for Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

Proposal 3: Ran4 to follow the measurement rules of cell re-selection and update the overlapping and non-overlapping EMR carrier definitions as following table

Table 3: Definition of overlapping and non-overlapping EMR carrier

| Inter-frequency/Inter-RAT E-UTRAN layers configured for both re-selection and EMR | higher priority layers | equal/lower priority layers |
| --- | --- | --- |
| Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Overlapping | Non-overlapping |
| Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Overlapping | Overlapping |

Proposal 4: Introduce measurement periodicity requirements for non-overlapping EMR carriers

Proposal 5: For measurement requirement of EMR carriers, evaluating time should be multiplied by the total carrier number to be monitored, i.e., the summation of total overlapping EMR carrier number, total non-overlapping EMR carrier number, and total reselection carriers without EMR configured

Proposal 6: RAN4 to reuse the evaluating time for overlapping inter-freq. and inter-RAT EMR carriers as the evaluating time for non-overlapping inter-freq. and inter-RAT EMR carriers

Proposal 7: RAN4 to specify the measurement requirements for non-overlapping EMR carriers when serving cell quality is lower than the s-NonIntraSearch thresholds as below table

Table 5: Measurement requirements for cell re-selection purpose and EMR purpose 
(Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ)

|  | Un-detected cell | detected cell |
| --- | --- | --- |
| Inter-freq. overlapping | Kcarrier’ * Tdetect,NR_Inter | Kcarrier’ * Tevaluate,NR_Inter |
| Inter-freq. non-overlapping | Kcarrier’ * Tdetect,NR_Inter_nonOverEMR | Kcarrier’ * Tevaluate,NR_Inter_nonOverEMR |
| Inter-RAT overlapping | NEUTRA_carrier’ * Tdetect,EUTRAN | NEUTRA_carrier’ * Tevaluate,EUTRAN |
| Inter-RAT non-overlapping | NEUTRA_carrier’ * Tdetect,EUTRA_nonOverEMR | NEUTRA_carrier’ * Tevaluate,EUTRAN_nonOverEMR |

Proposal 8: RAN4 to specify the measurement requirements for non-overlapping EMR carriers when serving cell quality is higher than the s-NonIntraSearch thresholds as below table

Table 6: Measurement requirements for cell re-selection purpose and EMR purpose 
(Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ)

|  | Un-detected cell | detected cell |
| --- | --- | --- |
| Inter-freq. overlapping | Kcarrier’ * (Thigher_priority_search + Tevaluate,NR_Inter) | Kcarrier’ * Tevaluate,NR_Inter |
| Inter-freq. non-overlapping | Kcarrier’ * (Thigher_priority_search + Tevaluate,NR_Inter_nonOverEMR) | Kcarrier’ * Tevaluate,NR_Inter_nonOverEMR |
| Inter-RAT overlapping | NEUTRA_carrier’ * (Thigher_priority_search + Tevaluate,EUTRAN) | NEUTRA_carrier’ * Tevaluate,EUTRAN |
| Inter-RAT non-overlapping | NEUTRA_carrier’ * (Thigher_priority_search + Tevaluate,EUTRAN_nonOverEMR) | NEUTRA_carrier’ * Tevaluate,EUTRAN_nonOverEMR |

Proposal 9: RAN4 to introduce additional time period TtimeIndex,NR_Inter in measurement requirement for UE who supports the beam level EMR reporting, where TtimeIndex,NR_Inter equals to 3* Tmeasure,NR_Inter in FR1 and 5* Tmeasure,NR_Inter in FR2

# Reference

- R4-2012246, WF on MR-DC RRM requirements, Nokia, Aug., 2020.

- R4-2009264, Big CR Introduction of UE requirement for MR-DC early measurement reporting in 38.133, Jun., 2020.

- R4-2014362, CR on TS38.133 for measurement capability of IDLE mode DC/CA measurement, Mediatek, Nov., 2020.