3GPP TSG-RAN WG4 Meeting #97-e                                                               R4-2014394

# Electronic Meeting, 2 - 13 Nov., 2020

Source: 	CATT

Title: 	Discussion on out of band CLTA maximum height

Agenda item:	4.5.4

Document for:	Discussion

- Introduction

In RAN4#96-e meeting, The WF [1] on out of band CLTA maximum height was approved as following with two options to be further discussed in this meeting.

This contribution will provide our further views on out of band CLTA definition.

- Discussion

The two options for out-of-band CLTA are discussed in the following.

Option 1

For Option 1, the two candidate out-of-band CLTAs are defined based on either the same height or the same vertical beamwidth as operating band antenna respectively. According to availability of two candidate out-of-band CLTAs, one candidate out-of-band CLTA will be used as out-of-band CLTA. It means that different testers may choose different out-of-band CLTA depending on the availability of the 2 type of CLTAs, which will affect co-located requirement verification.

Observation 1: The availability condition for option 1 is not clear, which may affect the selection of out-of-band CLTA and requirement verification.

Two kinds of operating band antennas as shown in Table 1 are used to analyse option 1. The two candidate out-of-band CLTAs for antenna 1, and antenna 2 are shown in Table 2 and Table 3 respectively.

From Table 2, two candidate out-of-band CLTAs for Antenna 1(foperatingBand=0.45GHz) are listed for some co-located bands. For fcolocatedBand =1GHz, both of the 2 candidate CLTA either based on the same height (height =1.5m, theta3dB=10.26degree) or based on the same beamwidth (height =0.68m, theta3dB=22.8degree) are available according to our experience. According to the analysis of coupling loss for different height of out-of-band CLTA [5], different testers may choose different out-of-band CLTA, which will result in different co-located testing results.

Observation 2: For option 1, two candidate out-of-band CLTAs might be available for a specific co-located band, which will result in different out-of-band CLTA selection and different test results.

From Table 3, two candidate out-of-band CLTAs for Antenna 2(foperatingBand=7GHz) are listed for some co-located bands. For fcolocatedBand = 0.45GHz, neither the CLTAs based on the same height (height =0.31m, theta3dB=108.89 degree) nor the CLTA based on the same beamwidth (height =4.82m, theta3dB=7degree) is reasonable. For CLTA based on the same height, 0.31m is less than the wavelength/2 at 0.45GHz (0.34m). For CLTA based on the same beamwidth, the height of 4.82m is too high.

Observation 3: For option 1, there is the case that no candidate out-of-band CLTA for a specific co-located band is available.

Table 1: 2 type of operating band antennas

|  | Antenna 1 | Antenna 2 |
| --- | --- | --- |
| foperatingBand (GHz) | 0.45 | 7 |
| Height(m) | 1.5 | 0.31 |
| Theta3dB(degree) | 22.8 | 7 |

Table 2: Two candidate out-of-band CLTAs for Antenna 1

| fcolocatedBand(GHz) | 0.75 |  | 1 |  | 2 |  | 3.5 |  | 5 |  | 7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | same Height | same beamwidth | same Height | same beamwidth | same Height | same beamwidth | same Height | same beamwidth | same Height | same beamwidth | same Height | same beamwidth |
| Height(m) | 1.5 | 0.9 | 1.5 | 0.68 | 1.5 | 0.34 | 1.5 | 0.19 | 1.5 | 0.14 | 1.5 | 0.1 |
| Theta3dB(degree) | 13.68 | 22.8 | 10.26 | 22.8 | 5.13 | 22.8 | 2.93 | 22.8 | 2.05 | 22.8 | 1.47 | 22.8 |

Table 3: Two candidate out-of-band CLTAs for Antenna 2

| fcolocatedBand(GHz) | 0.45 |  | 0.75 |  | 1 |  | 2 |  | 3.5 |  | 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | same Height | same beamwidth | same Height | same beamwidth | same Height | same beamwidth | same Height | same beamwidth | same Height | same beamwidth | same Height | same beamwidth |
| Height(m) | 0.31 | 4.82 | 0.31 | 2.89 | 0.31 | 2.17 | 0.31 | 1.09 | 0.31 | 0.62 | 0.31 | 0.43 |
| Theta3dB(degree) | 108.89 | 7 | 65.33 | 7 | 49 | 7 | 24.5 | 7 | 14 | 7 | 9.8 | 7 |

Option 2

For option 2, the only concern is 2.5m height limit in option 2 may be further discussed in RAN4#97-e.

Referring to antenna products for 698~824MHz at https://www.tycc.cn, the antenna parameters of antenna products are shown in the Table 4.

Table 4: Antenna products’ parameter for 698~824MHz

| f (MHz) | 698~803 [2] | 698~803 [3] | 698~824[4] |
| --- | --- | --- | --- |
| Height(mm) | 2580 | 1980 | 1380 |
| Gain (dBi) | 16.1±0.4 | 15.3±0.5 | 14.5 |
| Phi3dB(degree) | 71±3.2 | 69±2.1 | 68 |
| Theta3dB(degree) | 8.9±0.7 | 11.2±0.9 | 17.4 |
| size | 2580×300×146 | 1980×300×146 | 1380×300×146 |

From table 4, the height of antenna products for 698~824MHz from 1.3-2.5 are all available.  Considering the operability of the testing, the 1.5m height limit could be used as the height limit for option 2.

Observation 4: 1.5m height limit could be used as the height limit for option 2.

Proposal 1:  Adopt option 2 for out-of-band CLTA as following:

The half-power vertical beam width of the out of band CLTA equals .

Where,

h is the test object vertical radiating length in meter.

is the narrowest declared (D.3) vertical beam width of test object antenna.

is the centre frequency of operating band of test object antenna.

is the centre frequency of co-located band.

- Conclusion

This contribution further discussed out of band CLTA definition and gave the following proposals.

Observation 1: The availability condition for option 1 is not clear, which may affect the selection of out-of-band CLTA and requirement verification.

Observation 2: For option 1, two candidate out-of-band CLTAs might be available for a specific co-located band, which will result in different out-of-band CLTA selection and different test results.

Observation 3: For option 1, there is the case that no candidate out-of-band CLTA for a specific co-located band is available.

Observation 4: 1.5m height limit could be used as the height limit for option 2.

Proposal 1:  Adopt option 2 for out-of-band CLTA as following:

The half-power vertical beam width of the out of band CLTA equals .

Where,

h is the test object vertical radiating length in meter.

is the narrowest declared (D.3) vertical beam width of test object antenna.

is the centre frequency of operating band of test object antenna.

is the centre frequency of co-located band.

- References

- R4-2012589,WF on selecting CLTA height,Huawei,RAN4#96-e.

- TDJ-609018DEI-65F, https://www.tycc.cn.

- TDJ-609017DEI-65Fv02, https://www.tycc.cn.

- TDJ-609015DEH-65F, https://www.tycc.cn.

- R4-2009967, On the criteria for selecting a proper CLTA, Ericsson, RAN4#96-e.