3GPP TSG-RAN WG4 Meeting #97-e	R4-2014428

Electronic Meeting, 2 Nov. – 13 Nov., 2020

Title: 	Discussion on RLM/BFD relaxation for NR power saving

Source: 	CATT

Agenda item:	12.9.2

Document for:	Discussion

- Introduction

A new WI of UE power saving enhancements was approved in RAN#86 meeting [1]. In accordance to the WI objectives, RRM measurement relaxation in connected state need to be specified in RAN4, the related work objective is as follows:

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4]

  - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1]

- NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized.

  - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4]

NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

In this contribution, we provide our initial analysis on RLM/BFD relaxation for NR UE power saving.

- Discussion

Observation #1: Before Rel-17, evaluation periods for RLM and BFD are specified in 38.133 determined by multiple factors such as: DRX cycle / periodicity of SSB or CSI-RS resource / N (Rx beam sweeping, FR2 only) / P.

Whether to further relax on RLM/BFD measurement in Rel-17 should be studied in RAN4.

Proposal #1: RAN4 should investigate how to relax on RLM/BFD measurement. The basic idea is that UE can achieve power saving gain by increasing measurement period which can be done by adding new relaxation factor on RLM/BFD measurement.

Proposal #2: RAN4 should investigate on the relaxation condition.

- RLM-RS is based on SSB or CSI-RS or both?

RLM-RS types for relaxation shall be further studied and decided in RAN4.

- How to determine relaxation factor?

Option 1: to simplify UE implementation, define the fixed value for all conditions.

Option 2: allow for different values for different conditions.

Option 2 is preferred.

If option 2 is chosen, the different value of relaxation factor should be determined based on simulation assuming different conditions of:

- RLM-RS

- UE speed

- DRX cycle (no DRX/short/long)

- Periodicity of SSB or CSI-RS resource

- N (RX beam for FR2)

- P (scale factor with consideration of overlap with measurement gap and/or SMTC window)

If in some conditions, the power saving gain is too small, the relaxation factor will be 1 which means no relaxation for RLM/BFD measurement. Finally, specify the proper relaxation factor for each scenario.

Proposal #3: For different pairs of IS/OOS BLER values, whether to use the same value of relaxation factor. RAN4 needs to study on it.

- Reference

[1] RP-193239 New WID UE Power Saving Enhancements