3GPP TSG-RAN4 Meeting #97-e	                                                                R4-2014445

Electronic meeting, Nov. 2-13, 2020

Title:	Discussion on PRS RSTD measurement requirements

Source:	CATT

Agenda Item:	7.7.2.1

Document for:	Discussion

1 Introduction

In RAN4 #96-e meeting [1], the core part of NR positioning was completed and the measurement period of RSTD measurement was defined as below.

Agreement - TRSTD,i= (NRxBeam,i*CSSFRSTD,i*Nproc,i*Nsample - 1) *Teffect,i+ Tlast - CSSFRSTD,i is the CSSF for sharing between PRS and RRM measurements within a single positioning frequency layer - NRxBeam,i is the RX beam sweeping scaling factor within a single positioning frequency layer - Note: if issues are identified with CSSF then the equation can be revisited in the maintenance part Agreement: Tlast = Ti + LPRS,i Note: RAN4 assumption is that LPRS,i is the sample duration in ms. The decision can be further revisited in case the definition is not aligned with RAN1/2 specification. Agreement - When MGs and processing time T have overlap between different frequency positioning frequency layers - TRSTD, total=iTRSTD,i+X Agreement: X = Nlayer-1×maxTeffect,i.

From the conclusions above, the measurement requirements of RSTD are basically defined. But there are still some open issues need to be discussed in this meeting which are captured in the agreed WF [2] as below.

- Open issues to be discussed at RAN4#97-e - Calculation of PRS sample duration - Option 1: Based on the type (type 1 or type 2) as UE used to report {N,T} - Option 2: Based on Type 2. RSTD measurement period for Type 1 PRS duration calculation shall be no longer than Type 2. - Other options are not precluded - Multiple PRS periodicities - Option 1: Use the maximum PRS resource periodicity among all PRS resource in a same positioning frequency layer - Option 2: Use the maximum PRS resource periodicity among all PRS resource among all configured PRS frequency layers. - Measurement period extension due to SSB collision - Option 1: RSTD measurement period to be defined for cases when PRS occasions are not dropped - Option 2: The RSTD measurement period can be extended to compensate for the number of PRS symbols not available at the UE due to their overlap with SSB symbols - Option 3: The same measurement period requirement shall be met, regardless of whether some of the PRS symbols are dropped or not during this measurement period - Measurement period when configured with PRS-RSRP - Option 1: If RSTD is configured together with PRS-RSRP, then the measurement periods for both measurements are defined as: max(TRSTD,TPRS-RSRP), where TPRS-RSRP and TRSTD are the measurement periods for PRS-RSRP and RSTD, when configured without other measurements. - Option 2: RSTD measurement period is not impacted by PRS-RSRP measurement.

In this paper, we have some further discussion on the remaining issues and give our proposals.

2 Discussion

2.1 Calculation of PRS sample duration

From the agreements of RSTD measurement period, the measurement delay is related to the PRS sample duration. According to RAN1’s agreement in [3], the PRS sample duration can be calculated in two different ways (Type 1 and type 2) and reported as UE capability.

Agreement: - For the purpose of DL PRS processing capability, the duration of DL PRS symbols (K) in ms within any P msec window, is calculated by - Type 1 duration calculation with Ks=Tsend-Tsstart - Type 2 duration calculation with K=12μSwhere - Type 1 or Type 2 is reported as UE capability, - S is the set of slots of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets (target and reference), - for Type 1, [Tsstart, Tsend]Tsstart, Tsend is the smallest interval in ms within slot s corresponding to an integer number of OFDM symbols of a serving cell that covers the union of the potential PRS symbols and determines the PRS symbol occupancy within slot s. Interval[Tsstart, Tsend] considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets (target and reference).

Then in our understanding, the measurement requirements should not depend on a certain calculation type of PRS duration. When defining the measurement period, just the calculation results of sample duration are needed. So UE should be able to calculate the sample duration according to its own capability.

And in our opinion, the calculation way of PRS sample duration has no impact on the definition of measurement requirements in NR.

Proposal 1: The calculation of PRS sample duration should be based on the type (type 1 or type 2) as UE used to report {N,T}.

2.2 Multiple PRS periodicities

For single frequency layer measurement, when multiple PRS periodicities are configured, the RSTD measurement should be based on the maximum PRS periodicity. According to the conclusion in last meeting, the total RSTD measurement delay is the summation of the measurement period in each frequency layer. It means the measurement period of each frequency layer is calculated independently. So in our opinion, the measurement period in each frequency layer should be based on the maximum PRS periodicities in this layer.

Proposal 2: When multiple PRS periodicities are configured, use the maximum PRS resource periodicity among all PRS resource in a same positioning frequency layer.

2.3 Measurement period extension due to SSB collision

In our opinion the extension due to symbol overlapping is unnecessary when defining the requirements of PRS based measurement. This collision is very uncertain and the requirements should be defined only for the case without PRS dropping.

For another aspect, I think this issue is considered for without gap case while the gap sharing is considered for with gap case. Since the positioning measurement in R16 is always performed within gap, this issue is no need to discuss in our opinion.

Proposal 3: RSTD measurement period to be defined for cases when PRS occasions are not dropped.

2.4 Measurement period when configured with PRS-RSRP

When PRS-RSRP measurement is configured with RSTD measurement, it is usually used as assistance data. The measurement delay for positioning should not be expanded due to the assistance data which are not critical.

Since we propose to reuse the RSTD measurement period requirement for the PRS RSRP measurement configured alone, if it is agreed, there is no difference between the two options above.

Proposal 4: RSTD measurement period is not impacted by PRS-RSRP measurement.

3 Summary

In this paper, the requirements of  RSTD measurement are further discussed, and following proposals are given：

Proposal 1: The calculation of PRS sample duration should be based on the type (type 1 or type 2) as UE used to report {N,T}.

Proposal 2: When multiple PRS periodicities are configured, use the maximum PRS resource periodicity among all PRS resource in a same positioning frequency layer.

Proposal 3: RSTD measurement period to be defined for cases when PRS occasions are not dropped.

Proposal 4: RSTD measurement period is not impacted by PRS-RSRP measurement.

4 References

- RAN4_96e_RRM_chairman_report_12_EOM

- R4-2012283, WF on requirements for RSTD and UE Rx-Tx time difference measurement, Huawei

- Chairman's Notes RAN1#100b-e v030