3GPP TSG-RAN4 Meeting #97-e	R4-2014451

# Electronic meeting, Nov. 2-13, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PRS-RSRP accuracy requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2020-10-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The performance requirements for PRS-RSRP measurement need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the performance requirements for PRS-RSRP measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements for PRS-RSRP measurement are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.24.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

10.1.24	PRS-RSRP Measurements

10.1.24.1	Introduction

The requirements in Clause 10.1.24 shall apply, provided the UE has received nr-DL-TDOA-RequestLocationInformation or nr-Multi-RTT-RequestLocationInformation or nr-DL-AoD-RequestLocationInformation message from LMF via LPP [34] requesting the UE to report one or more DL PRS-RSRP measurements defined in TS 38.215 [4].

10.1.24.2	Measurement Accuracy Requirements

10.1.24.2.1	Intra-Frequency Accuracy Requirement for FR1

The accuracy requirements in Table 10.1.24.2.1-1 are valid under the following conditions:

Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

PRP 1,2|dBm according to Annex B.2.x for a corresponding Band

Table 10.1.24.2.1-1: PRS RSRP intra-frequency measurement accuracy for FR1

| Accuracy |  |  | Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | Minimum PRS bandwidth, which is minimum of serving cell channel bandwidth and the PRS bandwidths of the reference cell and the measured neighbour cell i Note 5 |  | Minimum number of available measurement subframes among the reference cell and the measured neighbour cell i |  | Io Note 7 range |  |  |  |  |
|  |  |  |  |  |  | NR operating band groups Note 8 |  | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB |  | RB |  |  |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 |  |  |
| 4.5 | ±9 | (PRS Ês/Iot)ref ≥-6dB and (PRS Ês/Iot)i ≥-13dB |  | ≥ 25 | 6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A |  | -134 | -131 |  | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B |  | -133.5 | -130.5 |  | -50 |
|  |  |  |  |  |  | NR_TDD_FR1_C |  | -133 | -130 |  | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  | -132.5 | -129.5 |  | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  | -132 | -129 |  | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_F |  | -131.5 | -128.5 |  | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_G |  | -131 | -128 |  | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_H |  | -130.5 | -127.5 |  | -50 |
| 3 | ±9 | (PRS Ês/Iot)ref ≥-6dB and (PRS Ês/Iot)i ≥-13dB |  | 75 | 6 | Note 4 |  |  | Note 4 |  | Note 4 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 6 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.x.y and B.x.y. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

10.1.24.2.2	Intra-Frequency Accuracy Requirement for FR2

The accuracy requirements in Table 10.1.24.2.2-1 are valid under the following conditions:

Conditions defined in 38.101-2 Clause 7.3 for reference sensitivity are fulfilled.

PRP 1,2|dBm according to Annex B.2.x for a corresponding Band

Table 10.1.24.2.2-1: PRS RSRP intra-frequency measurement accuracy for FR2

| Accuracy |  |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot |  | Minimum PRS bandwidth, which is minimum of serving cell channel bandwidth and the PRS bandwidths of the reference cell and the measured neighbour cell i Note 5 | Minimum number of available measurement subframes among the reference cell and the measured neighbour cell i |  | Io Note 7 range |  |  |  |
|  |  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  |  |  | Maximum Io |
| dB | dB | dB |  | RB |  | dBm / SCSPRS |  |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/120kHz Note 6 |  | dBm/240kHz Note 6 |  |  |
| 4.5 | ±9 | (PRS Ês/Iot)ref ≥-6dB and (PRS Ês/Iot)i ≥-13dB |  | ≥ 25 | 6 | Same value as PRP in Table B.2.10-2, according to UE Power class, operating band and angle of arrival |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  |  | -50 |
| 3 | ±9 | (PRS Ês/Iot)ref ≥-6dB and (PRS Ês/Iot)i ≥-13dB |  | 75 | 6 | Note 4 |  |  |  | Note 4 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 6 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.x.y and B.x.y. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

10.1.24.2.3	Inter-Frequency Accuracy Requirement for FR1

The accuracy requirements in Table 10.1.24.2.3-1 are valid under the following conditions:

Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

PRP 1,2|dBm according to Annex B.2.x for a corresponding Band

Table 10.1.24.2.3-1: PRS RSRP inter-frequency measurement accuracy for FR1

| Accuracy |  |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot |  | Minimum PRS bandwidth, which is minimum of serving cell channel bandwidth and the PRS bandwidths of the reference cell and the measured neighbour cell i Note 5 | Minimum number of available measurement subframes among the reference cell and the measured neighbour cell i |  | Io Note 7 range |  |  |  |
|  |  |  |  |  |  | NR operating band groups Note 8 |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB |  | RB |  |  |  | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 |  |
| 4.5 | ±9 | (PRS Ês/Iot)ref ≥-6dB and (PRS Ês/Iot)i ≥-13dB |  | ≥ 25 | 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A |  | -134 | -131 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B |  | -133.5 | -130.5 | -50 |
|  |  |  |  |  |  | NR_TDD_FR1_C |  | -133 | -130 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  | -132.5 | -129.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  | -132 | -129 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_F |  | -131.5 | -128.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_G |  | -131 | -128 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_H |  | -130.5 | -127.5 | -50 |
| 3 | ±9 | (PRS Ês/Iot)ref ≥-6dB and (PRS Ês/Iot)i ≥-13dB |  | 75 | 4 | Note 4 |  |  | Note 4 | Note 4 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 6 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.x.y and B.x.y. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |  |

10.1.24.2.4	Inter-Frequency Accuracy Requirement for FR2

The accuracy requirements in Table 10.1.24.2.4-1 are valid under the following conditions:

Conditions defined in 38.101-2 Clause 7.3 for reference sensitivity are fulfilled.

PRP 1,2|dBm according to Annex B.2.x for a corresponding Band

Table 10.1.24.2.4-1: PRS RSRP inter-frequency measurement accuracy for FR2

| Accuracy |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot |  | Minimum PRS bandwidth, which is minimum of serving cell channel bandwidth and the PRS bandwidths of the reference cell and the measured neighbour cell i Note 5 | Minimum number of available measurement subframes among the reference cell and the measured neighbour cell i |  | Io Note 7 range |  |  |
|  |  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB |  | RB |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/120kHz Note 6 |  | dBm/240kHz Note 6 |  |
| 4.5 | ±9 | (PRS Ês/Iot)ref ≥-6dB and (PRS Ês/Iot)i ≥-13dB |  | ≥ 25 | 4 | Same value as PRP in Table B.2.10-2, according to UE Power class, operating band and angle of arrival |  |  | -50 |
|  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  | -50 |
|  |  |  |  |  |  |  |  |  | -50 |
| 3 | ±9 | (PRS Ês/Iot)ref ≥-6dB and (PRS Ês/Iot)i ≥-13dB |  | 75 | 4 | Note 4 |  |  | Note 4 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 6 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.x.y and B.x.y. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

<End of Change 1>