3GPP TSG-RAN WG4 Meeting # 97-e                                                              R4-2014457

Electronic Meeting, 2-13 Nov., 2020

Source: 	CATT

Title: 	BS parameters for the frequency range 6.425-7.125GHz, 7.025-7.125GHz and 10.0-10.5GHz

Agenda Item:	11.1.2

Document for:	Discussion

- Introduction

In RAN#87-e meeting, a SI on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz was approved in [1]. There were extensive discussions on this topic and 2 WF was approved in [2, 3]. Companies are encouraged to have further inputs on how to reply the parameters regarding the open issue in [3]. No progress has been made in RAN4#96e due to misalignment between companies’ results.

This paper presents our view on BS parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz based on updated simulation results in [4, 5].

- Discussion

  - BS parameters for 6.425-7.125 GHz and 7.025-7.125GHz

The following items were agreed in [3]

- ACLR/ACS/SEM/Blocking: based on simulations - Power dynamic range: 0dB - Spurious emissions: - Option 1: TS 38.104 clause 6.6.5.2.1 and 9.7.5.2 - Option 2: 30MHz ≤ f ≤ 1 GHz: -36dBm/100kHz 1 GHz ≤ f ≤ Fbreak : -30dBm/1MHz Fbreak ≤ f ≤ 26 GHz: -20dBm/10MHz, with Fbreak is FFS. - Other options are not precluded - Maximum output power: To be aligned with antenna characteristics - Average output power: N/A. - Noise figure: 6dB for WA/11dB for MR/14dB for LA - Reference sensitivity: Do not consider in the reply

ACLR

According to the simulation results for uncoordinated deployment (100% shift) for 6GHz in [4], both the average throughput loss and the cell edge user throughput loss goes below 5% at 28dB. Since UE ACS is the dominating factor for downlink ACIR and usually BS ACLR is more than 10dB larger than the UE ACS, it is proposed to define BS ACLR as 40dB for 6.425-7.025GHz, 7.025-7.125GHz.

ACS

According to the simulation results for uncoordinated deployment (100% shift) for 6GHz in [5], both the average throughput loss and cell edge user throughput loss goes below 5% at 27dB. Since UE ACLR is the dominating factor for uplink ACIR and usually BS ACS is more than 10dB larger than the UE ACLR, it is proposed to define BS ACS as 40dB for 6.425-7.025GHz, 7.025-7.125GHz.

Blocking

According to the simulation results for uncoordinated deployment (100% shift) for 6GHz in [5], the blocking interference at 99.99% CDF point is about -44dB. It is proposed to define BS blocking level as -44dB for 6.425-7.025GHz, 7.025-7.125GHz.

Spurious emission

Since the frequency range is very close to the available FR1 frequency range in the spec. we propose to adopt similar spurious emissions as those defined for FR1 for 6.425-7.025GHz, 7.025-7.125GHz.

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 150 kHz | -36 dBm | 1 kHz |
| 150 kHz – 30 MHz |  | 10 kHz |
| 30 MHz – 1 GHz |  | 100 kHz |
| 1 GHz – 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz |

  - BS parameters for 10.0-10.5GHz

The following items were agreed for 10-10.5GHz in [3]

- ACLR/ACS/SEM/Blocking: based on simulations - Power dynamic range: 0dB - Spurious emissions: - Option 1: TS 38.104 clause 6.6.5.2.1 and 9.7.5.2 - Option 2: 30MHz ≤ f ≤ 1 GHz: -36dBm/100kHz 1 GHz ≤ f ≤ Fbreak : -30dBm/1MHz Fbreak ≤ f ≤ 26 GHz: -20dBm/10MHz, with Fbreak is FFS - Other options are not precluded - Maximum output power: To be aligned with antenna characteristics - Average output power: N/A. - Noise figure: : 7dB for WA/12dB for MR/15dB for LA - Reference sensitivity: Do not consider in the reply

ACLR

According to the simulation results for uncoordinated deployment (100% shift) for 10GHz in [4], both the average throughput loss and the cell edge user throughput loss goes below 5% at 26dB. Since UE ACS is the dominating factor for downlink ACIR and usually BS ACLR is more than 10dB larger than the UE ACS, it is proposed to define BS ACLR as 40dB for 10-10.5GHz.

ACS

According to the simulation results for uncoordinated deployment (100% shift) for 10GHz in [5], both the average throughput loss and cell edge user throughput loss goes below 5% at 23dB. Since UE ACLR is the dominating factor for uplink ACIR and usually BS ACS is more than 10dB larger than the UE ACLR, it is proposed to define BS ACS as 35-40dB for 10-10.5GHz.

Blocking

According to the simulation results for uncoordinated deployment (100% shift) for 6GHz in [5], the blocking interference at 99.99% CDF point is about -40dB. It is proposed to define BS blocking level as 40dB for 10-10.5GHz.

Spurious emission

It was pointed out by [6], the spurious emission cannot reference to 38.104 as there is no spurious emission for this band. So Option 2 is proposed as below. In general we are ok with this option.

    - Option 2:

30MHz ≤ f ≤ 1 GHz: -36dBm/100kHz

1 GHz ≤ f ≤ Fbreak : -30dBm/1MHz

Fbreak ≤ f ≤ 26 GHz: -20dBm/10MHz, with Fbreak is FFS

Regarding the breaking point, we can have some analysis based on the current spurious emission for FR1 and FR2 in 38.104. For FR1, the -30dBm is applicable to 5th  harmonic of the upper frequency edge of the DL operating band in GHz if we use 5th harmonic of the upper frequency edge of the DL operating band for 10GHz, the frequency range to be tested will goes too high, e.g. ~50GHz. So we propose to use the FR2 break point of 18GHz for 10-10.5GHz band. Then the spurious emission requirement for 10-10.5GHz will be

30MHz ≤ f ≤ 1 GHz: -36dBm/100kHz

1 GHz ≤ f ≤18GHz : -30dBm/1MHz

18GHz ≤ f ≤ 26 GHz: -20dBm/10MHz

- Conclusion

The contribution further discussed the open issue for BS parameters for the frequency range 6.425-7.125GHz, 7.025-7.125GHz and 10.0-10.5GHz.

References

[1]	RP-200513,	New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz	Ericsson, Huawei, HiSilicon

[2] R4-2005733, WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, CATT

[3] R4-2009151, WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, CATT

[4] R4-2014458, Simulation results for 6425-7125MHz and 10-10.5GHz-downlink, CATT

[5] R4-2014459, Simulation results for 6425-7125MHz and 10-10.5GHz-uplink, CATT