3GPP TSG-RAN WG4 Meeting #97-e		                                     R4-2014467

Electronic Meeting, 02 November – 13 November 2020

# Source:	Hughes Network Systems, Thales, GlobalStar, Ligado

# Title:	Possible FR2 exemplary band for NR based satellite networks

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	12.8.2 Use cases and deployment scenarios

Release	Rel-17

# Introduction

The list of all frequency bands that are allocated to Satellite networks are based on the different ITU regions here below depicted:

Figure 1: ITU regions and the dividing lines between them.  Region 1 (Yellow), Region 2 (Blue), Region 3 (Purple)

Any of these frequency bands could be considered for the operation of satellite network using NR radio interface. The use of some frequency bands may require coexistence studies if not already concluded in ITU-R. According to R4-2006894, potential frequency bands for consideration are:

- For FR1: Frequency bands L, S, C.

- For FR2: Frequency bands Ku, Ka, Q/V.

# Proposed exemplary band for NR based Satellite networks operating in FR2

RP-202120 was discussed at plenary meeting RAN#89-e, and the first and third proposals were:

“Proposal 1: In RAN4, the WID “NR-NTN-solutions” will define the generic and core requirements and consider at least one example satellite band. RAN4 to decide based on technical considerations which band(s) to consider as exemplary band for the WI “NR-NTN-solutions”, with at least the pre-condition that the intended usage is compliant with ITU-R Radio Regulations.”

“Proposal 3: The proponents of a RAN4 led “satellite” band specific WI are expected to reference all the relevant sources (including but are not limited to ITU-R Radio Regulations, relevant national regulations, pre-existing Harmonized Standards developed for example in ETSI, coexistence studies approved by regulatory bodies and/or 3GPP specifications) and contribute to the definition of the requirements that should be met. Moreover they shall clarify the use case scenarios and architectures (e.g. orbit, altitude, type of UE, duplex mode, channel bandwidth, SCS...) provided that the use case scenarios and architectures are within the scope of WID “NR-NTN-solutions” in order to define the requirements.”

In this section we propose to consider at least one example satellite band for FR2 (e.g. Ka-band).

## 2.1 Ka band spectrum for Satellite services

The following bands are commonly referred to as “Ka band” and identified by ITU Radio Regulation for use by Geo-stationary Orbit (GEO) and Non-Geostationary Orbit (NGSO) based Satellite services on a worldwide basis. Based on the satellite operation, the duplex mode will be FDD, as allocated by ITU Radio Regulations.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space-earth) | 17.3 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth-space) | 27.5 – 30.0 GHz | 27.0 – 30.0 GHz | 27.0 – 30.0 GHz |

Table 2.1-1: “Ka band” spectrum for GEO based Satellite services

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space-earth) | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth-space) | 27.5 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz |

Table 2.1-2: “Ka band” spectrum for NGSO Satellite services

Note: in some countries, certain portions of this range are co-primary with terrestrials and national rules & regulations are applicable.

## 2.2 List of relevant standardization sources

The following pre-existing harmonized standards developed in ETSI for satellite services in this band may be considered during the work:

- ETSI EN 303 978 V2.1.2 (2016-10) -Satellite Earth Stations and Systems (SES); Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU

- ETSI EN 303 979: Satellite Earth Stations and Systems (SES); Harmonized Standard for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27.5 GHz to 29.1 GHz and 29.5 GHz to 30.0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU

## 2.3 List of relevant regulatory sources

- Radio Regulations Article 5 – Frequency Allocations, 2016 Edition

- FINAL ACTS of the World Radiocommunication Conference (WRC-19)

- Article 3.2 of the Directive 2014/53/EU

- FCC-16-89A1, https://apps.fcc.gov/edocs_public/attachmatch/FCC-16-89A1.pdf

Proposal 1: RAN4 work should consider an exemplary FR2 band for NTN.

Proposal 2: Propose RAN4 to use an FR2 exemplary band of 17.7 – 20.2 GHz for DL and 27.5 – 30.0 GHz for UL with FDD duplex mode.

## 2.4 NR-ARFCN and channel bandwidth

The proposed NR-ARFCN and channel bandwidth in these frequency range are given in the table 2.4-1 and 2.4-2 respectively.

| Operating Band | Uplink (UL) operating band UE transmit | Downlink (DL) operating band UE Receive | Duplex mode |
| --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high | FDD |
| Ka Band | 27.75 – 30.0 GHz | 17.7 – 20.2 GHz | FDD |

Table 2.4-1: Proposed NR-ARFCN

| Operating band/SCS/UE channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS KHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 17.7 – 20.2 GHz | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| 27.75 – 30.0 GHz | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |

Table 2.4-2: Proposed channel bandwidth

2.5 Recommended parameter for UE with directive antenna

UE with directive antenna, can be categorized as “fixed or mounted” with the following power classes and capabilities:

- NTN capable UE of power class 1 – ESOMP/ESIM mounted on aircrafts, ships/vessels, fast train, vehicles. The UE can also act as relay to other UEs on board the moving platforms

- Fixed UE, example VSAT

Figure 2.4: Uu for satellite-based NR in Ka-Band

The recommended UE parameters for ESOMP/ESIM are as defined in ETSI EN 303 978 for GEO and ETSI EN 303 979- for NGSO.

The UE parameter description for VSAT is highlighted in TR38.821 [2] in Table 6.1.1.1-3: UE characteristics for system level simulations, see below.

| Characteristics | VSAT (NOTE 2) | Handheld | Other (NOTE 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band (i.e. 30 GHz UL and 20 GHz DL) | S band (i.e. 2 GHz) | Ka band (i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear: +/-45°X-pol | Linear: +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | TBD dBi per element |
| Antenna temperature | 150 K | 290 K | TBD K |
| Noise figure | 1.2 dB | 7 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | 0 dBi per element | TBD dBi per element |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |  |

TR 38.821: Table 6.1.1.1-3: UE characteristics for system level simulations

Note 1: ESIM (Earth stations in motion) [1] address a complex challenge – how to provide reliable and high-bandwidth Internet services to what are – literally – moving targets. They provide broadband communications, including Internet connectivity, on platforms in motion. There are currently three types of ESIM: ESIM on aircraft (aeronautical ESIM), ESIM on ships (maritime ESIM) and ESIM on land vehicles (land ESIM).

Note 2: The user link (UE-satellite) is based on Uu NR interface in FDD mode with DFT-s-OFDM in uplink and CP-OFDM in the downlink.

Note 3: The feeder link (gateway – satellite) may operate in any frequency outside the user link frequency bands in consideration. One can assume that it is dimensioned to produce negligible degradation to the user link (satellite – user equipment) performances.

Note 4: It is assumed that the satellite (and gateway in cases of transparent payload) is dimensioned to close the link budget between the gNB and UE (satellite terminal), including effects of the feeder link.

Note 4: As part of this work, some reference RF performances for satellite and possibly for the Gateway may be considered.

# Conclusions

Proposal 1: RAN4 work should consider an exemplary FR2 band for NTN.

Proposal 2: RAN4 to use an FR2 exemplary band of 17.7 – 20.2 GHz for DL and 27.5 – 30.0 GHz for UL with FDD duplex mode.

# Reference

- https://www.itu.int/en/mediacentre/backgrounders/Pages/Earth-stations-in-motion-satellite-issues.aspx

- TR 38.821

-END-