3GPP TSG-RAN4 Meeting #97-e	R4-2014468

E-meeting, 2  – 18 November, 2020

Source: 	Ligado Networks

Title: 	A-MPR and Emission Requirements for new SUL Band related to the UL of n24

Agenda item:	10.30.1

Document for: 	Approval

# Introduction

This document discusses and presents a proposal for developing the A-MPR and additional emission requirements for the new SUL band, n99, related to the UL of n24.

# Discussion

On April 19, 2020, the FCC adopted an Order [1] updating the technical rules for terrestrial deployment utilizing spectrum associated with Band 24.  As part of the Order, FCC prohibits DL transmission in 1545 – 1555 MHz frequency.  This results in the 10 MHz of the corresponding UL spectrum unusable for FDD operation.  However, this unused mid-band UL spectrum can be paired with high frequency spectrum bands as a supplemental uplink to enhance the NR operation for these high frequency bands.

A Work Item (WI) to introduce SUL for the UL of n24 was approved in RAN#87e [2].  One of the objectives in this WI was to specify “ transmitter requirements such as maximum output power, A-MPR and spurious emissions per updated FCC rules”.

Since the transmitter requirements for the new SUL band, n99 and the UL of n24 are going to be the same, it is proposed that the requirements for n99 be specified by referring to the appropriate A-MPR and additional emission requirements developed as part of the work item related to the introduction of n24 [3].

# Proposal

The A-MPR and additional emissions requirements for the SUL band, n99, be specified by reference to tables and requirements developed for the UL of n24 as part of work item related to the introduction of n24.

# References

[1] FCC Order DA 20-48A1 https://ecfsapi.fcc.gov/file/04222926711874/FCC-20-48A1.pdf

[2] RP-201251, New WID on introduction 1.6 GHz NR SUL band with same uplink frequency range of Band 24

[3] RP-201357, New WID proposal for Introduction of NR band n24