3GPP TSG-RAN WG4 Meeting # 97-e	R4-2014495

# Electronic Meeting, 2-13 Nov., 2020

Source: 	Skyworks Solutions, Inc.

Title: 	Band 24 and n24 A-MPR

Agenda Item:	10.29.1 UE RF (38.101-1) [NR_band_n24-Core]

Document for:	Approval

# Introduction

In the RAN4#96 e-meeting a way forward [1] was agreed in order to evaluate Band n24 A-MPR related to the FCC GNSS protection requirement. In this contribution we provide input on filter feasibility, A-MPR evaluation assumptions and preliminary back-off measurements for NR FDD and SUL Band n24 that is also relevant to LTE Band 24.

# Discussion

## Band 24 Characteristics and Duplexer Requirement

Based from the input to the work item, Table 1 provides:

- The Band 24 characteristics

- The FCC regulation requirements converted to dBm/BW

- Related duplexer design constraints

Table 1: Band 24 characteristics, regulation and filter requirements.

| Band | Flow [MHz] | Fhigh [MHz] | Requirement | Duplexer design constraint |
| --- | --- | --- | --- | --- |
| lower DL (Cell) | 1526 | 1536 | lowest REFSENS | TX-RX isolation >50dB, Low IL |
| Upper DL (Sat) | 1545 | 1555 | -75dBm/MHz | No used, assume no RX filter requirement |
| Lower UL | 1627.5 | 1637.5 | Lowest post PA losses | TX-RX isolation >50dB, Low IL |
| Upper UL | 1646.5 | 1656.5 | Lowest post PA losses | TX-RX isolation >50dB, Low IL |
| Aux DL | 1670 | 1675 | na | No used, assume no TX filter requirement |
| FCC regulation | Flow [MHz] | Fhigh [MHz] | Requirement | Duplexer design constraint |
| Sat+GNSS | 1541 | 1608 | -75dBm/MHz | Best possible TX filter attenuation |
| Ramp1 | 1608 | 1610 | +2.5dB/MHz from -75 to -70dBm/MHz | Best possible TX filter attenuation |
| Ramp2 | 1610 | 1627.5 | +4.4dB/MHz from -70 to 7dBm/MHz | Sharp TX filter transition below UL |
| Narrow PSD1 | 1627.5 |  | -37dBm/4kHz | No TX filter attenuation feasible |
| Narrow PSD2 | 1638.5 | 1645.5 | -28dBm/4kHz | No TX filter attenuation feasible |
| Narrow PSD3 | 1657.5 | 1660.5 | -28dBm/4kHz | No TX filter attenuation feasible |

Since we provided our duplexer design feasibility data anonymously to the WI proponent we will not provide here our exact filter specification. However, in Figure 1, we provide the general behavior of our duplexer feasibility done with our baseline filter technology, in worst case condition for the FCC protection where the TX filter is shifted to the lower frequencies with curves for:

- TX to antenna attenuation

- RX to antenna attenuation

- TX to RX isolation

Additionally, at the top of Figure 1, the different frequency ranges are visualized for:

- UL/DL bands in green

- Auxiliary DL bands in black

- FCC protection ranges:

  - Flat -75dBm/MHz part in plain red

  - Ramp part in dashed red

Figure 1: Duplexer feasibility curves in worst case condition for FCC regulation.

Observation:

- The relatively large worst case duplex distance allows a classical design with low in-band insertion losses and >50dB TX-RX isolations

- The design did not attempt to provide attenuation in the auxiliary DL bands

- Good TX attenuation >40dB is achieved below 1610 MHz

- The TX filter transition below the UL band is sharp but no significant attenuation is achieved between 1620 and 1627.5 MHz.

## PA and Filter assumptions for A-MPR Evaluation

For transmitter calibration and TRX impairments, the agreed 3GPP assumptions are used and based on our data, we propose the following filter crude assumptions as a first step. Some of those assumptions could be re-evaluated based on the A-MPR results. The linear extrapolation of the TX attenuation transition below the UL is good up to 1617 MHz and becomes slightly pessimistic above that frequency but this is a good approach because this is the region where the low order non-linearity of the PA will be critical.

Proposal 1 on A-MPR assumptions:

- 4dB post PA losses

- PA calibrated at 1dB MPR for -30dBc ACLR with 20MHz QPSK DFT-s-OFDM 100RB0 waveform (same can be used for LTE evaluation)

- TRX impairments:

  - -28dBc Image and carrier leakage (-25dBc for LTE)

  - -60dBc CIM3 and -70dBc CIM5 (same for LTE)

- Filter assumptions

  - TX-RX isolation > 50dB

  - TX attenuation in DL > 45dB

  - TX attenuation in 1541-1612 MHz > 40dB

  - TX attenuation in 1612-1620 MHz > -5 dB/MHz from 40dB to 0dB

  - TX attenuation 1620 -1627.5 0 dB.

## Measured Results

In order to evaluate the trade off in terms of filter attenuation and PA back-off, a number of PA measurements have been done using a mid-band PA at the lowest possible UL frequency and above proposed calibration points and frequency offsets adjusted accordingly. The results are shown in next chapter.

The following test experiment was done:

- 10MHz channel at 1632.5 MHz and 5MHz channel at 1630 and 1635 MHz

- Full and 1RB allocations:

  - 50RB00, 01RB51, 01RB00 for 10 MHz

  - 01RB24, 01RB00, 25RB00 for 5 MHz

- QPSK CP-OFDM (probably worst case) and DFT-s-OFDM (also valid for LTE) waveforms

- Output power from Pmax to Pmax-15 dB.

The following PA noise measurements were taken and evaluated using the filter assumptions:

- PA noise in RX channels

- WC PA noise in 1541-1608 MHz in 1 MHz measurement BW

- PA noise in 1 MHz measurement BW at 1609 to 1627 MHz in 1 MHz step

- PA noise in 100 kHz measurement BW at 1627.5 MH (to be extrapolated to 4kHz measurement BW)

- Image and carrier related IMD levels in 1 MHz measurement BW.

For the IMDs it is to be noted that:

- For the 10MHz channel,

  - IMD3 will fall from the 4kHz point to the ramp 2 region

  - IMD5 will fall from the 4kHz point to the ramp 1 region

  - IMD7-19 will fall from the 4kHz point to the GNSS protection region

  - IMD 21 will fall in DL

- For the lower 5 MHz channel,

  - IMD3-7 will fall from the 4kHz point to the ramp 2 region

  - IMD9 will fall from the 4kHz point to the ramp 1 region

  - IMD11-21 will fall from the 4kHz point to the GNSS protection region

- For the upper 5 MHz channel,

  - IMD3-9 will fall from the 4kHz point to the ramp 2 region

  - IMD11 will fall from the 4kHz point to the ramp 1 region

  - IMD13-21 will fall from the 4kHz point to the GNSS protection region.

### PA Noise in DL Channel at Maximum Power

Table 2 presents the PA noise at Pmax and equivalent thermal noise floor rise at the LNA input assuming a 50 dB TX-RX isolation in the last column.

Table 2: RX de-sense results.

| Ch BW | UL Freq | DL freq | waveform | Allocation | Noise BW | PA noise | PA noise | at LNA | at LNA | noise rise LNA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MHz | MHz | MHz |  |  | MHz | dBm/ BW | dBm/ Hz | RX/BW | RX/Hz | dB |
| 10 | 1632.5 | 1531 | DFT-s-OFDM | 50RB00 | 9.375 | -64.3 | -134.1 | -114.3 | -184.1 | 0.4 |
| 10 | 1632.5 | 1531 | DFT-s-OFDM | 01RB51 | 9.375 | -65.5 | -135.2 | -115.5 | -185.2 | 0.3 |
| 10 | 1632.5 | 1531 | DFT-s-OFDM | 01RB00 | 9.375 | -65.4 | -135.1 | -115.4 | -185.1 | 0.3 |
| 10 | 1632.5 | 1531 | CP-OFDM | 52RB00 | 9.375 | -61.2 | -130.9 | -111.2 | -180.9 | 0.8 |
| 10 | 1632.5 | 1531 | CP-OFDM | 01RB51 | 9.375 | -66.1 | -135.8 | -116.1 | -185.8 | 0.3 |
| 10 | 1632.5 | 1531 | CP-OFDM | 01RB00 | 9.375 | -66.1 | -135.8 | -116.1 | -185.8 | 0.3 |
| 5 | 1635 | 1533.5 | DFT-s-OFDM | 25RB00 | 4.515 | -68.3 | -134.9 | -118.3 | -184.9 | 0.3 |
| 5 | 1635 | 1533.5 | DFT-s-OFDM | 01RB24 | 4.515 | -68.6 | -135.2 | -118.6 | -185.2 | 0.3 |
| 5 | 1635 | 1533.5 | DFT-s-OFDM | 01RB00 | 4.515 | -68.5 | -135.1 | -118.5 | -185.1 | 0.3 |
| 5 | 1635 | 1533.5 | CP-OFDM | 25RB00 | 4.515 | -69.1 | -135.7 | -119.1 | -185.7 | 0.3 |
| 5 | 1635 | 1533.5 | CP-OFDM | 01RB24 | 4.515 | -69.1 | -135.6 | -119.1 | -185.6 | 0.3 |
| 5 | 1635 | 1533.5 | CP-OFDM | 01RB00 | 4.515 | -69.1 | -135.7 | -119.1 | -185.7 | 0.3 |
| 5 | 1630 | 1528.5 | DFT-s-OFDM | 25RB00 | 4.515 | -68.4 | -135.0 | -118.4 | -185.0 | 0.3 |
| 5 | 1630 | 1528.5 | DFT-s-OFDM | 01RB24 | 4.515 | -68.6 | -135.2 | -118.6 | -185.2 | 0.3 |
| 5 | 1630 | 1528.5 | DFT-s-OFDM | 01RB00 | 4.515 | -68.5 | -135.1 | -118.5 | -185.1 | 0.3 |
| 5 | 1630 | 1528.5 | CP-OFDM | 25RB00 | 4.515 | -69.2 | -135.7 | -119.2 | -185.7 | 0.3 |
| 5 | 1630 | 1528.5 | CP-OFDM | 01RB24 | 4.515 | -69.1 | -135.7 | -119.1 | -185.7 | 0.3 |
| 5 | 1630 | 1528.5 | CP-OFDM | 01RB00 | 4.515 | -69.2 | -135.7 | -119.2 | -185.7 | 0.3 |

Observation: Thermal noise floor rise at LNA input is negligible and no DL de-sense should be anticipated even with a -130 dBm/MHz PA output noise assumption.

### GNSS protection from 1541-1610MHz

Table 3 presents the PA noise in 1 MHz measurement BW in the 1541-1610 MHz at Pmax and the required TX filter attenuation to meet the FCC requirements in the last column.

Table 3: PA noise and required attenuation at Pmax for the 1541-1610 MHz range.

| CH BW | UL Freq | waveform | allocation | range | PAnoise | MinAtt |
| --- | --- | --- | --- | --- | --- | --- |
| MHz | MHz |  |  | MHz | dBm/MHz | dB |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1541-1608 | -37.8 | 37.2 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1541-1608 | -34.8 | 40.2 |
| 5 | 1630 | DFT-s-OFDM | 25RB00 | 1541-1608 | -48.4 | 26.6 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 1541-1608 | -53.7 | 21.3 |
| 5 | 1630 | CP-OFDM | 25RB00 | 1541-1608 | -45.8 | 29.2 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1541-1608 | -51.4 | 23.6 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1609 | -36.8 | 35.7 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1609 | -33.0 | 39.5 |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 1609 | -51.0 | 21.5 |
| 5 | 1630 | DFT-s-OFDM | 25RB00 | 1609 | -46.9 | 25.6 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 1609 | -51.8 | 20.7 |
| 5 | 1630 | CP-OFDM | 25RB00 | 1609 | -43.8 | 28.7 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1609 | -49.7 | 22.8 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1610 | -35.2 | 34.8 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1610 | -32.5 | 37.5 |
| 5 | 1630 | DFT-s-OFDM | 25RB00 | 1610 | -46.8 | 23.2 |
| 5 | 1630 | CP-OFDM | 25RB00 | 1610 | -42.5 | 27.5 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1610 | -48.8 | 21.2 |

Observation: The only cases where a 40 dB TX attenuation assumption is marginal is for CP-OFDM fully allocated 10 MHz channel but it would benefit from a 3dB MPR in any case and thus can be discounted. This is confirmed in Table 4 with margins at 2dB MPR .

Table 4: PA noise and required attenuation at Pmax-2 dB for CP-OFDM marginal case for the 1541-1610 MHz range.

| CHBW | UL Freq | waveform | allocation | range | PAnoise | MinAtt |
| --- | --- | --- | --- | --- | --- | --- |
| MHz | MHz |  |  | MHz | dBm/MHz | dB |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1541-1608 | -39.5 | 35.5 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1609 | -37.5 | 37.5 |

Proposal 2: TX filter Attenuation > 40dB in 1541-1610 MHz is adopted and guaranties that FCC regulation is met at A-MPR = MPR.

### PA Noise in 4 kHz Measurement BW at 1627.5 MHz

Table 5 presents the PA noise in 100 kHz measurement BW versus Pout at 1627.5 MHz, its conversion into a 4kHz measurement BW, and the margin to the -37dBm/4kHZ requirement in the last column. The cases and power range is narrowed to the margin of +/-3dB except for the upper 5 MHz channel showing good margin at Pmax. No filter attenuation is assumed.

Table 5: PA noise and margin vs -37 dBm/4kHz requirement.

| Ch BW | UL Freq | waveform | allocation | ANTout | PAnoise | PAnoise | Margin |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MHz | MHz |  |  | dBm | dBm/100kHz | dBm/4kHZ | dBm/4kHz |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 22 | -22.4 | -36.4 | 0.6 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 21 | -27.6 | -41.6 | -4.6 |
| 10 | 1632.5 | DFT-s-OFDM | 01RB00 | 21 | -22.0 | -36.0 | 1.0 |
| 10 | 1632.5 | DFT-s-OFDM | 01RB00 | 20 | -23.3 | -37.3 | -0.3 |
| 10 | 1632.5 | DFT-s-OFDM | 01RB00 | 19 | -24.4 | -38.4 | -1.4 |
| 10 | 1632.5 | DFT-s-OFDM | 01RB00 | 18 | -25.6 | -39.6 | -2.6 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 21 | -19.9 | -33.9 | 3.1 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 20 | -24.0 | -38.0 | -1.0 |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 20 | -21.9 | -35.9 | 1.1 |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 19 | -23.8 | -37.8 | -0.8 |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 18 | -25.2 | -39.1 | -2.1 |
| 5 | 1630 | DFT-s-OFDM | 25RB00 | 21 | -22.7 | -36.7 | 0.3 |
| 5 | 1630 | DFT-s-OFDM | 25RB00 | 20 | -26.9 | -40.8 | -3.8 |
| 5 | 1630 | DFT-s-OFDM | 01RB00 | 15 | -21.4 | -35.4 | 1.6 |
| 5 | 1630 | DFT-s-OFDM | 01RB00 | 14 | -22.6 | -36.6 | 0.4 |
| 5 | 1630 | DFT-s-OFDM | 01RB00 | 13 | -23.4 | -37.4 | -0.4 |
| 5 | 1630 | DFT-s-OFDM | 01RB00 | 12 | -24.8 | -38.7 | -1.7 |
| 5 | 1630 | DFT-s-OFDM | 01RB00 | 11 | -25.7 | -39.7 | -2.7 |
| 5 | 1630 | CP-OFDM | 25RB00 | 20 | -19.8 | -33.8 | 3.2 |
| 5 | 1630 | CP-OFDM | 25RB00 | 19 | -23.7 | -37.7 | -0.7 |
| 5 | 1630 | CP-OFDM | 25RB00 | 18 | -27.1 | -41.1 | -4.1 |
| 5 | 1630 | CP-OFDM | 01RB00 | 15 | -21.3 | -35.2 | 1.8 |
| 5 | 1630 | CP-OFDM | 01RB00 | 14 | -22.2 | -36.2 | 0.8 |
| 5 | 1630 | CP-OFDM | 01RB00 | 13 | -23.3 | -37.2 | -0.2 |
| 5 | 1630 | CP-OFDM | 01RB00 | 12 | -24.4 | -38.4 | -1.4 |
| 5 | 1630 | CP-OFDM | 01RB00 | 11 | -25.5 | -39.5 | -2.5 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 23 | -37.8 | -51.7 | -14.7 |
| 5 | 1635 | DFT-s-OFDM | 01RB24 | 23 | -61.4 | -75.4 | -38.4 |
| 5 | 1635 | DFT-s-OFDM | 01RB00 | 23 | -58.5 | -72.5 | -35.5 |
| 5 | 1635 | CP-OFDM | 25RB00 | 23 | -32.7 | -46.7 | -9.7 |
| 5 | 1635 | CP-OFDM | 01RB24 | 23 | -61.3 | -75.3 | -38.3 |
| 5 | 1635 | CP-OFDM | 01RB00 | 23 | -57.7 | -71.7 | -34.7 |

Observations for the 1627.5 MHz -37dBm/4kHz case:

- For the lower 5 MHz channel:

  - 1RB0 for the 5MHz lower channel requires at least 12dB back-off. This back-off will depend on narrow allocation position and BW and further study is needed

  - Full allocation requires at least 5 dB back-off

- The upper 5 MHz channel does not require back-off but it may not be the worst case depending on IMD5 position of 1RB

- For the 10 MHz channel: 1RB and Full allocation CP-OFDM requires at least 3dB back-off and DFT-s-OFDM at least 1dB back-off. A-MPR set slightly higher than MPR should be sufficient.

Table 6 presents the PA noise in 100 kHz measurement BW at MPR at 1627.5 MHz and its conversion into a 4kHz measurement BW is noted in the last column. This is indicative of the possibility to meet the -28 dBm/4kHz requirement. No filter attenuation is assumed.

Table 6: PA noise at Pmax vs -28 dBm/4kHz requirement.

| Ch BW | UL Freq | waveform | allocation | ANTout | PAnoise | PAnoise |
| --- | --- | --- | --- | --- | --- | --- |
| MHz | MHz |  |  | dBm | dBm/100kHz | dBm/4kHZ |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 22 | -22.4 | -36.4 |
| 10 | 1632.5 | DFT-s-OFDM | 01RB51 | 22 | -47.1 | -61.0 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 22 | -22.4 | -36.4 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 22 | -39.8 | -53.8 |
| 5 | 1635 | DFT-s-OFDM | 01RB24 | 22 | -61.6 | -75.6 |
| 5 | 1635 | DFT-s-OFDM | 01RB00 | 22 | -59.8 | -73.8 |
| 5 | 1630 | DFT-s-OFDM | 25RB00 | 22 | -18.4 | -32.4 |
| 5 | 1630 | DFT-s-OFDM | 01RB24 | 22 | -41.6 | -55.5 |
| 5 | 1630 | DFT-s-OFDM | 01RB00 | 22 | -13.4 | -27.4 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 20 | -24.0 | -38.0 |
| 10 | 1632.5 | CP-OFDM | 01RB51 | 20 | -48.8 | -62.8 |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 20 | -21.9 | -35.9 |
| 5 | 1635 | CP-OFDM | 25RB00 | 20 | -38.0 | -52.0 |
| 5 | 1635 | CP-OFDM | 01RB24 | 20 | -65.5 | -79.5 |
| 5 | 1635 | CP-OFDM | 01RB00 | 20 | -63.3 | -77.2 |
| 5 | 1630 | CP-OFDM | 25RB00 | 20 | -19.8 | -33.8 |
| 5 | 1630 | CP-OFDM | 01RB24 | 20 | -43.4 | -57.4 |
| 5 | 1630 | CP-OFDM | 01RB00 | 20 | -15.0 | -28.9 |

Observation for the -28dBm/4kHz cases: MPR is marginal for the lower 5 MHz channel 1RB case.

### PA Noise in 1 MHz BW for 1620-1627.5 MHz

Table 7 presents the PA noise in 1 MHz measurement BW vs power in the 1620-1627.5 range which does not benefit from the TX filter attenuation and the margin in the last column. The table is limited to the cases with less than 3dB margin except for the last rows (full row in grey) which show the worst cases for upper 5 MHz channel at Pmax.

Table 7: PA noise in ramp part without attenuation.

| Ch BW | UL freq | waveform | allocation | Meas Freq | ANTout | PAnoise | PSD spec | Margin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MHz | MHz |  |  | MHz | dBm | dBm/MHz | dBm/MHz | dB |
| 5 | 1630 | CP-OFDM | 25RB00 | 1626 | 23 | -1.8 | 0.4 | -2.2 |
| 5 | 1630 | CP-OFDM | 25RB00 | 1625 | 23 | -5.5 | -4.0 | -1.5 |
| 5 | 1630 | CP-OFDM | 25RB00 | 1620 | 23 | -28.3 | -26.0 | -2.3 |
| 5 | 1630 | CP-OFDM | 01RB00 | 1623 | 23 | -15.8 | -12.8 | -3.0 |
| 5 | 1630 | DFT-s-OFDM | 25RB00 | 1620 | 23 | -28.5 | -26.0 | -2.5 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1626 | 23 | -2.5 | 0.4 | -2.9 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1625 | 23 | -3.8 | -4.0 | 0.2 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1624 | 23 | -5.3 | -8.4 | 3.1 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1624 | 22 | -9.4 | -8.4 | -1.0 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1623 | 23 | -6.9 | -12.8 | 5.9 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1623 | 22 | -11.0 | -12.8 | 1.8 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1623 | 21 | -15.0 | -12.8 | -2.2 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1622 | 23 | -9.0 | -17.2 | 8.2 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1622 | 22 | -13.0 | -17.2 | 4.2 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1622 | 21 | -16.9 | -17.2 | 0.3 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1621 | 23 | -11.6 | -21.6 | 10.0 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1621 | 22 | -15.8 | -21.6 | 5.8 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1621 | 21 | -19.5 | -21.6 | 2.1 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1621 | 20 | -22.9 | -21.6 | -1.3 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1620 | 23 | -15.6 | -26.0 | 10.4 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1620 | 22 | -19.7 | -26.0 | 6.3 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1620 | 21 | -23.1 | -26.0 | 2.9 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1620 | 20 | -25.8 | -26.0 | 0.2 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1620 | 19 | -28.1 | -26.0 | -2.1 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1623 | 23 | -12.9 | -12.8 | -0.1 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1623 | 22 | -17.4 | -12.8 | -4.6 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1622 | 23 | -13.9 | -17.2 | 3.3 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1622 | 22 | -18.3 | -17.2 | -1.1 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1622 | 21 | -23.2 | -17.2 | -6.0 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1621 | 23 | -16.2 | -21.6 | 5.4 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1621 | 22 | -20.3 | -21.6 | 1.3 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1621 | 21 | -24.6 | -21.6 | -3.0 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1620 | 23 | -21.3 | -26.0 | 4.7 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1620 | 22 | -24.2 | -26.0 | 1.8 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1620 | 21 | -26.2 | -26.0 | -0.2 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1620 | 20 | -28.3 | -26.0 | -2.3 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1623 | 23 | -31.6 | -12.8 | -18.8 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1622 | 23 | -33.0 | -17.2 | -15.8 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 1622 | 23 | -35.9 | -17.2 | -18.7 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1621 | 23 | -34.5 | -21.6 | -12.9 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 1621 | 23 | -37.7 | -21.6 | -16.1 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 1620 | 23 | -40.0 | -26.0 | -14.0 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1620 | 23 | -35.9 | -26.0 | -9.9 |

Observations for the 1620-1627.5 MHz range:

- For the 5 MHz channels: MPR should be sufficient for the measured cases, more measurements are needed for different 1RB positions

- For the 10 MHz channel:

  - For full allocation CP-OFDM MPR may be sufficient except at 1620 MHz where 1dB extra back-off is needed

  - For full allocation DFT-s-OFDM:

    - At least 3dB back off is needed at 1620 MHz

    - At least 2dB back off is needed at 1622 and 1621 MHz

    - More measurements are needed for different 1RB positions.

### PA Noise in 1 MHz BW for 1610-1620 MHz

Table 8 presents the PA noise in 1 MHz measurement BW vs power in the 1612-1620 range, the proposed TX filter attenuation and the margin in the last column. The table is limited to the cases with less than 5dB margin except for the last rows (full row in grey) which show the worst cases for upper 5 MHz channel at Pmax.

Table 8: PA noise in ramp part with attenuation.

| Ch BW | UL freq | waveform | allocation | Meas Freq | ANTout | PAnoise | PSD spec | Filter Att | margin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MHz | MHz |  |  | MHz | dBm | dBm/MHz | dBm/MHz | dB | dB |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 1619 | 23 | -13.3 | -30.4 | 5 | 12.1 |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 1619 | 22 | -16.7 | -30.4 | 5 | 8.7 |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 1619 | 21 | -20.6 | -30.4 | 5 | 4.8 |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 1619 | 20 | -22.8 | -30.4 | 5 | 2.6 |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 1619 | 19 | -27.2 | -30.4 | 5 | -1.8 |
| 10 | 1632.5 | CP-OFDM | 01RB00 | 1618 | 23 | -29.4 | -34.8 | 10 | -4.6 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1620 | 23 | -15.6 | -26.0 | 0 | 10.4 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1620 | 22 | -19.7 | -26.0 | 0 | 6.3 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1620 | 21 | -23.1 | -26.0 | 0 | 2.9 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1620 | 20 | -25.8 | -26.0 | 0 | 0.2 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1620 | 19 | -28.1 | -26.0 | 0 | -2.1 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1619 | 23 | -21.1 | -30.4 | 5 | 4.3 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1619 | 22 | -25.3 | -30.4 | 5 | 0.1 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1619 | 21 | -27.5 | -30.4 | 5 | -2.1 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1618 | 23 | -23.9 | -34.8 | 10 | 0.9 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1618 | 22 | -27.9 | -34.8 | 10 | -3.1 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1617 | 23 | -25.3 | -39.2 | 15 | -1.1 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1617 | 22 | -28.9 | -39.2 | 15 | -4.7 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1616 | 23 | -26.7 | -43.6 | 20 | -3.1 |
| 10 | 1632.5 | CP-OFDM | 52RB00 | 1615 | 23 | -28.0 | -48.0 | 25 | -5.0 |
| 10 | 1632.5 | DFT-s-OFDM | 01RB00 | 1619 | 22.9 | -16.6 | -30.4 | 5 | 8.8 |
| 10 | 1632.5 | DFT-s-OFDM | 01RB00 | 1619 | 22 | -20.6 | -30.4 | 5 | 4.8 |
| 10 | 1632.5 | DFT-s-OFDM | 01RB00 | 1619 | 21 | -24.6 | -30.4 | 5 | 0.8 |
| 10 | 1632.5 | DFT-s-OFDM | 01RB00 | 1619 | 20 | -28.7 | -30.4 | 5 | -3.3 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1620 | 23 | -21.3 | -26.0 | 0 | 4.7 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1620 | 22 | -24.2 | -26.0 | 0 | 1.8 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1620 | 21 | -26.2 | -26.0 | 0 | -0.2 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1620 | 20 | -28.3 | -26.0 | 0 | -2.3 |
| 10 | 1632.5 | DFT-s-OFDM | 50RB00 | 1619 | 23 | -27.2 | -30.4 | 5 | -1.8 |
| 5 | 1630 | CP-OFDM | 25RB00 | 1620 | 23 | -28.3 | -26.0 | 0 | -2.3 |
| 5 | 1630 | CP-OFDM | 25RB00 | 1619 | 23 | -29.9 | -30.4 | 5 | -4.5 |
| 5 | 1630 | DFT-s-OFDM | 25RB00 | 1620 | 23 | -28.5 | -26.0 | 0 | -2.5 |
| 5 | 1630 | DFT-s-OFDM | 25RB00 | 1620 | 22 | -30.4 | -26.0 | 0 | -4.4 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1618 | 23 | -38.7 | -34.8 | 10 | -13.9 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1617 | 23 | -40.0 | -39.2 | 15 | -15.8 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 1617 | 23 | -43.5 | -39.2 | 15 | -19.3 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1616 | 23 | -41.3 | -43.6 | 20 | -17.7 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 1619 | 23 | -41.4 | -30.4 | 5 | -16.0 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 1618 | 23 | -42.4 | -34.8 | 10 | -17.6 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1619 | 23 | -37.2 | -30.4 | 5 | -11.8 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1615 | 23 | -42.7 | -48.0 | 25 | -19.7 |
| 5 | 1635 | DFT-s-OFDM | 25RB00 | 1620 | 23 | -40.0 | -26.0 | 0 | -14.0 |
| 5 | 1635 | CP-OFDM | 25RB00 | 1620 | 23 | -35.9 | -26.0 | 0 | -9.9 |

Observations for the 1610-1620 MHz range:

- For the 5 MHz channels: MPR should be sufficient for the measured cases, more measurements are needed for different 1RB positions

- For the 10 MHz channel:

  - For CP-OFDM at least 4dB back-off is needed in the worst cases at 1619 MHz with 1 RB and at 1620 MHz with full allocation, for other cases, MPR is sufficient

  - For DFT-s-OFDM: At least 3dB back off is needed at 1920 MHz for full allocation and at 1619 MHz for 1RB, for other cases, MPR is sufficient

- More measurements are needed for different 1RB positions.

### Study of IMDs Related to Image and Carrier

Due to a lack of time this part of the data could not be analyzed in time for the submission deadline but could provide insights on corner cases. If acceptable by other companies and time allows, this data could be provided in a revision of this document before the meeting.

# Conclusions

In this contribution, we provide inputs to the AMPR assumptions in terms of PA calibration and filter characteristics which results in the following proposal:

Proposal 1 on A-MPR assumptions:

- 4dB post PA losses

- PA calibrated at 1dB MPR for -30dBc ACLR with 20MHz QPSK DFT-s-OFDM 100RB0 waveform (same can be used for LTE evaluation)

- TRX impairments:

  - -28dBc Image and carrier leakage (-25dBc for LTE)

  - -60dBc CIM3 and -70dBc CIM5 (same for LTE)

- Filter assumptions

  - TX-RX isolation > 50dB

  - TX attenuation in DL > 45dB

  - TX attenuation in 1541-1612 MHz > 40dB

  - TX attenuation in 1612-1620 MHz > -5 dB/MHz from 40dB to 0dB

  - TX attenuation 1620 -1627.5 0 dB

Furthermore, a large set of measurements was performed to provide a preliminary input on A-MPR accounting for the proposed filter data. This resulted in one proposal confirming one filter assumption and numerous observations.

Please note that the DFT-S-OFDM cases are representative of LTE except for 1RB cases where TRX impairment would be 3dB higher and thus potentially result in higher back-off.

Proposal 2: TX filter Attenuation > 40dB in 1541-1610 MHz is adopted and guaranties that FCC regulation is met at A-MPR = MPR.

Observation: Thermal noise floor rise at LNA input is negligible and no DL de-sense should be anticipated even with a -130 dBm/MHz PA output noise assumption.

Observations for the 1627.5 MHz -37dBm/4kHz case:

- For the lower 5 MHz channel:

  - 1RB0 for the 5MHz lower channel requires at least 12dB back-off. This back-off will depend on narrow allocation position and BW and further study is needed

  - Full allocation requires at least 5 dB back-off

- The upper 5 MHz channel does not require back-off but it may not be the worst case depending on IMD5 position of 1RB

- For the 10 MHz channel: 1RB and Full allocation CP-OFDM requires at least 3dB back-off and DFT-s-OFDM at least 1dB back-off. A-MPR set slightly higher than MPR should be sufficient.

Observation for the -28dBm/4kHz cases: MPR is marginal for the lower 5 MHz channel 1RB case.

Observations for the 1620-1627.5 MHz range:

- For the 5 MHz channels: MPR should be sufficient for the measured cases, more measurements are needed for different 1RB positions

- For the 10 MHz channel:

  - For full allocation CP-OFDM MPR may be sufficient except at 1620 MHz where 1dB extra back-off is needed

  - For full allocation DFT-s-OFDM:

    - At least 3dB back off is needed at 1620 MHz

    - At least 2dB back off is needed at 1622 and 1621 MHz

    - More measurements are needed for different 1RB positions.

Observations for the 1610-1620 MHz range:

- For the 5 MHz channels: MPR should be sufficient for the measured cases, more measurements are needed for different 1RB positions

- For the 10 MHz channel:

  - For CP-OFDM at least 4dB back-off is needed in the worst cases at 1619 MHz with 1 RB and at 1620 MHz with full allocation, for other cases, MPR is sufficient

  - For DFT-s-OFDM: At least 3dB back off is needed at 1920 MHz for full allocation and at 1619 MHz for 1RB, for other cases, MPR is sufficient

- More measurements are needed for different 1RB positions.

# References

[1]	R4-2011820, 	WF on UE related work for n24, Ligado Networks, RAN4#96e