3GPP TSG-RAN WG4 Meeting # 97-e	R4-2014508

# Electronic Meeting, 2-13 Nov., 2020

Source: 	Skyworks Solutions, Inc., Apple Inc.

Title: 	PC2 UL CA Class B/C UE Architecture and MPR/A-MPR evaluation

Agenda Item:	12.2.2.4 HPUE for TDD intra-band contiguous UL CA [NR_RF_FR1_enh-Core]

Document for:	Approval

# Introduction

In the RAN#89 e-meeting a Release 17 FR1 UE RF enhancement Work Item [1] was approved to complement Release 16 PC3 NR UL CA to cover PC2 HPUE for contiguous UL CA. This contribution discusses the transmitter architecture options and related preliminary MPR and A-MPR results valid for PC2 PA in class B and C UL CA.

# Discussion

The Work Item for [1] HPUE targets 26dBm PC2 intra-band contiguous UL CA. It uses CA_n41C, CA_n78C as the exemplary combinations to develop the general specification (MPR/NS_01) and NS_04 A-MPR should be developed for n41C.

## Implementation Options

One aspect to be agreed is the baseline architecture assumption to develop the requirement for HPUE class C UL CA.

For PC3 class C UL CA, the baseline architecture is based on a single PA covering up to 200 MHz bandwidth, 2 PA solutions are allowed but only to achieve the larger bandwidth and thus require two PC3 PAs and shall meet the same MPR than the one PA case. Furthermore, this two PA option does not enable the support of UL CA and UL MIMO.

For PC2, the same approach is feasible as the PC2 and PC3 PAs are based on a common technology and can support the full bandwidth. With an additional PA, UL MIMO can be supported.

For PC2 single carrier implementations, using two PC3 PAs was promoted for bands n41 and n78 for China while 1 PC2 PA is used for n41 in the U.S. and is currently the baseline for the single CC PC2 MPR.

When comparing the two options, the two PC3 PA approach result in higher MPR due to slightly lower linearity of each PA (30dB ACLR PA for a 31dB requirement for the combination) and additional non-linearity due to reverse IMD. At this stage, the 2Tx PC2 MPR (Using TX diversity or UL MIMO) is still not finalized in Release 16. This two PC3 PA approach will face further challenges with the forward IMDs related to the 2CC 1RB+1RB cases and the larger bandwidth.

Because of all these aspects, the single PC2 PA approach is the only mature solution to develop PC2 UL CA requirements.

Proposal 1 on HPUE architecture for class C contiguous UL CA:

- A single TX PC2 PA is the baseline to develop requirement

- Additional requirement for two PC3 PAs architecture is FFS once single CC related requirements are finalized.

## Power Class 2 PA Measurements

PC2 PA measurements were performed using a band n41 PA calibrated according to the following 3GPP RAN4 assumptions:

- 4dB post PA losses

- 31dBc ACLR for 20 MHz QPSK DFT-s-OFDM 100RB0 waveform

The ACLR targets are based on PC2 and therefore ACLR for contiguous UL CA re-uses the related PC3 definition with a value of 31dB. The same SEM as with PC3 applies.

Proposal 2 on ACLR and SEM requirements for PC2 contiguous UL CA:

- PC2 ACLR is PC3 ACLR+1 dB at 31 dB

- PC2 SEM is the same as for PC3.

Similar QPSK waveform cases as those used for the Release 16 PC3 UL CA study were measured for back-off against NS01 and NS04 requirements and provided in Table 1:

Table 1: Measured waveforms

## Power Class 2 PA Measurements Summary

In this section, we only provide the worst cases back-off out of all the measured results that can be found in the corresponding annex sections. When large MPR/A-MPR numbers are found at maximum PA voltage, a lower voltage was evaluated and results are denoted with HV/LV. This is especially the case for non-contiguous allocations for NS04 requirements.

#### Contiguous Allocations

Table 2 provides the PC2 QPSK MPR/AMPR contiguous UL CA class C results for NS01/NSO4 for contiguous allocations, and compares them with PC3 specification. Inner+ case is an Outer allocation which is just at the edge of the Inner region.

Table 2: NS01 MPR and NS04 A-MPR for Contiguous allocations

| Cases |  | NS01 | Pmax [dBm] | PC2 MPR [dB] | PC3 MPR [dB] | NS04 | Pmax [dBm] | PC2 A-MPR [dB] | PC3 A-MPR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inner | DFT | ADL | 23.8 | 2.5 | 2.5 | -25 | 22.2 | 4 | 2.5 |
| Inner+ | DFT | ADL | 23.7 | 2.5 | 7 | -25 | 22.1 | 4 | 7 |
| Outer | DFT | ADL | 21.3 | 5 |  | ADL | 21.8 | 4.5 |  |
| Inner | CP | ADL | 23.3 | 3 | 3.5 | -25 | 21 | 5 | 3.5 |
| Inner+ | CP | ADL | 23.2 | 3 | 8 | -25 | 21 | 5 | 8 |
| Outer | CP | ADL | 20.5 | 5.5 |  | ADL | 20.4 | 6 |  |

Observations on contiguous allocations:

- PC3 Inner MPR is valid for PC2

- PC3 Outer MPR has margin for PC2

- This is justified by the fact that contiguous allocations are ACLR limited and that PC2 is specified with a 1dB better ACLR than PC3

- For NS04 AMPR the worst case inner and outer allocation are SEM limited thus with 3dB higher power, a larger back-off is needed at least for inner allocations.

Proposal 3 on contiguous allocations:

- Contiguous Inner and Outer allocations definition for PC3 is adopted for PC2 (1Tx)

- PC3 QPSK Inner MPR is adopted for PC2 (1Tx)

- PC3 QPSK Outer MPR is adopted for PC2 (1Tx) with a potential reduction due to better ACLR

- PC2 (1Tx) NS04 A-MPR is FFS but larger than PC3 especially for some inner allocation.

#### Non-contiguous Allocations

Table 3 provides the PC2 QPSK MPR/AMPR contiguous UL CA class C results for NS01/NSO4 for non-contiguous allocations, and compares them with PC3 specification. Inner+ case is an Outer1 allocation which is just at the edge of the Inner region and Outer1+ case is an Outer2 allocation which is just at the edge of the Outer1 region.

Table 3: NS01 MPR and NS04 A-MPR for Non-contiguous allocations

| Cases |  | NS01 | Pmax [dBm] |  | PC2 MPR [dB] |  | PC3 MPR [dB] | NS04 | Pmax [dBm] |  | PC2 A-MPR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HV | LV | HV | LV |  |  | HV | LV | HV | LV |
| Inner | DFT | S2H_I5 | 22.7 |  | 3.5 |  | 2.5 | I5-25 | 17.3 |  | 8 |  |
| Inner+ | DFT | S2L_I5 | 21.5 |  | 4.5 |  | 6 | I5-25 | 17.4 |  | 8 |  |
| Outer1 | DFT | S4L_I7 | 19.1 |  | 7 |  |  | I3L-25 | 11 | 10.7 | 15 | 15 |
| Outer1+ | DFT | S3L_I5 | 17.5 |  | 8.5 |  | 13 | I3L-25 | 11.8 | 11.2 | 14.5 | 14.5 |
| Outer2 | DFT | S4L_I5 | 15.6 | 11.8 | 10.5 | 14.5 |  | I3L-25 | 11.5 | 10.7 | 14.5 | 15 |
| Inner | CP | S2H_I5 | 22.6 |  | 3.5 |  | 3.5 | I5-25 | 18.1 |  | 9 |  |
| Inner+ | CP | S2L_I5 | 21.7 |  | 4.5 |  | 7 | I5-25 | 18 |  | 9 |  |
| Outer1 | CP | S4L_I7 | 18.8 |  | 7.5 |  |  | I3L-25 | 10.5 | 10.2 | 15.5 | 15.5 |
| Outer1+ | CP | S3L_I5 | 18.9 |  | 7.5 |  | 14 | I3L-25 | 11 | 10.2 | 15 | 15.5 |
| Outer2 | CP | S4L_I5 | 15.6 | 11.8 | 10.5 | 14.5 |  | I3L-25 | 11.5 | 10.2 | 14.5 | 15.5 |

Observations on non-contiguous allocations:

- In general, PC2 allocations requires higher MPR than PC3 and this is justified by the fact that the worst case allocations are limited by IMD3 or IMD5 reaching the SEM mask limit which is, in absolute values, the same as PC3 but with 3dB higher output power. We note that 1dB extra back-off should compensate for this as PC2 has a slightly better linearity at 31 dB ACLR instead of 30 dB.

- This issue is also the same for NS04 where narrow allocations need to meet -25dBm/MHz with their IMD3 or IMD5, at least 2 dB higher A-MPR is needed.

Proposal 4 on non-contiguous allocations:

- Contiguous Inner, Outer1 and Outer 2 allocations definition for PC3 is adopted for PC2

- PC3 QPSK MPR is adopted for PC2 (1Tx) with 1 dB additional back-off

- PC2 (1Tx) NS04 A-MPR is FFS but larger than PC3 especially for some inner allocation.

# Conclusions

In this contribution, we first discuss implementation options for PC2 contiguous UL AC, then, SEM and ACLR requirements resulting in the following proposals.

Proposal 1 on HPUE architecture for class C contiguous UL CA:

- A single TX PC2 PA is the baseline to develop requirement

- Additional requirement for two PC3 PAs architecture is FFS once single CC related requirements are finalized.

Proposal 2 on ACLR and SEM requirements for PC2 contiguous UL CA:

- PC2 ACLR is PC3 ACLR+1 dB at 31 dB

- PC2 SEM is the same as for PC3.

Furthermore, back-off measurements are presented for contiguous and non-contiguous allocation MPR and A-MPR.

Proposal 3 on contiguous allocations:

- Contiguous Inner and Outer allocations definition for PC3 is adopted for PC2 (1Tx)

- PC3 QPSK Inner MPR is adopted for PC2 (1Tx)

- PC3 QPSK Outer MPR is adopted for PC2 (1Tx) with a potential reduction due to better ACLR

- PC2 (1Tx) NS04 A-MPR is FFS but larger than PC3 especially for some inner allocation.

Proposal 4 on non-contiguous allocations:

- Contiguous Inner, Outer1 and Outer 2 allocations definition for PC3 is adopted for PC2

- PC3 QPSK MPR is adopted for PC2 (1Tx) with 1 dB additional back-off

- PC2 (1Tx) NS04 A-MPR is FFS but larger than PC3 especially for some inner allocation.

# References

[1]	RP-202088 New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1) in Rel-17, Huawei, RAN#89e

# Annexes

## Measurements Results for Contiguous Allocations

Table 4: NS_01 and NS_04 power capability for contiguous allocation

## Measurements Results for Non-Contiguous Allocations

Table 5: NS_01 and NS_04 power capability for non-contiguous allocation (HV)

Table 6: NS_04 power capability for non-contiguous allocation (LV)