3GPP TSG-RAN4#97e	R4-2014526

17th Aug – 28th Aug 2020

Source: 	Nokia, Telefonica

Title: 					TP for 38.717-03-01: CA_n1A-n8A-n78(2A)

Agenda Item: 			10.9.2

Document for:	Approval

# 1	Introduction

This is a text proposal to introduce CA_n1A-n8A-n78(2A) into TR 38.717-03-01 Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 1 band U.

# 2 	Text proposal

************************************Start of TP********************************************

## 6.19	CA_n1A-n8A-n78(2A)

### 6.19.1	Operating bands for CA

Table 6.19.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n8-n78 | n1 | 1920MHz | – | 1980MHz | 2110MHz | – | 2170MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300MHz | – | 3800MHz | 3300MHz | – | 3800MHz | TDD |

### 6.19.2	Channel bandwidths per operating band for CA

Table 6.19.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n8A-n78A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.19.3	Co-existence studies

Co-existence analysis is captured into REL16 TR 38.416-03-01 for CA_n1A-n8A-n78A.

### 6.19.4	∆TIB,c and ∆RIB,c values

Already in specification.

Table 6.19.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n8-n78 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.19.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n8-n78 | n1 | 0 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 6.19.5	REFSENS requirements

MSD requirements are captured in lower order combinations.

************************************End of TP********************************************