3GPP TSG-RAN WG4 Meeting #97-e		R4-2014540

E-meeting, 2 – 13 November, 2020

Agenda item:	7.6.3

Source:	Intel Corporation

Title:	Discussion on PDCCH-WUS requirements

Document for:	Discussion

# Introduction

In RAN4 #96-e meeting WF on power saving demodulation was approved [1]. In this paper we provide view on demodulation requirements for NR power saving WI.

# Discussion

In the previous RAN4 meeting the following agreements were reached on test methodologies for PDCCH-WUS requirements

- Option 1 Joint PDCCH-WUS/PDCCH test - Configure UE not to wake up when missing DCI format 2_6 in DRX-OFF period - “Wake-up indication” in DCI format 2_6 is set to ‘1’ - Transmit PDCCH-WUS in DRX-OFF period and PDCCH in DRX-ON period with the SNR reference value - No Power boosting for normal PDCCH - Verify that BLER of PDCCH meets the following performance requirement - Pm-dsgtotal = PPDCCH-WUS + (1- PPDCCH-WUS) PPDCCH = 1.099% or 1% - Option 2 - ps-WakeUp is not configured and “Wake-up indication” in DCI format 2_6 is set to ‘1’ - drx-LongCycleStartOffset is set to ms10. - Two search space sets for transmission of DCI format 2_6 is configured for UE, but there is only one position selected randomly to transmit DCI format 2_6 before each DRX on duration. - Avoid the impact of normal PDCCH to PDCCH-WUS by setting high enough power level for normal PDCCH during the test. - Transmit PDCCH-WUS in DRX-OFF period (opportunity for DRX) and PDCCH in DRX-ON period (On duration). - Verify that BLER of PDCCH meets the performance requirement. (0.1%)

Based on our understanding, the fundamental difference between these two options is just verification of PDCCH-WUS decoding impact on PDCCH performance in Option 1 and implicit verification of PDCCH-WUS performance by artificial improving of PDCCH performance in Option 2.

The benefits of Option 1 are the following:

- We can take existing Normal PDCCH test and just reuse existing requirements with proper PDCCH-WUS configuration selection without extra work on simulation alignment.

- It reflects the practical scenario where PDCCH-WUS performance is better than PDCCH performance.

- Potential testing time can be easily estimated (i.e. 10*Normal PDCCH testing time) which looks rather feasible.

Same time, the only benefit of Option 2 is implicit verification of PDCCH-WUS BLER performance (that BLER 0.1 % can be reached under certain SNR level) which based on our understanding is not the purpose of this test.

Therefore, we propose to use Option 1 for PDCCH-WUS requirements definition with Pm-dsgtotal = 1%.

Proposal 1:	Use the following test solution for PDCCH-WUS requirements:

- Configure UE not to wake up when missing DCI format 2_6 in DRX-OFF period

- “Wake-up indication” in DCI format 2_6 is set to ‘1’

- Transmit PDCCH-WUS in DRX-OFF period and PDCCH in DRX-ON period with the SNR reference value. No Power boosting for normal PDCCH

- Verify that BLER of PDCCH meets the following performance requirement: Pm-dsgtotal = 1%

Based on WF [1] the following simulation assumptions were agreed for further analysis

Table 1. Simulation assumption for PDCCH-WUS requirements

| Parameter | Value |  |
| --- | --- | --- |
|  | PDCCH-WUS | PDCCH |
| DCI format | 2_6 | 1_0, 1_1 |
| Bandwidth | 10MHz |  |
| SCS | 15kHz |  |
| Number of BS antennas | 1Tx, 2Tx |  |
| Number of UE antennas | 2Rx, 4Rx |  |
| DM-RS channel estimation | Realistic |  |
| Channel model | TDLA30-10, TDLC300-100 |  |
| DCI length (excluding 24bits CRS) | 12 bits, 36 bits, 30bits | 39bits, 52bits |
| Aggregation level | 8, 16 | 4, 8 |
| CORESET symbol | 2 |  |
| CORESET bandwidth | 48RB |  |
| Mapping type | Interleaved(size2), Not Interleaved | Interleaved(size2), Not Interleaved |
| REG bundle size | 6 |  |

For further analysis we take one of the existing PDCCH FDD test to understand which PDCCH-WUS configuration is more suitable to meet test purpose. Such candidate is Test 3 in Table 5.3.2.1.1-1 of TS 38.101-4 with the following PDCCH configuration

Table 2. Configuration of Normal PDCCH

| Parameter | Value |
| --- | --- |
|  | PDCCH |
| DCI format | 1_1 |
| Bandwidth | 10MHz |
| SCS | 15kHz |
| Number of BS antennas | 1Tx |
| Number of UE antennas | 2Rx |
| Channel model | TDLA30-10 |
| DCI length (excluding 24bits CRS) | 52bits |
| Aggregation level | 4 |
| CORESET symbol | 2 |
| CORESET bandwidth | 48RB |
| Mapping type | Not Interleaved |
| REG bundle size | 6 |

| PDCCH-WUS, Not Interleaved, 36 bits, AL 8 | PDCCH-WUS, Not Interleaved, 36 bits, AL 16 |
| --- | --- |
| PDCCH-WUS, Not Interleaved, 12 bits, AL 8 | PDCCH-WUS, Not Interleaved, 12 bits, AL 16 |
| Figure 1. Simulation analysis of joint PDDCH and PDCCH-WUS performance |  |

From these results we can observe that only for scenarios with PDCCH-WUS AL 8 and 36 bits PDCCH-WUS decoding has small impact on Normal PDCCH performance.

Proposal 2:	For FDD PDCCH-WUS demodulation requirements use the following assumptions:

- Reference Normal PDCCH configuration is based on Test 3 in Table 5.3.2.1.1-1 and Table 5.3.3.1.1-1 of TS 38.101-4

- PDCCH-WUS configuration: Non-interleaved mapping, 36 bits AL 16 or 12 bits AL 8.

# Conclusion

In this paper we provided view on demodulation requirements for power saving and made the following proposals:

Proposal 1:	Use the following test solution for PDCCH-WUS requirements:

- Configure UE not to wake up when missing DCI format 2_6 in DRX-OFF period

- “Wake-up indication” in DCI format 2_6 is set to ‘1’

- Transmit PDCCH-WUS in DRX-OFF period and PDCCH in DRX-ON period with the SNR reference value. No Power boosting for normal PDCCH

- Verify that BLER of PDCCH meets the following performance requirement: Pm-dsgtotal = 1%

Proposal 2:	For FDD PDCCH-WUS demodulation requirements use the following assumptions:

- Reference Normal PDCCH configuration is based on Test 3 in Table 5.3.2.1.1-1 and Table 5.3.3.1.1-1 of TS 38.101-4

- PDCCH-WUS configuration: Non-interleaved mapping, 36 bits AL 16 or 12 bits AL 8.

# References

- R4-2012645 “WF on power saving demodulation”, CATT, RAN4 #96-e, August 2020.