3GPP TSG-RAN WG4 Meeting #97-e		R4-2014542

E-meeting, 2 – 13 August, 2020

Agenda item:	7.8.1.1.2

Source:	Intel Corporation

Title:	Discussion on CSI requirements for Ultra-low BLER

Document for:	Discussion

# Introduction

In the previous RAN4 meeting way forward on ultra-low BLER CQI requirements was approved [1].

- Work assumption: Introduce ultra-BLER static CQI test cases under the assumption that test time is similar as Fixed MCS test cases; further discuss how to ensure reasonable test time with below possible options: - Decrease confidence level (from 99.999% ~95%) - Early pass/fail criteria - Note both options adopted not excluded. - How to define the requirement: - Median CQI, BLER for median CQI and median CQI+1 or median CQI - 1

In this paper we provide our view on CQI requirements for ultra-low BLER.

# Discussion

As a part of test feasibility analysis, methodology with early termination was considered. Based on TS 38.521-4, this methodology is not used for CQI requirements. Same time, the possibility to use early pass/fail methodology for CQI requirements was discussed. Based on our understanding, the following procedure can be considered for CQI testing:

- Set SNR equal the first of SNR from Test 1 (first set of SNR values)

- Median CQI search: collect 2000 wideband CQI reports

- Check whether BLER for Median CQI is higher or lower than target BLER (i.e. 10-5):

  - If DUT early passes the test than DUT BLER ≤ 10-5

  - If DUT early fails the test than DUT BLER > 10-5

- Check BLER for Median CQI+1 or Median CQI-1 (depending on result of Step 3)

- If DUT passes requirements for the first SNR from Test 1 (set in step 1) then SNR is changed by first SNR from Test 2 (second set of SNR values) and test is executed for new SNR value.

- If DUT fails requirements for the first SNR from Test 1 then SNR is changed by second SNR from Test 1 and test is executed for new SNR value.

For further analysis of CQI testing time, we prepared our calculation of single test time depending on DUT BLER and confidence level of pass/fail methodology (detailed parameters for all CL value are in Annex). Table 1 shows mean testing time of single test in minutes for scenarios with 15 kHz SCS.

Table 1. Mean testing time of single test for SCS 15 kHz, [min]

| BLER DUT | Confidence level |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 99.999% | 99.995% | 99.97% | 99.8% | 98.6% | 95% |
| 1e-1 | <1 | <1 | <1 | <1 | <1 | <1 |
| 1e-2 | <1 | <1 | <1 | <1 | <1 | <1 |
| 1e-3 | <1 | <1 | <1 | <1 | <1 | <1 |
| 1e-4 | 2 | 1 | 1 | 1 | 1 | 1 |
| 5e-5 | 6 | 5 | 4 | 3 | 2 | 2 |
| 1e-5 | 236 | 197 | 157 | 119 | 80 | 62 |
| 5e-6 | 54 | 46 | 38 | 30 | 22 | 19 |
| 1e-6 | 33 | 29 | 24 | 20 | 15 | 13 |
| 1e-7 | 35 | 30 | 25 | 20 | 14 | 12 |
| 1e-8 | 25 | 21 | 17 | 13 | 9 | 8 |
| Min test time to pass the test | 18 | 15 | 13 | 10 | 8 | 7 |

In Table 2 we provide our estimation of full CQI test under assumption that DUT passes the tests for first SNR points from Test 1 and Test 2 (see Annex for more details)

Table 2. Mean testing time of full CQI test for SCS 15 kHz, [min]

|  | Confidence level |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 99.999% | 99.995% | 99.97% | 99.8% | 98.6% | 95% |
| Test time, [min] | 70-472 | 60-394 | 50-314 | 42-238 | 32-160 | 28-124 |

From this analysis we can observe that CQI testing time can rather close to fixed MCS testing time (i.e. around 30 minutes) only for scenarios with CL 98.6% and 95%. Therefore, we propose to define CQI requirements for Table 3 verification only is case CL lower than 98.6% will be used for test methodology.

Proposal 1:	Define CQI requirements for CQI Table 3 verification only is case CL is lower than 98.6%

Also, similar to fixed MCS discussion, it is required to inform RAN5 if early/pass fail methodology with certain parameters will be agreed as feasible methodology for CQI requirements.

Proposal 2:	Inform RAN5 about RAN4 conclusion on feasible CQI testing methodology for CQI Table 3 verification.

# Conclusion

In this paper we provided view on CQI requirements for ultra-low BLER testing and made the following proposals:

Proposal 1:	Define CQI requirements for CQI Table 3 verification only is case CL is lower than 98.6%.

Proposal 2:	Inform RAN5 about RAN4 conclusion on feasible CQI testing methodology for CQI Table 3 verification.

# References

- R4-2012646 “Way Forward on ultra-low BLER requirements”, Ericsson, RAN4 #96-e, August 2020.

# Annex

Early/pass fail parameters for different CL

|  | Confidence level |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 99.999% | 99.995% | 99.97% | 99.8% | 98.6% | 95% |
| clp | 0.9999999 | 0.999999 | 0.99999 | 0.9999 | 0.999 | 0.9975 |
| df | 2e-7 | 8e-7 | 8e-6 | 8e-5 | 8e-4 | 4e-3 |

Potential options for DUT to pass the CQI tests for first SNR in each test (i.e. UE will be tested for two SNR points) and corresponding test time

- Option 1: DUT BLER is 1e-5 for Median CQI and 1e-3 for Median CQI+1 for both SNR points

| CL | Testing time, [min] |  |  |  |
| --- | --- | --- | --- | --- |
|  | Step 2 | Step 3 | Step 4 | Total |
| 99.999% | 2 x <1 | 2 x 236 | 2 x <1 | 472 |
| 99.995% | 2 x <1 | 2 x 197 | 2 x <1 | 394 |
| 99.97% | 2 x <1 | 2 x 157 | 2 x <1 | 314 |
| 99.8% | 2 x <1 | 2 x 119 | 2 x <1 | 238 |
| 98.6% | 2 x <1 | 2 x 80 | 2 x <1 | 160 |
| 95% | 2 x <1 | 2 x 62 | 2 x <1 | 124 |

- Option 2: DUT BLER is 1e-6 for Median CQI and 1e-4 for Median CQI+1 for both SNR points

| CL | Testing time, [min] |  |  |  |
| --- | --- | --- | --- | --- |
|  | Step 2 | Step 3 | Step 4 | Total |
| 99.999% | 2 x <1 | 2 x 33 | 2 x 2 | 70 |
| 99.995% | 2 x <1 | 2 x 29 | 2 x 1 | 60 |
| 99.97% | 2 x <1 | 2 x 24 | 2 x 1 | 50 |
| 99.8% | 2 x <1 | 2 x 20 | 2 x 1 | 42 |
| 98.6% | 2 x <1 | 2 x 15 | 2 x 1 | 32 |
| 95% | 2 x <1 | 2 x 13 | 2 x 1 | 28 |

- Option 3: DUT BLER is 5e-5 for Median CQI and 1e-6 for Median CQI-1 for both SNR points

| CL | Testing time, [min] |  |  |  |
| --- | --- | --- | --- | --- |
|  | Step 2 | Step 3 | Step 4 | Total |
| 99.999% | 2 x <1 | 2 x 6 | 2 x 33 | 78 |
| 99.995% | 2 x <1 | 2 x 5 | 2 x 29 | 68 |
| 99.97% | 2 x <1 | 2 x 4 | 2 x 24 | 56 |
| 99.8% | 2 x <1 | 2 x 3 | 2 x 20 | 46 |
| 98.6% | 2 x <1 | 2 x 2 | 2 x 15 | 34 |
| 95% | 2 x <1 | 2 x 2 | 2 x 13 | 30 |

Potential options for DUT to fail the CQI tests for first SNR in each test and corresponding test time

- Option 1: DUT BLER is 1e-7 for Median CQI and 1e-5 for Median CQI+1 for both SNR points

| CL | Testing time, [min] |  |  |  |
| --- | --- | --- | --- | --- |
|  | Step 2 | Step 3 | Step 4 | Total |
| 99.999% | 2 x <1 | 2 x 35 | 2 x 236 | 596 |
| 99.995% | 2 x <1 | 2 x 30 | 2 x 197 | 454 |
| 99.97% | 2 x <1 | 2 x 25 | 2 x 157 | 364 |
| 99.8% | 2 x <1 | 2 x 20 | 2 x 119 | 278 |
| 98.6% | 2 x <1 | 2 x 14 | 2 x 80 | 188 |
| 95% | 2 x <1 | 2 x 12 | 2 x 62 | 148 |

- Option 2: DUT BLER is 1e-3 for Median CQI and 5e-5 for Median CQI-1 for both SNR points

| CL | Testing time, [min] |  |  |  |
| --- | --- | --- | --- | --- |
|  | Step 2 | Step 3 | Step 4 | Total |
| 99.999% | 2 x <1 | 2 x <1 | 2 x 6 | 12 |
| 99.995% | 2 x <1 | 2 x <1 | 2 x 5 | 10 |
| 99.97% | 2 x <1 | 2 x <1 | 2 x 4 | 8 |
| 99.8% | 2 x <1 | 2 x <1 | 2 x 3 | 6 |
| 98.6% | 2 x <1 | 2 x <1 | 2 x 2 | 4 |
| 95% | 2 x <1 | 2 x <1 | 2 x 2 | 4 |