3GPP TSG-RAN WG4 Meeting #97-e		R4-2014546

E-meeting, 2 – 13 November, 2020

Agenda item:	7.10.1.1

Source:	Intel Corporation

Title:	Discussion on UE demodulation requirements for FR2 DL 256QAM

Document for:	Discussion

# Introduction

In the previous meeting WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM was agreed [1].

In this paper we provide our view on UE demodulation requirements for FR2 256QAM.

# Discussion

In the previous RAN4 meeting, the following agreements were reached on demodulation requirements:

- Rx impairment modelling and band agnostic requirements - Not explicitly model Rx impairment - Rank - Rank 1 only - MCS - Keep the agreement in the last meeting, i.e., use MCS 20 - Propagation condition - Use fading channel - In the next meeting, companies are encouraged to provide ideal and impairment results for both option 1A and option 1B, and down select one of the two options based on simulation results. - Option 1A: TDLA30-300 - Option 1B: TDLD30-75 - Note: extra effort on TDLD channel model simplification is needed. - 22.0dB for FR2 bands including n260 with 50MHz CBW. - Anritsu: Note that according to R4-2006352 the maximum ΔMBP,n is 0.75 dB , using the demodulation spreadsheet in TR 38.810 we estimate that under this scenario SNRs up to 22.0dB at baseband could be tested under 50MHz scenario.

Based on these agreements, the only open question is channel model for requirements definition (i.e. TDLA30-300 or TDLD3--75).

In Table 1 we provide the list of parameters used for link level analysis. In Figure 1 we provide the simulation results.

Table 1. Link level simulation assumptions.

| Parameter | Value |
| --- | --- |
| CBW | 50 MHz |
| SCS | 120kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-D 30ns delay spread, 35Hz Doppler frequency TDL-A 30ns delay spread, 300Hz Doppler frequency |
| MCS | MCS 20 in TS 38.214 Table 5.1.3.1-2 |
| Antenna configuration | 2x2 for Rank1 and Rank2, Low correlation |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPT-RS: 2 (every 2 RBs), LPT-RS: 1 (every 1 symbol) |
| Impairments | TX: EVM = 3% RX: No (Ideal) |

Figure 1. FR2 256QAM performance analysis.

In Table 3 summary of link level simulation results is presented.

Table 3. PDSCH performance summary

| Channel model | SNR for 70% of max T-put |  |
| --- | --- | --- |
|  | Alignment results | Impairment results |
| TDL-D | 17.8 | 20.3 |
| TDL-A | 18.9 | 21.4 |

For final requirements definition, the additional margin is added to average companies results (0.8 dB for 256QAM modulation). Based on this, we can conclude that testing point for TDL-A channel model will be very close to SNR limit 22 dB. Therefore, we propose to use TDL-D channel model for requirements definition.

Proposal 1:	Define FR2 256QAM demodulation requirements for TDL-D 30 ns 35 Hz channel model.

# Conclusion

In this paper we provided view on FR2 256QAM demodulation requirements definition and made the following proposals:

Proposal 1:	Define FR2 256QAM demodulation requirements for TDL-D 30 ns 35 Hz channel model.

# References

- R4-2012666 “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom, RAN4 #96-e, August 2020.