3GPP TSG-RAN WG4 Meeting #97e		R4-2014554

Electronic, 2nd – 13th November, 2020

Agenda item:	7.15.3.2.1

Source:	Intel Corporation

Title:	Simulation results for NR HST PRACH

Document for:	Discussion

# Introduction

Last meeting it was agreed to define HST PRACH demodulation requirements under multi-path fading propagation conditions [1]. In this contribution we provide simulation results for corresponding requirements definition.

# Discussion

## Simulation results

In this section, the ideal and impairment results for PRACH format 0 with restricted sets type A and B are presented. TDLC300-100 channel model was assumed for evaluations. Other simulation assumptions are fully aligned with previously agreed values[2].

Table 1. Simulation results for Format 0 Restricted Set Type A and Type B

| Format | SCS, kHz | Restricted Set | Tx/Rx | Frequency offset, Hz | Ideal, dB | Impairment, dB |
| --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 1.25 | Type A | 1T2R | 0 | -8,8 | -6,3 |
| Format 0 | 1.25 | Type A | 1T4R | 0 | -14,5 | -12 |
| Format 0 | 1.25 | Type A | 1T8R | 0 | -18,9 | -16.4 |
| Format 0 | 1.25 | Type B | 1T2R | 0 | -8,7 | -6,2 |
| Format 0 | 1.25 | Type B | 1T4R | 0 | -14,4 | -11,9 |
| Format 0 | 1.25 | Type B | 1T8R | 0 | -18,8 | -16.3 |

# Conclusion

In this contribution we provided simulation results for NR HST PRACH performance requirements definition TDLC300-100 channel model.

# References

- R4-2012676 “WF on Rel-16 NR HST BS demodulation requirements”, Nokia, RAN4 #96-e, August 2020

- R4-1915871 “WF on NR HST PRACH demodulation requirement”, Samsung, RAN4 #93, November 2019