3GPP TSG-RAN WG4 Meeting #97-e		R4-2014555

Electronic, 2nd – 13th November, 2020

Agenda item:	7.15.3.2.1

Source:	Intel Corporation

Title:	Simulation results for NR HST PUSCH

Document for:	Discussion

# Introduction

Last meeting it was agreed to define HST PUSCH demodulation requirements under multi-path fading propagation conditions [1]. In this contribution we provide corresponding simulation results and also address remaining simulation results from our side for HST Single tap scenario with additional CBWs.

# Discussion

## Simulation results for TDL-C

In this section, the ideal and impairment results for PUSCH TDL-C are presented.

Table 1. Simulation results for multi-path fading propagation conditions

| Propagation Conditions | SCS, kHz | Tx/Rx | MCS | Ideal, dB | Impairment, dB |
| --- | --- | --- | --- | --- | --- |
| TDLC300-600 | 15 | 1T2R | 2 | -9.8 | -7.3 |
| TDLC300-1200 | 30 | 1T2R | 2 | -9.8 | -7.3 |

## Simulation results for HST Single tap

In this section, the ideal and impairment results for HST Single tap for 350 km/h and 500 km/h UE with 5 MHz and 10 MHz CBW are presented.

Table 2. Simulation results for 350 km/h

| Propagation Conditions | SCS, kHz | CBW, MHz | Maximum Doppler shift, Hz | Tx/Rx | MCS | Ideal, dB | Impairment, dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tunnel | 15 | 5 | 1340 | 1T1R | 2 | -2.4 | 0.1 |
| Tunnel | 15 | 5 | 1340 | 1T2R | 2 | -5.3 | -2.8 |
| Tunnel | 30 | 10 | 2334 | 1T1R | 2 | -2.5 | 0 |
| Tunnel | 30 | 10 | 2334 | 1T2R | 2 | -5.4 | -2.9 |
| Open Space | 15 | 5 | 1340 | 1T2R | 2 | -5.4 | -2.9 |
| Open Space | 15 | 5 | 1340 | 1T2R | 16 | 6.7 | 9.2 |
| Open Space | 15 | 5 | 1340 | 1T8R | 2 | -10.7 | -8.2 |
| Open Space | 15 | 5 | 1340 | 1T8R | 16 | 1.3 | 3.8 |
| Open Space | 30 | 10 | 2334 | 1T2R | 2 | -5.4 | -2.9 |
| Open Space | 30 | 10 | 2334 | 1T2R | 16 | 6.3 | 8.8 |
| Open Space | 30 | 10 | 2334 | 1T8R | 2 | -10.8 | -8.3 |
| Open Space | 30 | 10 | 2334 | 1T8R | 16 | 0.4 | 2.9 |

Table 3. Simulation results for 500 km/h

| Propagation Conditions | SCS, kHz | CBW, MHz | Maximum Doppler shift, Hz | Tx/Rx | MCS | Ideal, dB | Impairment, dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tunnel | 15 | 5 | 1740 | 1T1R | 2 | -2.3 | 0.2 |
| Tunnel | 15 | 5 | 1740 | 1T2R | 2 | -5.2 | -2.7 |
| Tunnel | 30 | 10 | 3334 | 1T1R | 2 | -2.3 | 0.2 |
| Tunnel | 30 | 10 | 3334 | 1T2R | 2 | -5.3 | -2.8 |
| Open Space | 15 | 5 | 1740 | 1T2R | 2 | -5.3 | -2.8 |
| Open Space | 15 | 5 | 1740 | 1T2R | 16 | 7.1 | 9.6 |
| Open Space | 15 | 5 | 1740 | 1T8R | 2 | -10.6 | -8.1 |
| Open Space | 15 | 5 | 1740 | 1T8R | 16 | 1.8 | 4.3 |
| Open Space | 30 | 10 | 3334 | 1T2R | 2 | -5.4 | -2.9 |
| Open Space | 30 | 10 | 3334 | 1T2R | 16 | 6.7 | 9.2 |
| Open Space | 30 | 10 | 3334 | 1T8R | 2 | -10.8 | -8.3 |
| Open Space | 30 | 10 | 3334 | 1T8R | 16 | 1.1 | 3.6 |

# Conclusion

In this contribution we have provided simulation results for NR HST PUSCH performance requirements definition multi-path fading scenario and for Single tap with additional CWBs as well.

# References

- R4-2012676 “WF on Rel-16 NR HST BS demodulation requirements”, Nokia, RAN4 #96-e, August 2020