3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014567

# Electronic Meeting, 2nd, Nov.–13th, Nov., 2020

Agenda item:      7.13.2.2.4

Source:                Intel Corporation

Title:                    Discussion of testcase design for BWP switching on multiple CCs

Document for:    Discussion

# Introduction

In last meeting, the core part of BWP switching on multiple CCs is completed. The next step is test design in performance part. In this contribution, we provide our view regarding the test case design.

# Test case design discussion

For the testcase design for BWP switch for multiple CCs, the design methodology can be similar with single BWP switch. There are several issues which have already been identified during the pre-meeting email discussion. Here, we share our view.

Issue 1: Can we only consider 2CCs for multiple BWP switch?

The possible combination of CCs can be:

- FR1+FR1

- FR2 +FR2

- FR1+FR2

For FR1+FR2 case, if UE is capable of per-FR gap and there is no SCS changing, UE will handle BWP switching in FR1 and FR2 independently. If UE is not capable of per-FR gap, the total delay time will include the delay in FR1 and FR2 together, which is similar with case where all BWP switching happen in the same frequency range.  In order to reduce the test case number, we suggest only consider multiple BWP switch test case in the same FR and don’t consider FR1+FR2 CA case which needs to consider the UE capability of per-FR gap or not.

Next question is how many CCs will be considered for simultaneous BWP switching. For simplicity, we suggest the number is 2.

Proposal 1: For multiple BWP switching testcases, only consider BWP switches happens simultaneously in two CCs, i.e. FR1+FR1, FR2+FR2.

Issue 2: Shall we only consider simultaneous case or simultaneous + partial?

For partial overlap triggering, DCI and RRC based multiple BWP switch are defined for FR1+FR2 in NR-DC operation, when BWP switch doesn’t involve SCS change and UE supports per-FR gap.

From our point of view, the chance that two BWP switches collide with each other in NR-DC case is small. Don’t need to design testcase for partial overlap BWP switch on multiple CCs.

Proposal 2: Only define test case for simultaneous BWP switch on multiple CCs. Don’t need to define test case for partial overlap BWP switching on multiple CCs.

Issue 3: Shall all the test case be duplicated in both EN-DC and SA?

From our point of view, all the test case can be duplicated in both EN-DC and SA.

Proposal 3: all the testcase can be duplicated in both EN-DC and SA.

Issue 4: Similar as legacy Rel-15, Both DCI+Timer based BWP switch can be tested in one testcase.

The same test method can be used for multiple BWP switch which is similar with BWP switch in single CC.

Proposal 4: Both DCI+Timer based BWP switch can be tested in one testcase.

Issue 5: how many total Cells are considered

In legacy Rel-15, There are two scenarios for DCI/timer based BWP switch in SA. One is that there is only NR PCell and the other scenario is that one NR PCell+ NR SCell are considered. In order to reduce the testcase number, we suggest to only consider the NR PCell+ NR SCell case which are undergoing simultaneous two BWP switches. On the same time, one more NR SCell (as a victim cell) may be needed to verify NR interruption requirements as well, which is similar with the legacy single BWP switch test case. Therefore, there are totally 3 cells for DCI/timer based BWP switch in SA. For EN-DC case, since the interruption function can be verified in E-UTRAN cell, there are one PSCell + one SCell in NR cells. The total cell number is 3 as well.

Proposal 5: For DCI/timer based multiple BWP switch in EN-DC and SA case, there are totally 3 cells where two cells are undergoing simultaneous BWP switch and one another cell is used to verify interruption delay.

Issue 6: Cross-carrier scheduling based Simultaneous BWP switching

We suggest to define testcase for self carrier scheduling firstly. For cross-carrier scheduling, it might be discussed in dormancy SCell.

Proposal 6: Define testcase for self carrier scheduling firstly.

Issue 7: Need interruption test or not

Since interruption for BWP switch on multiple CCs is defined based on each CC respectively, we can verify it the same way as in singe BWP switch interruption case. As explained in issue 5, one more SCell can be added to test the interruption functionality in SA case.

Proposal 7: Similar with Rel-15, interruption test and delay test can be applied in one testcase.

Issue 8: Simultaneous RRC based BWP switch on multiple CCs scenario

In companied paper[1], we suggest to further discuss the  scenarios for Simultaneous RRC based BWP switch on multiple CCs. If the RRC signaling involves BWP switches for both PCell and SCell, the applied BWP switch timing may be different for PCell and SCell. The UE switch to firstActiveDownlinkBWP-id upon activation of an SCell. If the status of SCell is deactivated, extra activation time for SCell is needed. if BWP switches involves multiple SCells, it will not guarantee that multiple SCell activation will finish at the same time. Then the BWP switch on multiple SCells may not start at the same time.

Proposal 8: Before designing the test case, clarify the applicable scenario and delay requirement for simultaneous RRC based BWP switch on multiple CCs firstly.

# Test case list

Considering all the above discussions, there are totally 4 testcases.

Proposal 9:  The total testcase list is:

TC1: DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR1 in EN-DC TC2: DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR2 in EN-DC TC3: DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR1 in SA TC4: DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR2 in SA

# Conclusion

In this paper we provide our views on the remaining open issues related to BWP switching on multiple CCs. Our proposals are captured below:

Proposal 1: For multiple BWP switching testcases, only consider BWP switches happens simultaneously in two CCs, i.e. FR1+FR1, FR2+FR2.

Proposal 2: Only define test case for simultaneous BWP switch on multiple CCs. Don’t need to define test case for partial overlap BWP switching on multiple CCs.

Proposal 3: All the testcase can be duplicated in both EN-DC and SA.

Proposal 4: Both DCI+Timer based BWP switch can be tested in one testcase.

Proposal 5: For both EN-DC and SA case, there are totally 3 cells where two cells are undergoing simultaneous BWP switch and one another cell is used to verify interruption delay.

Proposal 6: Define testcase for self carrier scheduling firstly.

Proposal 7: Similar with Rel-15, interruption test and delay test can be applied in one testcase.

Proposal 8: Before designing the test case, clarify the applicable scenario and delay requirement for simultaneous RRC based BWP switch on multiple CCs firstly.

Proposal 9:  The total testcase list is:

TC1: DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR1 in EN-DC TC2: DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR2 in EN-DC TC3: DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR1 in SA TC4: DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR2 in SA

# Reference

[1] R4-2014570, Discussion of RRC based BWP switching on multiple CCs, Intel