3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014569

# Electronic Meeting, 2-13, Nov., 2020

Agenda item:      7.13.2.2.8

Source:                Intel Corporation

Title:                    Discussion on test case for UL spatial relation switch

Document for:    Discussion

# Introduction

In last meeting, the core part of UL spatial relation switch is completed. The next step is test design in performance part.In this contribution, we provide our view regarding the test case design.

# Test case discussion

According to the requirement agreement for UL spatial info switch:

#; Scenario; PHY channel; Triggering method; associated source; Whether to introduce requirement. 1; PUCCH-RRC-DL; PUCCH; RRC; DL RS; No. 2; PUCCH-RRC-UL; PUCCH; RRC; UL SRS; No. 3; PUCCH-MAC-DL; PUCCH; MAC; DL RS; Yes. 4; PUCCH-MAC-UL; PUCCH; MAC; UL SRS; No. 5; PUSCH-DCI; PUSCH; DCI; -; No. 6; pSRS-RRC-DL; P-SRS; RRC; DL RS; Yes. 7; pSRS-RRC-UL; P-SRS; RRC; UL SRS; No. 8; spSRS-MAC-DL; SP-SRS; MAC; DL RS; Yes. 9; spSRS-MAC-UL; SP-SRS; MAC; UL SRS; No. 10; aSRS-DCI-DL; A-SRS; DCI; DL RS; Yes. 11; aSRS-DCI-UL; A-SRS; DCI; UL SRS; No.

The scenarios where test cases may be needed in RAN4 are:

#; Scenario; PHY channel; Triggering method; associated source; Whether to introduce requirement. 1; PUCCH-MAC-DL; PUCCH; MAC; DL RS; Yes. 2; spSRS-MAC-DL; SP-SRS; MAC; DL RS; Yes. 3; pSRS-RRC-DL; P-SRS; RRC; DL RS; Yes. 4; aSRS-DCI-DL; A-SRS; DCI; DL RS; Yes.

Similar with DCI based TCI state switch, the delay time for DCI based uplink spatial relationship switch is small, therefore, it’s preferred that don’t define test case for DCI based uplink spatial relationship switch.

Both PUCCH and SP-SRS are supported with MAC CE based uplink spatial relationship switch. In order to reduce the testcase number, it’s suggested to define test case for MAC CE based uplink spatial relationship switch of PUCCH.

In summary, the total test lists is:

- FR2 EN-DC MAC-CE based uplink spatial relation switch for PUCCH

- FR2 EN-DC RRC based uplink spatial relation switch for pSRS

- FR2 SA MAC-CE based uplink spatial relation switch for PUCCH

- FR2 SA RRC based uplink spatial relation switch for pSRS

Proposal 1: The total test list for uplink spatial relation switch is:

- FR2 EN-DC MAC-CE based uplink spatial relation switch for PUCCH

- FR2 EN-DC RRC based uplink spatial relation switch for pSRS

- FR2 SA MAC-CE based uplink spatial relation switch for PUCCH

- FR2 SA RRC based uplink spatial relation switch for pSRS

# Conclusion

In this paper we provide our views on introducing test case for UL spatial relation info switching.

Proposal 1: The total test list for uplink spatial relation switch is:

- FR2 EN-DC MAC-CE based uplink spatial relation switch for PUCCH

- FR2 EN-DC RRC based uplink spatial relation switch for pSRS

- FR2 SA MAC-CE based uplink spatial relation switch for PUCCH

- FR2 SA RRC based uplink spatial relation switch for pSRS

# Reference

1.