3GPP TSG-RAN WG4 Meeting # 97-e 	                                              R4-2014571

Electronic Meeting, 2-13 Nov., 2020

Agenda Item:	7.7.3.1.

Source:	Intel Corporation

Title:	Discussion on NR Positioning test cases configuration and list

Document for:		Discussion

# Introduction

In the last RAN4 meeting, the RRM core requirements related to the positioning for NR were completed [1]. Accordingly, RAN4 is required to define the necessary test cases to verify the RRM requirements in TS38.133 [2].

In this paper we firstly provide the considerations on the basic test configurations for PRS based measurement. And the list of the test cases to verify the core requirements for NR positioning (e.g. RSTD, UE Rx-Tx time difference, PRS RSRP and E-CID) is proposed. Finally, a time plan for the completion of this work is also provided.

# Configuration Patterns for PRS

Similarly as SSB configurations for RRM test cases in TS38.133, it is more efficient to specify the common PRS configurations before the detailed test cases definition. Then each test case for NR positioning defined in TS38.133 can also refer to these PRS configuration patterns more clearly.

NR-DL-PRS-Info-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetList-r16		SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF

NR-DL-PRS-ResourceSet-r16,

...

}

NR-DL-PRS-ResourceSet-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16,

dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

dl-PRS-ResourceRepetitionFactor-r16	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,	-- Need OP

dl-PRS-ResourceTimeGap-r16			ENUMERATED {s1, s2, s4, s8, s16, s32, ...}

OPTIONAL, 	-- Cond Rep

dl-PRS-NumSymbols-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-MutingOption1-r16			DL-PRS-MutingOption1-r16			OPTIONAL,	-- Need OP

dl-PRS-MutingOption2-r16			DL-PRS-MutingOption2-r16			OPTIONAL,	-- Need OP

dl-PRS-ResourcePower-r16			INTEGER (-60..50),

dl-PRS-ResourceList-r16				SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF

NR-DL-PRS-Resource-r16,

...

}

Particularly, the above PRS parameters above which are configured by LMF for the different types of positioning method (e.g. RSTD, UE Rx-Tx time difference) can be included in these PRS patterns.

- PRS periodicity: Periodicity of DL PRS allocation in slots configured per DL PRS Resource Set (i.e. all DL PRS Resources of a given Set have the same periodicity). And in order to minimize the testing efforts 2^u *160ms periodicity can be taken consideration into the test case as the most typical deployment scenarios.

- PRS SCS and transmission BW: In our simulations [3], the impacts on the PRS RSTD and RSRP measurement accuracy due to the different BW can be observed. Thus, if the more than accuracy requirements regarding to the PRS BW is introduced in TS38.133, the multiple PRS BW under the difference numerologies is necessary to be defined in these PRS patterns.

- Number of PRS symbol and Comb size: To simplify the test case, the number of PRS symbol can be same as comb size. In the previous meeting the accuracy requirements agonistic to comb size is under debating [1]. As a start point, the one comb size can be assumed up to further discussions.

- Repetition factor: This parameter controls how many times each DL-PRS Resource is repeated for a single instance of the DL-PRS Resource Set. It is applied to all resources of DL PRS Resource Set. In [simulation assumptions], the possible candidates of this parameter can be {1, 2, 4, 8, 16, 32}. But the very high repetition is rarely happened in our view and introduce extra UE implementation complexity (e.g. cross-slots soft combination). Thus, the parameter for testing can be {1, 2}.

In summary, as examples in the table below several PRS patterns depending on these parameters combinations above can be defined.

Table 1: PRS configuration patterns for NR positioning measurement

|  | PRS Pattern 1 | PRS Pattern 2 | PRS Pattern 3 |
| --- | --- | --- | --- |
| SCS | FR1, 15k | FR1, 30k | FR2, 120k |
| PRS periodicity | 160ms | 80ms | 20ms |
| PRS transmission bandwidth in RBs (System Bandwidth) | 52 (10MHz) | 132 (50MHz) | 128(200MHz) |
| Number of PRS symbol | 2 | 4 | 6 |
| Comb size | 2 | 4 | 6 |
| Repetition factor | 4 | 2 | 1 |

Proposal 1: The PRS configuration patterns in Table 1 for NR Positioning measurement tests can be specified in the Annex A.3.x of TS 38.133.

Beside parameters above which can be included in the common PRS configuration patterns, the other parameters depending on measured cell/TRPs below shall be also considered.

- SRS configuration

When measuring UE Rx-Tx time difference, the SRS transmission parameters shall be also specified. We can also reuse the SRS configuring pattern 1 for timing accuracy test [2, 38.133] below:

Table A.6.4.1.1.1-3: SRS Configuration for Timing Accuracy Test[2]

|  | Field | SRSConf.1 | Comments |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |  |
|  | srs-ResourceIdList | 0 |  |
|  | resourceType | Periodic |  |
|  | Usage | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 |  |
|  | nrofSRS-Ports | Port1 |  |
|  | transmissionComb | n2 |  |
|  | combOffset-n2 | 0 |  |
|  | cyclicShift-n2 | 0 |  |
|  | resourceMapping startPosition | 0 |  |
|  | resourceMapping nrofSymbols | n1 |  |
|  | resourceMapping repetitionFactor | n1 |  |
|  | freqDomainPosition | 0 |  |
|  | freqDomainShift | 0 |  |
|  | freqHopping c-SRS | 14 for test configuration 1,2 25 for test configuration 3 | Matches NRB,c |
|  | freqHopping b-SRS | 0 |  |
|  | freqHopping b-hop | 0 |  |
|  | groupOrSequenceHopping | Neither |  |
|  | resourceType | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | Any 10 bit number |

Proposal 2: SRS configuration pattern 1 for timing accuracy test in [2] can be reused for NR positioning measurements.

- DRX

As no requirements for DRX case, we need not any test cases for DRX.

Proposal 3: NO DRX case will be tested only for NR positioning measurement requirements in Rel16.

- Number of cells

For RSTD measurement, preferably the total number of cells including the serving cell should be limited to 3 in order to avoid testing complexity, i.e. reporting of RSTD for up to 2 neigbour cells can be settable. We also assume that the serving cell is the reference cell used for RSTD measurements. The related parameter (e.g. ) can be defined in the cell-specific parameters. For other types of NR positioning measurement (e.g. UE Rx-Tx timing difference and PRS RSRP) the 2 cells (one is serving cell, the other neighbor cell) are needed.

Proposal 4-1:  3 cells deployment scenarios (one is serving/reference cell , the other two neighbor cells) can be used for NR RSTD .

Proposal 4-2:  2 cells deployment scenarios (one is serving/reference cell , the other neighbor cell) can be used for UE Rx-Tx time difference, PRS RSRP and E-CID measurement tests.

- Synchronous/Asynchronous

The positioning sub-frames between the cells can be synchronized or asynchronized within a slot (± 0.5 ms). However, the same measurement delay requirements will be applied for both scenarios [2]. On the other hand, in order to reduce the number of tests we propose that all tests are done using synchronous positioning sub-frames. If necessary all or some of the test cases are performed using asynchronous positioning sub-frames at a later stage for the accuracy requirements tests.

Proposal 5: The synchronous cells will be tested for the measurement delay requirements test.

- Muting

As there is no any difference of the core requirements (e.g. reporting delay) between the scenarios with and w/o muting), muting could not be applied in the positioning tests.

Proposal 6: For the core requirements test cases, only the non-muting PRS configuration will be used.

- Positioning frequency layer

In order to simplify our testing design , the test case for a single positioning frequency layer is preferred. That is a single PRS pattern is needed for all tested cells. On the other hand, to verify the measurement cases with multiple positioning frequency layers with minimum efforts and complexity, the more than one positioning frequency layer shall be considered in the testing parameters. For an instance, if in the test there are 3 cells (e.g. cell 1, 2,3) to be measured, the PRS pattern 1 shall be used for cell1 and pattern 2 for both cell2 and cell3.

Therefore, we can propose that:

Proposal 7: The number of positioning frequency layers measured can not be larger than 2.

- Gap configuration

Since the new gap pattern for PRS measurement was introduced in Rel16, both the legacy measurement gap pattern and new gap pattern shall be tested for the core requirements.

Proposal 8: Gap pattern #0 and #24 can be used for NR Positioning tests.

In summary, as an example, several cell specific test parameters for FR1 RSTD measurement can be listed in the table below. The more detailed parameters configuration for these RSTD test case are also given in Appendix.

Table 2-1: Cell-specific test parameters NR RSTD measurement reporting delay

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| NR postioining frequency layer |  | 1 | 2 | 2 |
| PRS configuration Index |  | 0 | 12 | 171 |
|  | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -5.5 | -11.5 | -11.5 |
| Es/Iot | dB | -6.0 | -12.8 | -12.8 |
| Propagation Condition |  | TDL-A (30 ns delay spread, 5Hz) |  |  |
| GAP |  | #0/#24 |  |  |
| PRS muting |  | No muting |  |  |
| Power boosting |  | No power boosting |  |  |
| time shifts between the reference cell and neighbor cell | us | - | 0 | 3 |
| ExpectedRSTD-Uncertainty | us | 500 | 500 | 500 |

# Test Cases List and Time plan

The aim of these test cases is to verify the requirements of NR Positioning measurements defined in sections 9.9 and 10.1.23~25 which are related to the measurement reporting delay and accuracy respectively.

Meanwhile, it shall be noted that for NR E-CID positioning method the requirements defined in Section 9.9.5 [2] are reused from these of SS-RSRP, SS-RSRQ, CSI-RSRP, and CSI-RSRQ [2]. As a result, the test case for NR E-CID measurement can also follow the same ones defined for SS-RSRP, SS-RSRQ, CSI-RSRP, and CSI-RSRQ.

Proposal 9: No need to define the new test cases for NR E-CID measurement requirements in TS38.133.

Furthermore, NR positioning measurements configured by NR PCell (EN-DC) are supported only in Rel-16. That is currently it is enough to define the test cases for NR standalone scenarios.

Proposal 10: It is enough to define the test cases for NR standalone cells.

Therefore, generally the following test cases for NR positioning are necessary:

- Reporting delay test cases for NR standalone cells

  - RSTD

  - UE RX-TX time difference

  - PRS RSRP

- Measurement accuracy tests for NR standalone cells

  - RSTD

  - UE RX-TX time difference

  - PRS RSRP

It shall also be noted that the testing principle explained in Section 2 can be applied for the test case design for RSTD reporting delay.

Nevertheless, the testing procedure can reuse that of LTE OTDOA[4, TS36.133]. That is the test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2.

Proposal 11: The following test cases for core requirement (e.g. reporting delay tests) and accuracy requirements are listed in Table 3.1 and Table 3.2 respectively.

Table 3.1 Test cases for NR positioning core requirements

| No | Type of Test | Description | Test purpose | Responsible company |
| --- | --- | --- | --- | --- |
| 1-1 | FDD RSTD measurement reporting in FR1 | FDD, PRS pattern 1 Gap#24 Positioning period (T1,T2, T3) = (3,1.28,1.28) ms No DRX cycle Alignment b/w cells = synchronous 3 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us TDL-A (30 ns delay spread, 5Hz) | Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells | Intel |
| 1-2 | TDD RSTD measurement reporting in FR1 | TDD, PRS pattern 2 Gap#0 Positioning period (T1,T2, T3) = (3,1.28,1.28) ms No DRX cycle Alignment b/w cells = synchronous 3 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us TDL-A (30 ns delay spread, 5Hz) | Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells | Intel |
| 1-3 | TDD RSTD measurement reporting in FR2 | TDD, PRS pattern 3 Gap#0 Positioning period (T1,T2, T3) = (3,1.28,1.28) ms No DRX cycle Alignment b/w cells = synchronous 3 cells in total Total measured positioning frequency layer is 4 DL-PRS-expectedRSTD is +/- 500us TDL-A (30 ns delay spread, 5Hz) | Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells | Intel |
| 2-1 | FDD UE Rx-Tx time difference measurement reporting in FR1 | FDD, PRS pattern 1, SRSConf.1 Gap#24 Positioning period (T1,T2, T3) = (3,1.28,1.28) ms No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us TDL-A (30 ns delay spread, 5Hz) | Core requirements in section 9.9.4.4 which is also rely on UE’s processing capability to be verified. UE reports UE Rx-Tx time difference within required delay for certain number of cells |  |
| 2-2 | TDD UE Rx-Tx time difference measurement reporting in FR1 | TDD, PRS pattern 2, SRSConf.1 Gap#0 Positioning period (T1,T2, T3) = (3,1.28,1.28) ms No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us TDL-A (30 ns delay spread, 5Hz) | Core requirements in section 9.9.4.4 which is also rely on UE’s processing capability to be verified. UE reports UE Rx-Tx time difference within required delay for certain number of cells |  |
| 2-3 | TDD UE Rx-Tx time difference measurement reporting in FR2 | TDD, PRS pattern 3, SRSConf.1 Gap#0 Positioning period (T1,T2, T3) = (3,1.28,1.28) ms No DRX cycle Alignment b/w cells = synchronous 3 cells in total Total measured positioning frequency layer is 4 DL-PRS-expectedRSTD is +/- 500us TDL-A (30 ns delay spread, 5Hz) | Core requirements in section 9.9.4.4 which is also rely on UE’s processing capability to be verified. UE reports UE Rx-Tx time difference within required delay for certain number of cells |  |
| 3-1 | FDD PRS RSRP measurement reporting in FR1 | FDD, PRS pattern 1 Gap#24 Positioning period (T1,T2, T3) = (3,1.28,1.28) ms No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us TDL-A (30 ns delay spread, 5Hz) | Core requirements in section 9.9.3.4 which is also rely on UE’s processing capability to be verified. UE reports PRS RSRP within required delay for certain number of cells |  |
| 3-2 | TDD PRS RSRP measurement reporting in FR1 | TDD, PRS pattern 2 Gap#0 Positioning period (T1,T2, T3) = (3,1.28,1.28) ms No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us TDL-A (30 ns delay spread, 5Hz) | Core requirements in section 9.9.3.4 which is also rely on UE’s processing capability to be verified. UE reports PRS RSRP within required delay for certain number of cells |  |
| 3-3 | TDD PRS RSRP measurement reporting in FR2 | TDD, PRS pattern 3 Gap#0 Positioning period (T1,T2, T3) = (3,1.28,1.28) ms No DRX cycle Alignment b/w cells = synchronous 3 cells in total Total measured positioning frequency layer is 4 DL-PRS-expectedRSTD is +/- 500us TDL-A (30 ns delay spread, 5Hz) | Core requirements in section 9.9.3.4 which is also rely on UE’s processing capability to be verified. UE reports PRS RSRP within required delay for certain number of cells |  |

Table 3.2 Test cases for NR positioning accuracy requirements

| No | Type of Test | Description | Test purpose | Responsible company |
| --- | --- | --- | --- | --- |
| 4-1 | FDD RSTD measurement accuracy in FR1 | FDD, PRS pattern 1 Gap#24 Measurement positioning period : TRSTD,Total No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 1 DL-PRS-expectedRSTD is +/- 500us AWGN | Accuracy requirements in section 10.1.23.2 is to be verified. UE reports RSTD within required measurement accuracy for certain number of cells | Intel |
| 4-2 | TDD RSTD measurement accuracy in FR1 | TDD, PRS pattern 2 Gap#0 Measurement positioning period : TRSTD,Total No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 1 DL-PRS-expectedRSTD is +/- 500us AWGN | Accuracy requirements in section 10.1.23.2 is to be verified. UE reports RSTD within required measurement accuracy for certain number of cells | Intel |
| 4-3 | TDD RSTD measurement accuracy in FR2 | TDD, PRS pattern 3 Gap#0 Measurement positioning period : TRSTD,Total No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 1 DL-PRS-expectedRSTD is +/- 500us AWGN | Accuracy requirements in section 10.1.23.2 is to be verified. UE reports RSTD within required measurement accuracy for certain number of cells | Intel |
| 5-1 | FDD UE Rx-Tx time difference measurement accuracy in FR1 | FDD, PRS pattern 1, SRSConf.1 Gap#24 Measurement positioning period : TUERxTx, Total No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us AWGN | Accuracy requirements in section 10.1.25.2 is to be verified. UE reports UE Rx-Tx time difference within required measurement accuracy for certain number of cells |  |
| 5-2 | TDD UE Rx-Tx time difference measurement accuracy in FR1 | TDD, PRS pattern 2, SRSConf.1 Gap#0 Measurement positioning period : TUERxTx, Total No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us AWGN | Accuracy requirements in section 10.1.25.2 is to be verified. UE reports UE Rx-Tx time difference within required measurement accuracy for certain number of cells |  |
| 5-3 | TDD UE Rx-Tx time difference measurement accuracy in FR2 | TDD, PRS pattern 3, SRSConf.1 Gap#0 Measurement positioning period : TUERxTx, Total No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us AWGN | Accuracy requirements in section 10.1.25.2 is to be verified. UE reports UE Rx-Tx time difference within required measurement accuracy for certain number of cells |  |
| 6-1 | FDD PRS RSRP measurement reporting in FR1 | FDD, PRS pattern 1 Gap#24 Measurement period TPRS-RSRP, total No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us AWGN | Accuracy requirements in section 10.1.24.2 is to be verified. UE reports PRS RSRP within required measurement accuracy for certain number of cells |  |
| 6-2 | TDD PRS RSRP measurement reporting in FR1 | TDD, PRS pattern 2 Gap#0 Measurement period TPRS-RSRP, total No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us AWGN | Accuracy requirements in section 10.1.24.2 is to be verified. UE reports PRS RSRP within required measurement accuracy for certain number of cells |  |
| 6-3 | TDD PRS RSRP measurement reporting in FR2 | TDD, PRS pattern 3 Gap#0 Measurement period TPRS-RSRP, total No DRX cycle Alignment b/w cells = synchronous 2 cells in total Total measured positioning frequency layer is 2 DL-PRS-expectedRSTD is +/- 500us AWGN | Accuracy requirements in section 10.1.24.2 is to be verified. UE reports PRS RSRP within required measurement accuracy for certain number of cells |  |
| Notes: - TRSTD,Total can be derived according to the RSTD measurement requirements specified in Clause 9.9.2.5. depending on UE capability - TPRS-RSRP, total can be derived according to the RSTD measurement requirements specified in Clause 9.9.3.5. depending on UE capability - TUERxTx, Total can be derived according to the RSTD measurement requirements specified in Clause 9.9.4.5. depending on UE capability |  |  |  |  |

# Summary

This paper proposes the main PRS configurations parameters and test cases list to verify the RRM requirements related to NR positioning.

Proposal 1: The PRS configuration patterns in Table 1 for NR Positioning measurement tests can be specified in the Annex A.3.x of TS 38.133.

Proposal 2: SRS configuration pattern 1 for timing accuracy test in [2] can be reused for NR positioning measurements.

Proposal 3: NO DRX case will be tested only for NR positioning measurement requirements in Rel16.

Proposal 4-1:  3 cells deployment scenarios (one is serving/reference cell , the other two neighbor cells) can be used for NR RSTD .

Proposal 4-2:  2 cells deployment scenarios (one is serving/reference cell , the other neighbor cell) can be used for UE Rx-Tx time difference, PRS RSRP and E-CID measurement tests.

Proposal 5: The synchronous cells will be tested for the measurement delay requirements test.

Proposal 6: For the core requirements test cases, only the non-muting PRS configuration will be used.

Proposal 7: The number of positioning frequency layers measured can not be larger than 2.

Proposal 8: Gap pattern #0 and #24 can be used for NR Positioning tests.

Proposal 9: No need to define the new test cases for NR E-CID measurement requirements in TS38.133.

Proposal 10: It is enough to define the test cases for NR standalone cells.

Proposal 11: The following test cases for core requirement (e.g. reporting delay tests) and accuracy requirements are listed in Table 3.1 and Table 3.2 respectively.

# References

- R4-2012283, WF on requirements for RSTD and UE Rx-Tx time difference measurement

- 3GPP TS 38.133 v16.5.0

- R4-2014579, Link-level simulation results for PRS RSRP measurement

- 3GPP TS 38.133 v16.5.0

# Appendix: [Draft] CR for RSTD core requirement test cases in FR1

## A.6.6.x	RSTD Measurements

### A. 6.6.x.1	NR RSTD measurement reporting delay test case

#### A. 6.6.x.1.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 9.9.2 in an environment with fading propagation conditions.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel but not the same positioning frequency layers.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 37.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T is the maximum processing time of the OTDOA assistance data which is up to UE processing capability and measurement gap periodicity.

Supported test configurations are shown in table A.6.6.x.1.1-1. The test parameters are as given in Table 6.6.x.1.1-2, Table A. 6.6.x.1.1-3 and Table A6.6.x.1.1-4.

Table A.6.6.x.1.1-1: Supported test configurations for NR RSTD

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 100 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A6.6.x.1.1-2: General test parameters for RSTD measurement reporting delay under fading propagation conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
| PRS Configuration | Config 1 |  | PRS patten 1 | As specified in clause A.3.xx |
|  | Config 2 |  | PRS patten 2 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 500 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | No | Correponds to prs-MutingInfo defined in TS 37.355 [24] |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |
| T3 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T2 |

Table A6.6.x.1.1-3: Cell-specific test parameters for RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.5 FDD | N/A | N/A |
| EPRE ratio of PSS to SSS |  | dB | 0 | N/A | N/A |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -70.05 | -70.05 | -70.05 |
|  | Config 2 | dBm/ 96.48MHz | -59.9 | -59.9 | -59.9 |
| SSB RP Note4 | Config 1 | dBm/SCS | -Infinity | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -Infinity | -Infinity | -Infinity |
|  |  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  |  | TDL-A (30 ns delay spread, 5Hz) |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.8.12.1.1-4: Cell-specific test parameters for RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG patterns defined in A.3.2.1 |  |  | OP.5 TDD |  | OP.6 TDD |  | OP.6 TDD | N/A |
| EPRE ratio of PSS to SSS |  | dB | 0 |  | 0 |  | 0 | N/A |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | N/A | N/A | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -95 | -95 | -95 | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.5 | -Infinity | -Infinity | -11.5 | -11.5 | -Infinity |
|  | Config 2 | dB | -5.5 | -Infinity | -Infinity | -11.5 | -11.5 | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -69.08 | -70.05 | -70.05 | -69.83 | -69.83 | -70.05 |
|  | Config 2 | dBm/ 96.48MHz | -58.95 | -59.9 | -59.9 | -59.71 | -59.71 | -59.9 |
| PRS Note 4 |  | dB | -6 | -Infinity | -Infinity | -13 | -13 | -Infinity |
| Propagation Condition |  |  | TDL-A (30 ns delay spread, 5Hz) |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |  |  |  |

#### 6.6.x.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 9.9.2.5.1

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 2560 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i , where

TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast ,

End of Change