3GPP TSG-RAN WG4 Meeting # 97-e 	                                              R4-2014573

Electronic Meeting, 2-13 Nov., 2020

Agenda Item:	7.7.2.1.

Source:	Intel Corporation

Title:	Further Discussion on NR PRS RSTD Measurement Report Requirements

Document for:		Discussion

# Introduction

In the last RAN4 meeting, the core part of UE PRS RSTD requirements was completed, and some remaining issues were also summarized in [1]. In this contribution we will provide further considerations on open issues of the core requirements below.

- Measurement period requirements

# Measurement Reporting Period

According to the previous RAN4’s discussion [1], the following factors composing the baseline formulation of PRS RSTD measurement requirement shall be further considered in detail.

- Calculation of PRS sample duration

There are several alternatives for this issue were captured in WF [1].

Candidate options: - Option 1: Based on the type (type 1 or type 2) as UE used to report {N,T} - Option 2: Based on Type 2. RSTD measurement period for Type 1 PRS duration calculation shall be no longer than Type 2. - Other options are not precluded

Based on RAN1’s agreements below, the two types of DL PRS symbols duration were defined for UE PRS processing capability {N, T} and {N’ }. That is when the corresponding measurement reporting delay requirements which depend on UE processing capability {N, T} was applied to UE, the same type of PRS symbol duration shall be assumed.

Agreement: - For the purpose of DL PRS processing capability, the duration of DL PRS symbols (K) in ms within any P msec window, is calculated by - Type 1 duration calculation with Ks=Tsend-Tsstart - Type 2 duration calculation with K=12μSwhere - Type 1 or Type 2 is reported as UE capability, - S is the set of slots of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets (target and reference), - for Type 1, [Tsstart, Tsend]Tsstart, Tsend is the smallest interval in ms within slot s corresponding to an integer number of OFDM symbols of a serving cell that covers the union of the potential PRS symbols and determines the PRS symbol occupancy within slot s. Interval[Tsstart, Tsend] considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets (target and reference).

Observation 1: The type of PRS symbol duration for UE measurement delay was same as what UE indicate its PRS processing capability with {N, T}.

As a result, when UE performing RSTD measurement the PRS sample duration can use the same type of PRS symbol duration as UE processing capability {N,T}.

Proposal 1: For RSTD measurement delay, the PRS sample duration shall be based on the same type (either type 1 or type 2) as UE used to report {N,T}.

- Multiple PRS periodicities

The options for the multiple PRS periodicities can be:

- Option 1: Use the maximum PRS resource periodicity among all PRS resource in a same positioning frequency layer - Option 2: Use the maximum PRS resource periodicity among all PRS resource among all configured PRS frequency layers.

RAN1/2 has no any explicit restrictions on the different PRS periodicities for the PRS resources within a same PRS measurement layer. On the other hand, for the different frequency positioning frequency layer, the measurement delay will be calculated individually as below[2]. Thus, the maximum PRS resource periodicity can be assumed when RAN4 defining the PRS measurement period.

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Proposal 2: Use the maximum PRS resource periodicity among all PRS resource in a same positioning frequency layer.

- PRS occasion dropping due to SSB

In the last meeting, it was agreed that

- FFS Whether PRS occasion dropping due to SSB collision should be accounted in measurement period - Option 1. Yes - Option 2. No, measurement period will be defined for cases when PRS occasions are not dropped for any reason

And according to the agreements in the last RAN plenary meeting[3], the open issue of concurrent PRS and RRM measurement will be down-scoped. Hence, the requirement in Rel16 can be applicable for the case without concurrent PRS and RRM measurement.

Proposal 3: The requirement for RSTD measurement reporting in Rel16 need not account the PRS occasion dropping due to PRS and RRM measurement happened simultaneously.

- Measurement period when configured with PRS-RSRP

- Option 1: If RSTD is configured together with PRS-RSRP, then the measurement periods for both measurements are defined as: max(TRSTD,TPRS-RSRP), where TRSTD and TPRS-RSRP are the measurement periods for PRS-RSRP and RSTD, when configured without other measurements. - Option 2: RSTD measurement period is not impacted by PRS-RSRP measurement

One of most important arguments for Option 1 is the concurrent RSTD and PRS RSRP within a same positioning session will contribute equally to UE’s location estimation. However, in RAN1 and RAN2 [4] the mandatory measurement in OTDoA is RSTD while PRS-RSRP mainly serves as optional quality indicator. In other words, For DL TDoA positioning method, RSTD is more critical than PRS RSRP measurement. As a result, we can not restrict RSTD measurement with PRS RSRP.

Proposal 4: RSTD measurement period shall not be impacted by PRS-RSRP measurement.

# Conclusion

In this contribution, some considerations on NR PRS RSTD measurement requirements are provided and the following observations and proposals can be drawn:

Observation 1: The type of PRS symbol duration for UE measurement delay was same as what UE indicate its PRS processing capability with {N, T}.

Proposal 1: For RSTD measurement delay, the PRS sample duration shall be based on the same type (either type 1 or type 2) as UE used to report {N,T}.

Proposal 2: Use the maximum PRS resource periodicity among all PRS resource in a same positioning frequency layer.

Proposal 3: The requirement for RSTD measurement reporting in Rel16 need not account the PRS occasion dropping due to PRS and RRM measurement happened simultaneously.

Proposal 4: RSTD measurement period shall not be impacted by PRS-RSRP measurement.

# References

- R4-2012283, “WF on requirements for RSTD and UE Rx-Tx time difference measurement”

- 3GPP TS 38.133 v16.5.0: "NR: Requirements for support of radio resource management "

- RP-201343

- TS 37.255 v16.1.0