3GPP TSG-RAN WG4 Meeting # 97-e 	                                     R4-2014579

Electronic Meeting, 2-13 Nov., 2020

Agenda Item:	7.7.3.2.1.2

Source:	Intel Corporation

Title:	Link-level simulation results for PRS RSRP measurement

Document for:		Discussion

# Introduction

In the previous RAN4 meeting the link-level simulation assumption for PRS RSTD measurement in NR positioning was agreed [1]. In this contribution, we provided a set of initial link-level simulation results and observations for PRS RSRP measurements.

# Simulation Results

The general parameters and PRS-specific parameter agreed in the last RAN4 meeting are provided in Table 1 and Table 2, respectively in Appendix.

It was noted that in the last RAN4 meeting, the performance of metric in terms of PRS RSRP error was also defined as:

- PRS-RSRP error = estimated PRS-RSRP – ideal PRS-RSRP (based on perfect channel knowledge).

In the figures below the results for the difference PRS measurement configuration (e.g. PRS comb size and measurement bandwidth) were given.

Figure 1. RSRP simulation results with different PRS Comb size/Numsymbol per slot

Figure 2. RSRP simulation results with different PRS BW

Observation 1: There is not obvious performance gap when PRS repetition factor is different.

Observation 2: There is not obvious performance gap when increasing PRS BW when SINR is up to -6dB. Observation 3: There is about >2dB performance difference when SINR is up to -13dB.

Observation 4: PRS RSRP accuracy requirements when SINR >-13dB can be less than ±6dB

Thus in RAN4 it is possible to cover all measurement PRS configuration (e.g. PRS bandwidth and PRS resource symbols ) with a single requirement. It is better to define single performance requirements for the different PRS resource configuration which can reduce the testing efforts greatly.

Based on the above observations and analysis, the proposal can be drawn as:

Proposal 1: It is feasible to define a single PRS RSRP measurement accuracy.

# Conclusion

In this contribution, link level simulation results for PRS RSRP are provided.

Observation 1: There is not obvious performance gap when PRS repetition factor is different.

Observation 2: There is not obvious performance gap when increasing PRS BW when SINR is up to -6dB. Observation 3: There is about >2dB performance difference when SINR is up to -13dB.

Observation 4: PRS RSRP accuracy requirements when SINR >-13dB can be less than ±6dB

Proposal 1: It is feasible to define a single PRS RSRP measurement accuracy.

# References

- R4-2002287, Updated link level simulation assumption for RSTD and RSRP, Huawei, HiSilicon

# Appendix

Table 1: General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell 3>, where cell 1 is the reference cell |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 10 MHz, 20 MHz, 50 MHz - 15 kHz - FDD, TDD - 4 GHz - 20 MHz, 50 MHz, 100 MHz - 30 kHz - FDD, TDD | - 40 GHz - 50 MHz, 100 MHz, 200 MHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, TDL-C (300 ns delay spread, 100 Hz), TDL-A (30 ns delay spread, 5Hz), TDL-B (100 ns delay spread, 200Hz) | TDL-C (60 ns delay spread, 300 Hz) |
| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | (-6, -13, -13) (-3, -13, -13) | (-6, -13, -13) (-3, -10, -10) |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| UE measurement bandwidth | Full carrier bandwidth |  |

Table 2: PRS transmission configuration parameters

| Parameter | Value |  |
| --- | --- | --- |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: - 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: - 48 (20MHz),132 (50MHz), 272 (100MHz) | - 120 kHz: - 32(50MHz),64(100MHz), 128 (200MHz) |
| Comb | Comb-4, comb-6, comb-2 (with muting) |  |
| DL-PRS-ResourceRepetitionFactor | 1, 2, 4, 16 |  |
| PRS muting | No muting (comb-4, comb-6), muting (comb-2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-Uncertainty | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |