3GPP TSG-RAN WG4 Meeting # 97-e	R4-2014593

# Electronic Meeting, 2-13 Nov., 2020

Source: 	Skyworks Solutions, Inc.

Title: 	n40 MPR and Interference for Additional Channel Bandwidths

Agenda Item:	10.21.2.2 MPR/A-MPR/NS signaling [NR_bands_R17_BWs -Core]

Document for:	Approval

# Introduction

In the updated WI [1] agreed in RAN#89 e-meeting, the addition of 90 and 100MHz channel bandwidths are requested for Band n40. In this contribution, we discuss the related fractional BW criteria issue, MPR and potential interference to ISM band.

# Discussion

For TDD bands, the general MPR applies up to a fractional BW of 4%, above that MPR may apply.

In Table 1 we calculate the fractional bandwidth for the newly requested channel bandwidths for n40 and compare with n41 100MHz case.

Table 1: Fractional bandwidth for n40 and n41

| Band | FUL_low | FUL_High | Channel BW | Fractional BW |
| --- | --- | --- | --- | --- |
| n40 | 2300 | 2400 | 90 | 3.8% |
| n40 | 2300 | 2400 | 100 | 4.3% |
| n41 | 2496 | 2690 | 100 | 3.9% |

Observations:

- For 90 MHz in n40 the fractional BW criteria is met in a similar way than for 100 MHz in n41

- For 100 MHz in n40 the fractional BW criteria is slightly exceeded

- However,

  - On one end, 100MHz for n40 is the full band and thus MPR criteria are largely helped by the filter and thus MPR is probably not needed

  - On the other end, the abutting ISM band does not favor the use of all UL resources to avoid interference to Bluetooth or WiFi systems in-device or at the network level. Note that this is also true for 90 MHz channel bandwidth if placed at the top of the band.

Proposal: The need for 100MHz UL BW in n40 is further evaluated in relation to potential interference to the whole 2.4 GHz ISM band. If still adopted, MPR may be ignored.

# Conclusions

In this contribution, we briefly discuss the potential issues of 100 MHZ UL channel BW in Band n40 and make the following proposal.

Proposal: The need for 100MHz UL BW in n40 is further evaluated in relation to potential interference to the whole 2.4 GHz ISM band. If still adopted, MPR may be ignored.

# References

[1]	RP-201546 Revised WID: Core part: Rel-17 Adding channel bandwidth support to existing NR bands, Ericsson, RAN#89e