3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014603

# Electronic Meeting, 2-13 Nov., 2020

Agenda Item: 7.9.3.2

Source: MediaTek Inc.

Title: Discussion on L1-SINR measurement accuracy requirement

Document for: Discussion

- Introduction

In last meeting, the discussion of RRM core part for eMIMO has been closed and the RRM performance part will be discussed in this meeting. Thus, our view on accuracy requirement for L1-SINR is provided in this paper.

- Discussion

In the RAN4 #93 meeting, L1-SINR simulation assumption is provided and companies can discuss the L1-SINR accuracy requirement based on the simulation assumption. To be clear, our view on the L1-SINR accuracy requirement is summarized in Section 4 (Appendix) in this paper.

2.1 CMR only scenario

For CMR only scenario, in order to calculate L1-SINR measurement, the signal power and noise power are estimated based on the same resources. Thus, the L1-SINR measurement will become less accurate, compared to the cases with IMR configured, when either signal power or noise power is too low.

Observation 1: For CMR only scenario, the L1-SINR measurement will become less inaccurate if either signal power is low (i.e. high SNR) or noise power is low (i.e. low SNR).

Based on Observation 1, RAN4 need to consider the worst case for the L1-SINR accuracy requirement among low and high SNR side condition. Thus, according to RAN4 agreement in RAN4 #92bis and #93 meeting shown as follows, RAN4 need to consider the L1-SINR accuracy requirement with side condition on Es/Iot = 25 dB and Es/Iot = -3 dB for CMR only scenario.

RAN4 # 92 bis meeting

Side condition for L1-SINR measurement - CMR only scenarios: reuse side condition for L1-RSRP measurement

RAN4 #93 meeting

Agreement Side condition for CMR only scenario In addition to lower bound, the accuracy requirements for L1-SINR in CMR only scenarios is applicable for CSI-RS Es/Iot <= 25 dB as upper bound (Same note 3 applied as L1-RSRP: “NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB”).

Proposal 1: For CMR only scenario, RAN4 need to evaluate L1-SINR accuracy requirement with side condition on Es/Iot = 25 dB, in addition to Es/Iot = -3 dB.

The simulation assumption is provided as below for reference:

- L1-SINR accuracy evaluation simulation assumption for CMR only: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; N/A. Side condition (SNR) on CMR; -3dB. Density (D); 3. Number of samples (M); 1, 3, 5. Number of PRBs; 48.

The corresponding simulation result is provided in Table 1 (at SNR of -3dB) and Table 2 (at SNR of -25dB)

Table 1. CSI-RS based CMR L1-SINR absolute accuracy (dB) @ SNR=-3dB, 1, 3, 5 sample(s)

| SCS | # of samples | 5%-ile | 50%-ile | 95%-ile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.01 | 0.70 | 2.24 |
|  | 3 | -0.19 | 0.75 | 1.67 |
|  | 5 | 0.04 | 0.74 | 1.53 |
| 30 kHz | 1 | -1.15 | 0.60 | 2.22 |
|  | 3 | -0.30 | 0.67 | 1.65 |
|  | 5 | -0.05 | 0.66 | 1.46 |
| 120 kHz | 1 | -0.89 | 0.72 | 2.27 |
|  | 3 | -0.13 | 0.79 | 1.71 |
|  | 5 | 0.09 | 0.80 | 1.50 |

Table 2. CSI-RS based CMR L1-SINR absolute accuracy (dB) @ SNR=25dB, 1, 3, 5 sample(s)

| SCS | # of samples | 5%-ile | 50%-ile | 95%-ile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.35 | -0.25 | 1.11 |
|  | 3 | -0.82 | -0.20 | 0.59 |
|  | 5 | -0.69 | -0.18 | 0.38 |
| 30 kHz | 1 | -1.23 | -0.27 | 1.03 |
|  | 3 | -0.83 | -0.18 | 0.59 |
|  | 5 | -0.64 | -0.17 | 0.38 |
| 120 kHz | 1 | -0.94 | 0.20 | 1.49 |
|  | 3 | -0.43 | 0.25 | 1.16 |
|  | 5 | -0.31 | 0.28 | 0.97 |

From Table 1 and Table 2, for 1 sample SSB measurement at SNR of -3dB, the measurement accuracy about 3 dB can be achieved. Considering implementation margin, accuracy of 4.5 dB is proposed in FR1. In FR2, no extra margin is assumed, by following the same logic in R15 SS-SINR accuracy measurement.

Proposal 2: For CMR only scenario L1- SINR for reporting, the absolute measurement accuracy is +/- 4.5 dB for FR1; +/- 4.5 dB for FR2 with side condition on CMR=-3dB.

2.2 SSB based CMR + NZP-IMR scenario

For SSB based CMR + NZP-IMR scenario, the simulation assumption is provided as below for reference:

- L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + NZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); SSB. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; option-1: 0dB, other options not excluded. Side condition (SNR) on IMR; option-1: 0dB, other options not excluded. Density (D) for IMR; 3. Number of samples (M) for IMR/CMR; 1, 3, 5. Number of PRBs for IMR; 48. Propagation condition; AWGN.

The corresponding simulation result is provided in Table 3.

Table 3. SSB based CMR + NZP IMR L1-SINR absolute accuracy (dB), CMR and IMR at SNR of 0dB with 1, 3, 5 sample(s)

| SCS | # of samples | 5%-ile | 50%-ile | 95%-ile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.15 | 0.03 | 1.15 |
|  | 3 | -0.64 | 0.03 | 0.66 |
|  | 5 | -0.49 | 0.01 | 0.54 |
| 30 kHz | 1 | -1.11 | 0.03 | 1.19 |
|  | 3 | -0.63 | 0.03 | 0.65 |
|  | 5 | -0.47 | 0.05 | 0.53 |
| 120 kHz | 1 | -1.15 | 0.01 | 1.13 |
|  | 3 | -0.63 | 0.02 | 0.63 |
|  | 5 | -0.48 | 0.00 | 0.52 |

From The corresponding simulation result is provided in Table 3.

Table 3, for 1 sample SSB based CMR + NZP IMR measurement at SNR of -3 dB, the measurement accuracy about 2 dB can be achieved. Considering implementation margin, accuracy of 4 dB is proposed in FR1. In FR2, no extra margin is assumed, by following the same logic in R15 SS-SINR accuracy measurement.

Proposal 3: For SSB based CMR + NZP IMR L1-SINR for reporting, the absolute measurement accuracy is +/- 4 dB for FR1; +/- 4 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

2.3 SSB based CMR + ZP-IMR scenario

For SSB based CMR + ZP-IMR scenario, the simulation assumption is provided as below for reference:

- L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + ZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); SSB. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; -3dB. Side condition (SNR) on IMR; N/A (only AWGN noise). Density (D) for IMR; 3. Number of samples (M) for IMR/CMR; 1, 3, 5. Number of PRBs for IMR; 48. Propagation condition; AWGN.

The corresponding simulation result is provided in Table 4.

Table 4. SSB based CMR and ZP-IMR L1-SINR absolute accuracy (dB) @ SNR=-3 dB, 1, 3, 5 sample(s)

| SCS | # of samples | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -1.00 | 0.40 | 1.69 |
|  | 3 | -0.40 | 0.38 | 1.14 |
|  | 5 | -0.22 | 0.41 | 0.99 |
| 30 kHz | 1 | -0.97 | 0.39 | 1.69 |
|  | 3 | -0.40 | 0.42 | 1.18 |
|  | 5 | -0.14 | 0.41 | 0.98 |
| 120 kHz | 1 | -1.02 | 0.40 | 1.65 |
|  | 3 | -0.43 | 0.39 | 1.13 |
|  | 5 | -0.22 | 0.39 | 0.99 |

From Table 4, for 1 sample SSB based CMR + ZP-IMR measurement at SNR of -3dB, the measurement accuracy about 2 dB can be achieved. Considering implementation margin, accuracy of 4dB is proposed in FR1. In FR2, no extra margin is assumed, by following the same logic in R15 SS-SINR accuracy measurement.

Proposal 4: For SSB based CMR + ZP-IMR L1-SINR for reporting, the absolute measurement accuracy is +/- 4 dB for FR1; +/- 4 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

2.4 CSI-RS-based CMR + NZP-IMR scenario

For CSI-RS based CMR + NZP-IMR scenario, the simulation assumption is provided as below for reference:

- L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + NZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; option-1: 0dB, other options not excluded. Side condition (SNR) on IMR; option-1: 0dB, other options not excluded. Density (D) for CMR/IMR; 3. Number of samples (M) for CMR/IMR; 1, 3, 5. Number of PRBs for CMR/IMR; 48. Propagation condition; AWGN.

The corresponding simulation result is provided in Table 5.

Table 5. CSI-RS based CMR + NZP-IMR L1-SINR absolute accuracy (dB) @ SNR=0 dB, 1, 3, 5 sample(s)

| SCS | # of samples | 5%-ile | 50%-ile | 95%-ile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -0.77 | 0.21 | 1.03 |
|  | 3 | -0.33 | 0.23 | 0.75 |
|  | 5 | -0.19 | 0.22 | 0.66 |
| 30 kHz | 1 | -0.71 | 0.15 | 1.04 |
|  | 3 | -0.35 | 0.16 | 0.71 |
|  | 5 | -0.21 | 0.17 | 0.61 |
| 120 kHz | 1 | -0.65 | 0.23 | 1.06 |
|  | 3 | -0.24 | 0.25 | 0.74 |
|  | 5 | -0.14 | 0.25 | 0.61 |

From Table 5, for 1 sample CSI-RS based CMR + NZP IMR measurement at SNR of -3 dB, the measurement accuracy about 2 dB can be achieved. Considering implementation margin, accuracy of 4 dB is proposed in FR1. In FR2, no extra margin is assumed, by following the same logic in R15 SS-SINR accuracy measurement.

Proposal 5: For CSI-RS based CMR + NZP IMR L1- SINR for reporting, the absolute measurement accuracy is +/- 4 dB for FR1; +/- 4 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

2.5 CSI-RS-based CMR + ZP-IMR scenario

For CSI-RS based CMR + ZP-IMR scenario, the simulation assumption is provided as below for reference:

- L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + ZP-IMR: Parameters; Values. SCS; 15kHz (FR1), 30kHz (FR1), 120kHz (FR2). Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; CSI-RS. Periodicity; CMR Periodicity = IMR Periodicity. Ideal SINR; -3dB. Side condition (SNR) on CMR; -3dB. Side condition (SNR) on IMR; N/A (only AWGN noise). Density (D) for CMR/IMR; 3. Number of samples (M) for CMR/IMR; 1, 3, 5. Number of PRBs for CMR/IMR; 48. Propagation condition; AWGN.

The corresponding simulation result is provided in Table 6.

Table 6. CSI-RS based CMR + ZP-IMR L1-SINR absolute accuracy (dB) @ SNR=-3 dB, 1, 3, 5 sample(s)

| SCS | # of samples | 5%-ile | 50%-ile | 95%-ile |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | -0.94 | 0.51 | 1.63 |
|  | 3 | -0.24 | 0.51 | 1.15 |
|  | 5 | -0.04 | 0.52 | 1.00 |
| 30 kHz | 1 | -1.00 | 0.49 | 1.66 |
|  | 3 | -0.30 | 0.49 | 1.11 |
|  | 5 | -0.12 | 0.50 | 0.99 |
| 120 kHz | 1 | -0.88 | 0.41 | 1.62 |
|  | 3 | -0.31 | 0.42 | 1.16 |
|  | 5 | -0.14 | 0.42 | 0.96 |

From

The corresponding simulation result is provided in Table 6.

Table 6, for 1 sample CSI-RS based CMR and ZP-IMR measurement at SNR of -3dB, the measurement accuracy about 2 dB can be achieved. Considering implementation margin, accuracy of 4 dB is proposed in FR1. In FR2, no extra margin is assumed, by following the same logic in R15 SS-SINR accuracy measurement.

Proposal 6: For CSI-RS based CMR + ZP-IMR L1- SINR for reporting, the absolute measurement accuracy is +/- 4 dB for FR1; +/- 4 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

According to the discussion above, our view on L1-SINR measurement accuracy requirement is summarized in Table 7.

Table 7. Proposed L1- SINR accuracy requirement

| scenario | Side condition on CMR (dB) | Side condition on IMR (dB) | Accuracy requirement for FR1 and FR2 (dB) |
| --- | --- | --- | --- |
| CMR only | -3 | N.A. | ± 4.5 |
| SSB-based CMR + NZP-IMR | 0 | 0 | ± 4 |
| SSB-based CMR + ZP-IMR | -3 | -3 |  |
| NZP CSI-RS based CMR + NZP-IMR | 0 | 0 |  |
| NZP CSI-RS based CMR + ZP-IMR | -3 | -3 |  |

- In CMR only case, the absolute L1-SINR measurement accuracy is ± 4.5dB for FR1 and FR2.

- In other cases, i.e., CMR + IMR case, the absolute L1-SINR measurement accuracy is ± 4 dB for FR1 and FR2.

- Summary

In this paper, the discussion of L1-SINR measurement is provided. We have the following proposal:

Observation 1: For CMR only scenario, the L1-SINR measurement will become less inaccurate if either signal power is low (i.e. high SNR) or noise power is low (i.e. low SNR).

Proposal 1: For CMR only scenario, RAN4 need to evaluate L1-SINR accuracy requirement with side condition on Es/Iot = 25 dB, in addition to Es/Iot = -3 dB.

Proposal 2: For CMR only scenario L1- SINR for reporting, the absolute measurement accuracy is +/- 4.5 dB for FR1; +/- 4.5 dB for FR2 with side condition on CMR=-3dB.

Proposal 3: For SSB based CMR + NZP IMR L1-SINR for reporting, the absolute measurement accuracy is +/- 4 dB for FR1; +/- 4 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

Proposal 4: For SSB based CMR + ZP-IMR L1-SINR for reporting, the absolute measurement accuracy is +/- 4 dB for FR1; +/- 4 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

Proposal 5: For CSI-RS based CMR + NZP IMR L1- SINR for reporting, the absolute measurement accuracy is +/- 4 dB for FR1; +/- 4 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

Proposal 6: For CSI-RS based CMR + ZP-IMR L1- SINR for reporting, the absolute measurement accuracy is +/- 4 dB for FR1; +/- 4 dB for FR2 with side condition on CMR=-3dB and IMR=-3dB.

References

[1] 3GPP TS 38.133 - NR; Requirements for support of radio resource management