3GPP TSG-RAN WG4 Meeting # 97-e	R4-2014652

# Electronic Meeting, 2 - 13 November, 2020

Source:	LG Electronics

Title:	Discussion on NR V2X single link test cases

Agenda item:	7.3.7.2

Document for:	Discussion

Introduction

In last meeting, the WF [1] on work scope and general assumption for NR V2X demodulation was approved, and following single link test cases were derived.

- PSSCH demodulation

- PSCCH demodulation

- PSBCH demodulation

- PSFCH demodulation

There are still open some parameters for simulation assumptions for demodulation performance requirements. In this contribution, we provide our views on the parameters for single link test cases.

Discussion

PSSCH demodulation

The number of DMRS and periodicity of PSFCH transmission

In [1], there are two options for the number of DMRS configuration and it is related the periodicity of PSFCH transmission.

- Option 1: Following cases can be used depending on decision on relative velocity

  - Case 1: Max speed: 500km/h DMRS symbol: 4 for slots without PSFCH, 3 for slots with PSFCH.

  - Case 2: Moderate speed: 260km/h DMRS symbol: 3 for slots without PSFCH, 2 for slots with PSFCH

  - Case 3: low speed: 30km/h DMRS symbol: 2 for slots with/without PSFCH

- Option 2: 3 DMRS symbols for all test cases

To verify demodulation using various DMRS configuration, option 1 is reasonable. For the speed for PSSCH demodulation, it is not necessary to test all relative velocity, so case 1 and case 3 can be considered for PSSCH demodulation requirements like LTE V2X demodulation. For case 3, 3 for slots without PSFCH and 2 for slots with PSFCH can be considered.

From this DMRS configuration point of view, every 4 slots PSFCH transmission should be considered.

- Proposal 1: DMRS configuration for PSSCH demodulation should be considered depending on relative velocity as case 1 and case 3 in option 1.

- Proposal 2: PSFCH transmission should be considered every 4 slots.

Modulation order

In [2], there are two options for modulation order; QPSK/64QAM and QPSK and 16QAM. NR V2X supports up to 64QAM as mandatory and 256QAM as optional. Therefore, 64QAM modulation order should be verified, and one lower modulation order can be consider such as QPSK. We prefer QPSK and 64QAM modulation order for PSSCH demodulation requirements. For 256QAM, RAN4 has agreed that 256QAM reception is an option UE capability. General use case for 256QAM transmission and reception is for unicast mode, it is newly introduced in NR V2X to support advanced application. So, if a UE supports this feature, it should be tested. To avoid increasing test cases, applicability rule can be considered – UE with support of receiving 256QAM, 64QAM demodulation test can be skipped if UE fulfills this requirement [3].

- Proposal 3: QPSK and 64QAM modulation order should be considered for PSSCH demodulation requirements

- Proposal 4: 256QAM modulation order should be verified with applicability rule.

PSCCH demodulation

Propagation channel

In [2], two propagation conditions were considered for PSCCH demodulation. RAN4 has agreed three relative velocity such as 30km/h, 260km/h, and 500km/h. The propagation channel with 30km/h and 500km/h can be verified by PSSCH demodulation as Proposal 1. So for PSCCH demodulation requirement, TDLA30-1350 can be used.

- Proposal 5: TDLA30-1350 should be used for PSCCH demodulation requirement.

PSFCH demodulation test

RAN4 has discussed which HARQ feedback mode should be considered for PSFCH requirements. As companies are mentioned in last meeting, the detection algorithm for ACK/NACK and NACK only feedback would be the same. The performance ability for ACK/NACK feedback is more comprehensive than NACK only feedback mode. RAN4 has two PSFCH test cases for single link and multiple link, so we can use two HARQ feedback mode for each test case. But, for PSFCH demodulation test for single link, ACK/NACK feedback mode should be used. For PSFCH period, we prefer to use 4 slots the same as other test case.

- Proposal 6: Only ACK/NACK feedback mode should be considered for PSFCH demodulation (single link) requirement.

Simulation assumptions

Based on above proposals, we provide simulation assumptions for single link demodulation.

PSSCH demodulation

Table 1 Simulation assumptions for PSSCH demodulation performance requirements

| Parameters | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Test |  | SCH_Test1 | SCH_Test2 | SCH_Test3 |
| Synchronization source |  | GNSS | GNSS | GNSS |
| Propagation condition |  | TDLA-30-180 | TDLA-30-2700 | TDLA-30-180 |
| Channel bandwidth | MHz | 20MHz | 20MHz | 20MHz |
| Allocated resource blocks | RB | 15 | 15 | 15 |
| Subcarrier spacing | kHz | 30 | 30 | 30 |
| Timing offset |  | CP/2-12Ts | CP/2-12Ts | CP/2-12Ts |
| Frequency offset | Hz | 600 | 600 | 600 |
| OFDM symbols per slotNote1 |  |  |  |  |
| For slot without PSFCH |  | 12 | 12 | 12 |
| For slot with PSFCH |  | 9 | 9 | 9 |
| The number of DMRS symbols |  |  |  |  |
| For slot without PSFCH |  | 3 | 4 | 3 |
| For slot with PSFCH |  | 2 | 3 | 2 |
| Modulation order |  | 64QAM | QPSK | 256QAM |
| MCS index |  | 17 | 4 | 20 |
| The number of REs for 2nd stage SCI (SCI format 2-A)Note5 |  | 174(βoffset=2.5,α=1) | 174 (βoffset=1.75,α=1) | 186 (βoffset=4,α=1) |
| Transport Block Size |  |  |  |  |
| For slot without PSFCH | Bits | 3752 | 848 | 7680 |
| For slot with PSFCH | Bits | 2408 | 552 | 4736 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 |
| Binary Channel BitsNote2 |  |  |  |  |
| For slot without PSFCH | Bits | 8676 | 2712 | 11472 |
| For slot with PSFCH | Bits | 5436 | 1812 | 7152 |
| periodPSFCHresource |  | 4 | 4 | 4 |
| Antenna configuration |  | 1x2 Low | 1x2 Low | 1x2 Low |
| Performance metricNote3 |  | SNRPSSCH@10%BLER |  |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: 15 RBs and 2 symbols are allocated for PSCCH, and 2nd stage SCI is also allocated. Note 3: The performance metric is used for PSSCH simulation evaluation. Note 4: PSFCH is transmitted on every 4 slot. Note 5: γ value is included. |  |  |  |  |

For information, in the last RAN1 meeting, agreements for QSCI2' and TBS determination have been made.

Agreements: - M^SCI2 = M^PSSCH – M^DMRS – M^PTRS – M^PSCCH (“sc” is omitted in each variable) is replaced with M^SCI2 = M^PSSCH– M^PSCCH - This is applied commonly to TBS determination and actual 2nd SCI mapping. Agreements: - In calculation of N^SCI,2_RE of TBS calculation, gamma is assumed to be zero.

PSCCH demodulation

Table 2 Simulation assumptions for PSCCH demodulation performance requirements

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | CCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLA30-1350 |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 10 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSCCH symbols |  | 2 |
| Modulation order |  | QPSK |
| Payload size (without CRC) | Bits | 28 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 360 |
| PSCCH performance metric (Note 1) |  | SNRPSCCH@1%BLER |
| Note 1: The performance metric is used for PSCCH simulation evaluation. Note 2: OCC index i (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 3: Two propagation conditions are listed as options for alignment purpose, one of the option will be chosen for defining requirement |  |  |

PSBCH demodulation

Table 3 Simulation assumptions for PSBCH demodulation performance requirements

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | BCH_Test1 |
| SLID |  | 0 |
| Synchronization source |  | S-SSB |
| Propagation condition |  | TDLA30-180 |
| Channel bandwidth | MHz | 20 |
| Number SSB per SL period(NperiodS-SSB) |  | 1 |
| Allocated resource blocks |  | 11 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | 0 |
| Frequency offset | Hz | 0 |
| The number of symbols (Note 1) |  | 8 |
| Modulation order |  | QPSK |
| Transport Block Size (without CRC) | Bits | 32 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 1782 |
| PSBCH performance metric (Note 2) |  | SNRPSBCH@1%BLER |
| Note 1: The first symbol is for AGC. Note 2: The performance metric is used for PSBCH simulation assumption. |  |  |

PSFCH demodulation

Table 4 Simulation assumptions for PSFCH demodulation performance requirements

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | FCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLA30-180 |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 1 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSFCH symbols |  | 2 |
| Number of information bits | bit | 1 |
| PSFCH period | Slot | 4 |
| Cyclic shift pair index |  | 1 |
| PSFCH performance metric (Note 1) |  | Pr(ACK miss) < 1% |
| Note 1: The performance metric is used for PSFCH simulation assumption. Pr(NACK to ACK) < 0.1% and Pr(DTX to ACK)<1% should be fulfilled. Note 2: The 1st PSFCH which is used for AGC doesn’t need to be considered in performance evaluation. |  |  |

Conclusion

In this contribution, we provide our views on the parameters and final simulation assumptions for single link test cases to define performance requirements. We propose

- Proposal 1: DMRS configuration for PSSCH demodulation should be considered depending on relative velocity as case 1 and case 3 in option 1.

- Proposal 2: PSFCH transmission should be considered every 4 slots.

- Proposal 3: QPSK and 64QAM modulation order should be considered for PSSCH demodulation requirements

- Proposal 4: 256QAM modulation order should be verified with applicability rule.

- Proposal 5: TDLA30-1350 should be used for PSCCH demodulation requirement.

- Proposal 6: Only ACK/NACK feedback mode should be considered for PSFCH demodulation (single link) requirement.

- Proposal 7: Use simulation assumptions Table 1~4 for single link tests

Reference

[1] R4-2012758, “WF on work scope and general assumptions for NR V2X demodulation performance,” LG Electronics.

[2] R4-2012757, “Simulation assumptions for NR V2X test cases,” MediaTek.

[3] R4-2011381, “On NR V2X Demod Requirement,” Qualcomm Inc.