3GPP TSG-RAN WG4 Meeting # 97-e	R4-2014668

# Electronic Meeting, 2 - 13 November, 2020

Source:	LG Electronics

Title:	Initial simulation results for NR V2X single link test cases

Agenda item:	7.3.7.2

Document for:	Discussion

Introduction

In this contribution, we provide initial simulation results for NR V2X single link test cases based on simulation assumptions [1] and [2].

Simulation results

PSSCH demodulation

Simulation assumptions [1]. (The number of RE for 2nd stage SCI is modified)

| Parameters | Unit | Value |  |
| --- | --- | --- | --- |
| Test |  | SCH_Test1 | SCH_Test2 |
| Synchronization source |  | GNSS | GNSS |
| Timing offset |  | CP/2-12Ts | CP/2-12Ts |
| Frequency offset | Hz | 600 | 600 |
| Propagation condition |  | [TDLA30-2700] | [TDLA30-180] |
| Channel bandwidth | MHz | 20 | 20 |
| Allocated resource blocks |  | [15] | [15] |
| Subcarrier spacing | kHz | 30 | 30 |
| CP-OFDM symbols per slot (Note 1) |  | [9] | [9] |
| DMRS position in time domain (symbol index) |  | [#1,#4,#7] | [#1,#4,#7] |
| Modulation order |  | QPSK | 64QAM |
| MCS index |  | [4] | [17] |
| The number of REs for 2nd stage SCI |  | 174 | [174] |
| Transport Block Size | Bits | 608 | [2536] |
| Transport block CRC | Bits | 24 | 24 |
| Maximum number of HARQ retransmissions |  | 0 | 0 |
| Binary Channel Bits (Note 2) | Bits | 1992 | [5976] |
| Antenna configuration |  | 1x2 Low | 1x2 Low |
| Performance metric(Note 3) |  | SNRPSSCH@[10]%BLER |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: 10 RBs and 2 symbols are allocated for PSCCH, and 2nd stage SCI is also allocated. Note 3: The performance metric is used for PSSCH simulation evaluation. Note 4: PSFCH is transmitted on [every slot]. Note 5: 15RB allocation is listed here, but 10RB is another option for simulation results alignment. |  |  |  |

Figure 1 Simulation result for PSSCH demodulation (PSFCH period: 1)

Simulation assumptions [1]. (The number of RE for 2nd stage SCI is modified)

| Parameters | Unit | Value |  |
| --- | --- | --- | --- |
| Test |  | SCH_Test1 | SCH_Test2 |
| Synchronization source |  | GNSS | GNSS |
| Propagation condition |  | TDLA-30-1400 | TDLA-30-2700 |
| Channel bandwidth | MHz | 20MHz | 20MHz |
| Allocated resource blocks | RB | [15] | [15] |
| Subcarrier spacing | kHz | 30 | 30 |
| Timing offset |  | CP/2-12Ts | CP/2-12Ts |
| Frequency offset | Hz | 600 | 600 |
| CP-OFDM symbols per slot (Note 1) |  | [12] for slots without PSFCH 9 for slots with PSFCH | [12] for slots without PSFCH 9 for slots with PSFCH |
| The number of DMRS symbols |  | [3] for slot without PSFCH;[2] for slot with PSFCH | [4] for slot without PSFCH;[3] for slot with PSFCH |
| Modulation order |  | [4] | [2] |
| MCS index |  | [11](4,378/1024) | [4](2,308/1024) |
| The number of REs for 2nd stage SCI (SCI format 2-A) |  | 186(βoffset=2.25,α=1) | 198 (βoffset=2,α=1) |
| Transport Block Size | Bits | 2280 for slots without PSFCH 1544 for slots with PSFCH | 848 for slots without PSFCH 608 for slots with PSFCH |
| Transport block CRC | bits | 24 | 24 |
| Maximum number of HARQ transmissions |  | 1 | 1 |
| Binary Channel Bits (Note 2) | Bits |  |  |
| periodPSFCHresource |  | 4 | 4 |
| Antenna configuration |  | 1x2 Low | 1x2 Low |
| Performance metric(Note 3) |  | SNRPSSCH@10%BLER |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: 10 RBs and 2 symbols are allocated for PSCCH, and 2nd stage SCI is also allocated. Note 3: The performance metric is used for PSSCH simulation evaluation. Note 4: PSFCH is transmitted on every [4] slot. |  |  |  |

Figure 2 Simulation result for PSSCH demodulation (PSFCH period: 4)

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | SCH_Test3 |
| Synchronization source |  | GNSS |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| Propagation condition |  | [TDLA30-180] |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | [15] |
| Subcarrier spacing | kHz | 30 |
| CP-OFDM symbols per slot (Note 1) |  | [9] |
| DMRS position in time domain (symbol index) |  | [#1,#4,#7] |
| Modulation order |  | 256QAM |
| MCS index |  | [20] |
| The number of REs for 2nd stage SCI |  | 186 |
| Transport Block Size | Bits | 5248 |
| Transport block CRC | Bits | 24 |
| Maximum number of HARQ retransmissions |  | 0 |
| Binary Channel Bits (Note 2) | Bits | 7872 |
| Antenna configuration |  | 1x2 Low |
| Performance metric(Note 3) |  | SNRPSSCH@[10]%BLER |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: 10 RBs and 2 symbols are allocated for PSCCH, and 2nd stage SCI is also allocated. Note 3: The performance metric is used for PSSCH simulation evaluation. Note 4: 15RB allocation is listed here, but 10RB is another option for analysis of 256QAM demodulation. |  |  |

Figure 3 Figure 2 Simulation result for PSSCH demodulation (256QAM, PSFCH period: 1)

PSCCH demodulation

Simulation assumptions [1].

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | CCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | [TDLA30-180, TDLA30-1350] |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 10 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSCCH symbols |  | 2 |
| Modulation order |  | QPSK |
| Payload size (without CRC) | Bits | [28] |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 360 |
| PSCCH performance metric (Note 1) |  | SNRPSCCH@1%BLER |
| Note 1: The performance metric is used for PSCCH simulation evaluation. Note 2: OCC index i (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 3: Two propagation conditions are listed as options for alignment purpose, one of the option will be chosen for defining requirement |  |  |

Figure 4 Simulation results for PSCCH demodulation

PSBCH demodulation test

Simulation assumptions [1].

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | BCH_Test1 |
| SLID |  | 0 |
| Synchronization source |  | S-SSB |
| Propagation condition |  | [TDLA30-180] |
| Channel bandwidth | MHz | 20 |
| Number SSB per SL period(NperiodS-SSB) |  | Option 1: 1 Option 2: 2 S-SSB with different contents |
| Allocated resource blocks |  | 11 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | 0 |
| Frequency offset | Hz | 0 |
| The number of symbols (Note 1) |  | 8 |
| Modulation order |  | QPSK |
| Transport Block Size (without CRC) | Bits | 32 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 1782 |
| PSBCH performance metric (Note 2) |  | SNRPSBCH@1%BLER |
| Note 1: The first symbol is for AGC. Note 2: The performance metric is used for PSBCH simulation assumption. |  |  |

Figure 5 Simulation result for PSBCH demodulation

PSFCH demodulation test

Simulation assumptions [1].

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | FCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | [TDLA30-180] |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 1 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSFCH symbols |  | 2 |
| Number of information bits | bit | 1 |
| PSFCH period | Slot | [1] |
| Cyclic shift pair index |  | 1 or 2 |
| PSFCH performance metric (Note 1) |  | [Pr(ACK miss) < 1%], [Pr(NACK to ACK) < 0.1%], [Pr(DTX to ACK)<1%] |
| Note 1: The performance metric is used for PSFCH simulation assumption. [Note 2: The 1st PSFCH which is used for AGC doesn’t need to be considered in performance evaluation.] |  |  |

Figure 6 Simulation result for PSFCH demodulation

Conclusion

In this contribution, we provide initial simulation results for single link test cases for alignment.

Reference

[1] R4-2012757, “Simulation assumptions for NR V2X test cases,” MediaTek.

[2] R4-2012758, “WF on work scope and general assumptions for NR V2X demodulation performance,” LG Electronics.