3GPP TSG-RAN WG4 Meeting #97-e	R4-2014679

Electronic Meeting, 2 November – 13 November 2020

Source:	China Unicom, ZTE

Title:	TP for TR 37.8126: PC2 for DC_1A_n78A

Agenda item:			10.20.2

Document for:	Approval

1. Introduction

In RAN#89-e meeting, the Revised WID on High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band [1] was approved. This contribution is a text proposal for TR 37.826 to include Power Class 2 for combination DC_1A_n78A.

2. Text Proposal

---Start of changes---

6.x	DC_1A_n78A

6.x.1	Transmitter Characteristics

6.x.1.1	Maximum Output Power

Table 6.x.1.-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_1A_n78A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |

6.x.1.2	Co-existence study

The co-existence study for PC2 DC_1A_n78A can be reused from the PC3 DC_1A_n78A captured in TR37.863-11-11, where:

-    4th intermodulation product may fall into band n1 Rx.

Therefore, additional MSD for IMD4 needs to be defined for PC2 DC_1A_n78A.

6.x.2	Receiver Characteristics

6.x.2.3	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

Table 6.x.2.3-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n78A | 1 | 1950 | 5 | 25 | 2140 | 17.4 | IMD4 |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | N/A |

---End of changes---

Reference

[1] 		RP-202079	Revised WID on High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band, China Unicom