3GPP TSG RAN WG4 Meeting #97-e	R4-2014689

Electronic Meeting, 2-13 Nov., 2020

Agenda item:	10.7.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-21: EN-DC_1-8-42_n28-n77

Document for:	Approval

1	Introduction

EN-DC of 3B LTE and 2B NR of DC_1-8-42_n28-n77 was approved in RAN#88 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

8.X	DC_1-8-42_n28-n77

8.X.1	Operating bands for DC

Table 8.X.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-42_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.X.2	Channel bandwidths per operating band for DC

Table 8.X.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  |  | 8 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n28 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_1A-8A-42A_n28A-n77(2A) |  | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A |  | 1 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 270 |  |
|  |  |  |  | 8 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 42 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n28 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n28 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_1A-8A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 290 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n28 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.X.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-42_n28, DC_1-8-42_n77, DC_1-8_n28-n77, DC_1-42_n28-n77 and DC_8-42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.X.4	∆TIB and ∆RIB values

For DC_1-8-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-42_n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

8.X.5	MSD

As mentioned in 8.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]

7.  Reference

[1]  RP-201372  New WID on Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter- band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), LG Electronics