3GPP TSG-RAN4 Meeting #97-e	R4-2014713

# Online, , 2nd Nov 2020 - 13th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0510 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Tx diversity in tor 38101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of TX diversity requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | 4.3 Added suffix G for TX D Added 6.1G Genral for Tx D and defined power is defined as sum Added 6.2G, 6.2G.1, 6.2G.2, 6.2G3 and 6.2G4 with all pointing requirements are defined in general section and applied similarly Added 6.4G, 6.4G.1 and 6.4G.2 with statement to point requirements from general section and added a formula for composite EVM that applies for UE with TX diversity Added 6.5G with sub-clauses mainly referring to requirements in genral section and added composite ACLR definition for TX div UE’s |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TX diversity requirements are ambiguous and UE with TX diverstiy can not pass conformance |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.512-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact: Requirements are detailed further. UE’s with no TX diversity follow same general requirements and impact is only to UE with TX diversity which have not been able to pass conformance before the change. Change is contained to these UE’s. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

## 4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Transparent TX diversity |

A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional clause (suffixes A to G) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional clause requirements (suffixes A to G) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional clause.

A terminal which supports more than one feature in clauses 5, 6 and 7 shall meet all of the separate corresponding requirements.

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly. For a terminal that supports SUL, the current version of the specification assumes the terminal is not configured with UL MIMO on SUL carrier.

For a terminal that supports operation in shared spectrum, the current version of this specification assumes in the uplink sub-bands within a wideband channel shall be contiguously allocated to the UE.  The uplink requirements for one or more non-transmitted sub-bands between two transmitted sub-bands does not form a part of the current version of this specification.

<End of changes>

<Start of changes>

## 6.1G	General for TX diversity

Unless otherwise stated for UE supporting TX diversity, the transmitter characteristics are defined at of the two TX antenna connectors of the UE. The requirements for the wanted and the unwanted power apply to the sum from the power’s from two TX connectors. For UE with integral antenna only, two reference antennas with a gain of 0 dBi is assumed.

<End of changes>

<Start of changes>

## 6.2G	Transmitter power for TX diversity

### 6.2G.1		UE maximum output power

The UE power class applies according to the section 6.2.1 for the UE with TX diversity.

### 6.2G.2		UE maximum output power reduction

Maximum power reduction fo the UE with TX diversity is applied according to the section 6.2.2. The MPR applies to the sum of the powers from two connectors.

### 6.2G.3		UE additional maximum output power reduction

Additional emission requirements for UE that implments TX diveraity are applied according to the sub-clause 6.2.3 and network signalling (NS)  with the the field additionalSpectrumEmission. Follows section 6.2.3 similarly. Additional maximum power reduction fo the UE with TX diversity is applied according to the section 6.2.3. The A-MPR applies to the sum of the powers from two connectors.

### 6.2G.4		Configured transmitted power

Configured maximum output power for UE which implements TX diversity follows configured transmitted power from sub-clause 6.2.4. The Pcmax and Pumax equaltions and power is defined as thre sum of power’s from two antenna connectors.

<End of changes>

<Start of changes>

## 6.4G	Transmit signal quality for TX diversity

### 6.4G.1		Frequency error for TX divrsity

Frequency error for UE with TX diversity is defined according to sub-clause 6.4.1 with error applicable fo the transmissions from each antenna connector separately.

### 6.4G.2		Transmit modulation quality

#### 6.4G.2.1	Error Vector Magnitude

For UE which implement TX diversity, the composite EVM against what requirements in sub-clause 6.4.2.1 are met is defined as

𝐸𝑉𝑀=√((𝑃_1∗〖𝐸𝑉𝑀〗_1^2+ 𝑃_2∗〖𝐸𝑉𝑀〗_2^2)/(𝑃1 + 𝑃2))

<End of changes>

<Start of changes>

## 6.5G	Output RF spectrum emissions for TX diversity

### 6.5G.1 General

For UE that impelements TX diversity the emission requirements are defined as a sum from both TX connector and limits and conditions in section 6.5 apply.

### 6.5G.2	Adjacent channel leakage requirements

ACLR requirement for UE which implements TX diversity is defined as UE level composite ACLR (ACLRUE) according to:

ACLRUE = (PADJ, TX1 + PADJ, TX2) / (POWN, TX1 + POWN, TX2)

Where

PADJ, TX1 = power of the adjacent channel on TX port 1

POWN, TX1 = power of own channel on TX port 1

PADJ, TX2 = power of the adjacent channel on TX port 2

POWN, TX2 = power of own channel on TX port 2.

The limits, and conditions are defined in sub-clause 6.5.2.4.

<End of changes>