3GPP TSG-RAN WG4 Meeting #97-e	R4-2014724

# Electronic Meeting, 2-13 Nov., 2020

Title:	Discussion on Rel-17 FR2 inter-band CA

Source:	Samsung

Agenda item:	12.3.2

Document for:	Discussion

1.	Introduction

Inter-band CA is main objective of Rel-17 FR2 RF WI. In Rel-16, only IBM requirements for DL CA configuration between different frequency groups, i.e. CA_n260-n261, have been specified. CBM requirements for DL CA configurations with same frequency group are postponed to be specified in Rel-17 due to some controversial issues and limited time. There are two kinds of CA configurations: inter-band CA with same frequency group (e.g. 28+28GHz) and inter-band CA between different frequency groups (e.g. 28+39GHz). A framework is expected on how to apply IBM or CBM to a particular CA configuration, which is a fundamental problem for both DL CA and UL CA. In this contribution, we share our overall views on Rel-17 FR2 inter-band CA including discussion of applicability of CBM and IBM, CBM requirements for DL CA with same frequency group, and IBM requirements for UL CA between different frequency groups.

2. 	Discussion

2.1	applicability of CBM and IBM

According to the WID [1] of Rel-17 FR2 RF work item, a method needs to be agreed on how to apply CBM/IBM to a particular CA configuration:

- Agree a method how applicable CBM/IBM information is captured into specification for a particular CA configuration. Agree how it is decided whether a certain CA configuration is assuming CBM or IBM based requirements (for-example is applicability based on operator request or some general rule or are all CA configurations applicable for both CBM and IBM).

Though it is an objective of DL CA, actually it is also an unavoidable problem for UL CA. IBM requirements for UL CA between different frequency groups have been explicitly captured as objective for UL CA. UL CA is based on DL CA, if DL CA is CBM for a certain CA combination, then UL CA can only be CBM. It is not a reasonable scenario that IBM for UL CA and CBM for DL CA for a certain CA combination.

Observation 1:	for a particular CA configuration, the same CBM/IBM attributes apply for both DL CA and UL CA as long as UL CA is supported

Based on observation 1, when discussing the method on how to apply CBM/IBM to a particular CA configuration, it is better to consider both inter-band DL CA and UL CA. To determine the applicability of CBM/IBM to a particular CA configuration, the pre-condition is the feasibility of the following two scenarios which are also objectives for both DL CA and UL CA:

- Study and if feasible define UE requirements for CBM between different freq. groups (e.g. 28GHz + 37GHz)

- Study and if feasible define UE RF requirements for inter-band CA within the same freq. group (e.g. 28GHz + 28GHz) for (IBM) based on explicitly requested band combinations.

It is necessary to clarify the meaning of CBM and IBM. CBM and IBM are UE capabilities on whether it can support independent beam management for carriers on different bands. They are attributes of UE perspective rather than gNB perspective. For CA configurations between different freq. groups (e.g. 28GHz + 37GHz), the beam forming parameters for the two bands are different to steer beams to the same direction, consequently UE can not perform common beam management between the two bands, no matter gNB is co-located or non-co-located. From UE capability perspective, it is our understanding that CBM is only applicable within same freq. group.

Observation 2:	CBM is not applicable for CA configurations between different freq. groups, but only applicable within same freq. group.

For inter-band CA within the same freq. group (e.g. 28GHz + 28GHz), though there is no practical commercial band combo available yet, in case network deployment is co-located, both CBM UE and IBM UE works; in case network deployment is non-co-located, IBM is the only choice. From UE perspective, IBM within same freq. group is feasible though hardware implementation is more complicated than CBM. It is natural to leave flexibility to UE to report CBM or IBM depending on UE capability.

Observation 3:	IBM is not only applicable for CA configurations between different freq. groups, but also applicable within same freq. group.

Based on above observations on feasibility, a general rule is proposed to apply CBM/IBM for a certain CA configuration, i.e. IBM is applicable for all CA configurations while CBM is only applicable for CA configurations with same freq. group, as illustrated in Table 2.1-1 as an example:

Table 2.1-1 example of CBM/IBM applicability

| CA configuration | IBM or CBM applicability |
| --- | --- |
| 28GHz + 28GHz | UE can report either IBM or CBM depending on UE capability |
| 39GHz + 39GHz | UE can report either IBM or CBM depending on UE capability |
| 28GHz + 39GHz | IBM |

Proposal 1:	IBM is applicable for all CA configurations while CBM is only applicable for CA configurations with same freq. group. UE can report either IBM or CBM depending on UE capability for CA configurations with same freq. group.

2.2	CBM requirements for DL CA with same frequency group

In Rel-16 discussion, CBM requirements for DL CA with same frequency group encounter more difficulties than expected, including MRTD issue, whether or not to define common spherical coverage requirement, whether to introduce new frequency separation class, etc.

MRTD issue will be handled in RRM session. About common spherical coverage, it is our understanding that common spherical coverage is mainly proposed to accommodate IBM. After CBM UE capability is introduced, it is not necessary to apply common spherical coverage requirement to both IBM and CBM. CBM inter-band CA has more similar characteristics to intra-band CA which is also the consensus of companies during previous discussions. From RF requirement perspective, it is natural to apply most intra-band CA RF requirements to CBM inter-band CA.

There is no EIS spherical coverage requirement for intra-band CA. For CBM inter-band CA, it is also not necessary to define spherical coverage requirement to align with intra-band CA. Moreover, CBM is fundamentally different from IBM. For IBM, UE implementation can be more flexible (e.g. different panels between band pairs) so that the spherical coverage correlation between band pairs will be unpredictable, in this case a common spherical coverage requirement is reasonable. But for CBM, just as its name implies, its spherical coverage will almost be in common inherently and it is not necessary to verify again which will be just double effort.

So we propose that no spherical coverage requirement will be defined for CBM band pairs. No spherical coverage requirement for CBM band pairs does not mean the spherical coverage performance of CBM UE is not guaranteed. The inter-band CA spherical coverage performance for both CBM and IBM is guaranteed by single band (non-CA) EIS spherical coverage requirement. The only difference is that IBM has an extra requirement on common coverage area due to its independent beam management feature.

Proposal 2:	For CBM band pairs, no spherical coverage requirement will be defined.

About whether to new frequency separation class for CBM, it is related to the definition of “same frequency group”. Usually we consider n257/258/261 as one frequency group with maximum 5.25GHz frequency separation, n259/260 as one frequency group with maximum 6.5GHz frequency separation. When n262 is specified, it could be considered in another frequency group. UE may not support so wide frequency separation based on CBM, so new frequency separation class is proposed like FR2 intra-band non-contiguous CA. When looking into frequency group and frequency separation class together, we think there are two options for discussion:

Option 1: each band belongs to a frequency group. CBM is applicable for all the CA configurations within the same frequency group depending on UE capability, and introduce new frequency separation class to address too wide frequency separation between bands.

Option 2: there is no explicit frequency group definition. RAN4 decides if CBM is applicable for requested CA combinations, and CBM apply to total frequency separation between bands.

In our understanding, there is no obvious boundary to differentiate frequency groups, in case that new bands might be introduced in the future, for the sake of forward compatibility, it is better not to explicitly define frequency group in specification. CBM applicability could be specified per CA band combinations.

Proposal 3:	For forward compatibility, the “frequency group” term shall not be defined in specification.

2.3	IBM requirements for UL CA with different frequency group

Inter-band UL CA is one of the main objectives for Rel-17 FR2 RF. Different from FR1 inter-band UL CA where UL power of different component carriers are non-directional and summed up as total power, FR2 inter-band CA based on IBM are directional EIRP power for different component carriers, where the peak EIRP may be located in different direction and there is no meaning to sum them up.

Observation 4:	requirements for FR2 inter-band UL CA based on IBM are directional and peak EIRP of different component carriers could not be summed up

So it is natural to define RF requirements per band for UL CA based on IBM. When defining TX requirements per band, power splitting in conduction domain should be considered. Typically, equal power splitting is adopted.

Proposal 4:	IBM requirements shall be defined per band for inter-band UL CA with different frequency group, and power splitting should be considered when deriving requirements.

3. 	Conclusion

Observation 1:	for a particular CA configuration, the same CBM/IBM attributes apply for both DL CA and UL CA as long as UL CA is supported

Observation 2:	CBM is not applicable for CA configurations between different freq. groups, but only applicable within same freq. group.

Observation 3:	IBM is not only applicable for CA configurations between different freq. groups, but also applicable within same freq. group.

Proposal 1:	IBM is applicable for all CA configurations while CBM is only applicable for CA configurations with same freq. group. UE can report either IBM or CBM depending on UE capability for CA configurations with same freq. group.

Proposal 2:	For CBM band pairs, no spherical coverage requirement will be defined.

Proposal 3:	For forward compatibility, the “frequency group” term shall not be defined in specification.

Observation 4:	requirements for FR2 inter-band UL CA based on IBM are directional and peak EIRP of different component carriers could not be summed up

Proposal 4:	IBM requirements shall be defined per band for inter-band UL CA with different frequency group, and power splitting should be considered when deriving requirements.

4. 	References

- RP-202107    “New WID on NR RF Enhancements for FR2”, Nokia, Nokia Shanghai Bell

- R4-2011857 	  “Email discussion summary for [96e][117] NR_RF_FR2_req_enh_Part_3”, Moderator(Qualcomm)