3GPP TSG-RAN WG4 Meeting # 97-e 	                                                          R4-2014737

Electronic Meeting, 2-13 Nov., 2020

Source:	CMCC

Title:	Bandwidth and numerology for NR in 52.6GHz ~ 71GHz

Agenda Item:	13.2.1.1

Document for:	Discussion and Decision

- Introduction

NR specifications that have been developed in Rel-15 and Rel-16 define operation for frequencies up to 52.6GHz, where all physical layer channels, signals, procedures, and protocols are designed to be optimized for users under 52.6GHz. In RAN#86, a new Rel-17 SI (study item) focusing on band between 52.6GHz and 71GHz was approved targeting the imminent commercial opportunities for high data rate communications. The SI will include the following objectives [1]:

- Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz

  - Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4].

  - Identify potential critical problems to physical signal/channels, if any [RAN1].

- Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1].

Note: It is clarified that potential interference impact, if identified, may require interference mitigation solutions as part of channel access mechanism.

In this contribution, we further discuss the required changes for frequencies between 52.6GHz  and 71GHz.

- Numerology and bandwidth

In RAN4#96e-meeting, WF on numerologies for FS_NR_52_to_71GHz for was agreed [2]:

- Channel Bandwidth

  - Maximum channel bandwidth is in [400 – 2160] MHz

    - RAN4 continues to discuss about a maximum channel bandwidth.

  - Minimum channel bandwidth is in [50 – 800] MHz.

    - Companies are encouraged to provide input in the next meeting.

- Sub-Carrier Spacing

  - Further evaluation on feasibility of SCS from 120 kHz to 960 kHz in the next meeting.

    - Companies are encouraged to evaluate feasibility from RAN4 perspective, i.e.,

      - EVM

      - Timing requirement

      - Etc.

  - FFS on 1920 kHz

In RAN1#102e-meeting, it was agreed that NR should be designed with maximum FFT size of 4096 and maximum of 275RBs per carrier; and the candidate supported maximum carrier bandwidth(s) for a cell is between 400MHz and 2160MHz. Since RAN1 agreed the same FFT size 4096 and 275RB number are reused for 52.6GHz to 71GHz, the only way to support larger bandwidth (>400MHz) is to increase the SCS. In Table 1, the values of maximum supported channel bandwidth for larger SCSs are listed.

Table 1: Transmission numerologies and maximum channel bandwidth

|  |  | CP length | Maximum channel bandwidth |
| --- | --- | --- | --- |
| 0 | 15 | 4687ns | 50MHz |
| 1 | 30 | 2344ns | 100MHz |
| 2 | 60 | 1172ns | 200MHz |
| 3 | 120 | 586ns | 400MHz |
| 4 | 240 | 293ns | 800MHz |
| 5 | 480 | 146ns | 1.6GHz |
| 6 | 960 | 73ns | 3.2GHz |

Currently, in unlicensed spectrum from 52.6GHz to 71GHz, 802.11ad/ay systems already support multiple of 2.16GHz blocks. From the Table 1, it can be seen that over 2GHz channel bandwidth can be supported with 4K FFT assumption (3300*960KHz/0.99=3.2GHz) only with 960KHz SCS.

However, larger SCS brings challenges in various aspects.

- Delay spread:  Considering about 7% CP overhead, 960KHz supports 73ns CP length, which is very challenging to cover the delay spread and will cause interference. According to the TR38.901, 960KHz with 73ns CP can only apply to indoor scenario. Further study on the feasibility of 960KHz is needed.

- Multi-beam operation: the beam switching time for existing FR2 is considered to be covered in the CP duration (586ns for 120KHz). However, 73ns CP length of 960KHz is very challenging to cover the beam switching time. So the SSB pattern designs needs further investigation for larger SCS.

- Coverage: in Table 2, we calculate the maximum cell radius that can be supported for preamble formats with different SCSs. It can be seen that the coverage of 960KHz is very limited and can only support over 100m cell radius in format C2.

Table 2: Maximum cell radius for different SCSs and Preamble formants

| Preamble format | Maximum Cell radius (m) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 60 kHz SCS | 120 kHz SCS | 240 kHz SCS | 480 kHz SCS | 960 kHz SCS |
| A1 | 234 | 117 | 59 | 29 | 15 |
| A2 | 527 | 264 | 132 | 66 | 33 |
| A3 | 879 | 439 | 220 | 110 | 55 |
| B1 | 88 | 44 | 22 | 11 | 5 |
| B2 | 264 | 132 | 66 | 33 | 16 |
| B3 | 439 | 220 | 110 | 55 | 27 |
| B4 | 967 | 483 | 242 | 121 | 60 |
| C0 | 1337 | 668 | 334 | 167 | 83 |
| C2 | 2324 | 1162 | 581 | 290 | 145 |

Except for above challenges, 960KHz supports maximum 3.2GHz channel bandwidth. The very large bandwidth also has various implementation challenges, such as IF conversion, I/Q mismatch, increase of noise power, flatness of filter, and ADC, DAC performance.

As the SCS increases, there is a trade-off of many aspects, including the supported maximum channel bandwidth, CP required for delay spread/multi-beam operation, coverage and etc. So we propose to consider up to 480KHz SCS for 52.6GHz~71GHz in Rel-17.

Proposal 1: It is proposed to consider up to 480KHz SCS for 52.6GHz~71GHz.

As we discussed in section 2, assuming 480KHz SCS, the supported maximum channel bandwidth is 1.6GHz. NR beyond 52.6 GHz is anticipated to operate in different bands with different maximum possible bandwidths and support configurable transmission bandwidths as NR does. NR should also support relatively narrow bandwidth smaller than the configured transmission bandwidth to improve RF and baseband power consumption and potentially to improve coverage with peak power efficiency. Additionally, support of narrow bandwidth that is smaller than the configured transmission bandwidth in the UL would help benefit transmission of HARQ feedback and small payload sizes.

Proposal 2: Considering up to 480KHz SCS, the maximum supported channel bandwidth in 52.6GHz ~71 GHz should be less than or equal to 1.6GHz.

802.11ad/ay systems currently support multiple of 2.16 GHz block in unlicensed spectrum.  Considering 1.6GHz maximum channel bandwidth, isn order to achieve competitive high peak data rate with 802.11ad/ay, there is a need for carrier aggregation in 52.6GHz ~71 GHz.

Proposal 3: Carrier aggregation is needed to achieve competitive high peak data rate with 802.11ad/ay in 52.6GHz ~71 GHz

- Conclusion

In this contribution, multiple aspects on required changes for frequencies between 52.6GHz & 71GHz are discussed. Based on the discussion, we have the following proposal:

Proposal 1: It is proposed to consider up to 480KHz SCS for 52.6GHz~71GHz.

Proposal 2: Considering up to 480KHz SCS, the maximum supported channel bandwidth in 52.6GHz ~71 GHz should be less than or equal to 1.6GHz.

Proposal 3: Carrier aggregation is needed to achieve competitive high peak data rate with 802.11ad/ay in 52.6GHz ~71 GHz

References

- 3GPP RP-193259 New SID Study on supporting NR above 52_6GHz, Intel Corporation.

- R4-2011838 WF on numerologies for FS_NR_52_to_71GHz, Intel.