3GPP TSG-RAN WG4 Meeting #97-e                                            	R4-2014740

Electronic Meeting, 2-13 November 2020

Source:	Samsung

Title:	Views and simulation results for Rel-16 Type II PMI test case

Agenda item:		7.9.4.3

Document for:	Discussion

- Introduction

In last RAN4 meeting, test set-up for Rel-16 Type II codebook still open with two options:

- Test set-up with one target UE scheduled  a.k.a SU-MIMO test set-up

- Test set-up with co-scheduled interference UE a.k.a MU-MIMO  set-up

In this contribution, simulation results for both options were evaluated and views on test set-up were provided.

- Simulation results and discussion

  - Simulation results for R16 Type II codebook

    - Results for SU-MIMO Set-up

In following figures, we evaluated performance under following PMI and random PMI with Rel-15 Type I, Rel-15 Type II and Rek-16 enhanced Type II codebook constructions under XP Medium channel correlation and 64QAM rank2.

Overall, we observed enough performance gap with Rel-16 enhanced Type II codebook over than Rel-15 Type II , Rel-15 Type I codebook under SU-MIMO set-up.

The SNR point at 90% full throughput and the TP ratio are summarized in the following table：

Table 1: SNR point and TP ratio in different cases

| Test cases Codebook Type | FDD 16x2 | FDD 16x4 | TDD 16x2 | TDD 16x4 |
| --- | --- | --- | --- | --- |
| Type I | 15.0dB/2.78 | 8.5dB/3.01 | 13.2dB/2.83 | 9.1dB/4.12 |
| Rel-15 Type II | 12.6dB/2.90 | 7.7dB/3.05 | 11.7dB/2.97 | 8.7dB/2.91 |
| Rel-16 Enhanced Type II | 6.3dB/65.8 | 1.8dB/192 | 5.8dB/193 | 3.2dB/--- |

- FDD

Fig.1 Throughput performance of FDD 16x2

Fig.2 Throughput performance of FDD 16x4

- TDD

Fig.3 Throughput performance of TDD 16x2

Fig.4 Throughput performance of TDD 16x4

2.2.2 Results for MU-MIMO Set-up

For MU-MIMO, following precoder generation as the agreed WF [1] with two candidate options for the generation of PMI of co-scheduled UE)

- How to generate Xb (channel for co-scheduled UE) - Option 1: Random PMI - Option 2: Fixed PMI - The PMI is generated once at the start of the simulation and kept in memory to be used in the ZF precoder algorithm when a new PMI from DUT is reported.

For Fixed PMI, we fixed PMI as first beam with beam direction (0,0), phase coefficient for other beams as 1/sqrt(2), and phase coefficient index as 0 (no phase difference).

For MCS, we evaluated MCS7 (QPSK), MCS 13 (16QAM) rank1 transmission.

Based on our evaluation results, it’s not feasible to introduce performance requirements with current agreed MU-MIMO test set-up.  The performance with Type II /Enhanced Type II codebook even worse than with Type I codebook.

  - Summary

From the above summary, we observe that:

Observation 1: With SU-MIMO set-up, the Rel-16 Enhanced Type II could achieve significant gain over Rel-15 Type II and Rel-15 Type I codebook with following PMI.

  - The Rel-16 Enhanced Type II could achieve about 5~6dB gain over Type II codebook, while achieve around 8dB gain over Type I case.

  - It’s feasible to using SU-MIMO set-up to discriminate UE behaviour to ensure UE report Rel-16 Type II codebook other than Rel-15 Type II or Rel-15 Type I codebook.

Observation 2: With MU-MIMO Set-up, the performance of Type II codebook even worse than Type I codebook based on the agreed test parameters.

Observation 3: With SU-MIMO set-up, the throughput ratio among following PMI and random under Rel-16 Type II codebook is quite huge at 90% relative throughput.

  - Test set-up with R16 Type II codebook

SU-MIMO vs MU-MIMO

In last RAN4 meeting, two candidate test-up “SU-MIMO vs MU-MIMO” were discussed. Both of them above test set-up have pros and cons as summarized below:

- Option 1: Test set-up with one target UE scheduled  a.k.a SU-MIMO test set-up

  - Pros: Widely used in existing LTE and NR PMI test cases, the feasibility already been verified; proceed the work quickly since RAN4 has lot of experience

  - Cons: not close to the real network scenario of scheduling  Type II codebook, not reflect the advantage of   Type II codebook over than Type I codebook under Multi-user co-scheduling scenario

- Option 2: Test set-up with co-scheduled interference UE a.k.a MU-MIMO  set-up

  - Pros: Close to type II codebook usage scenario

  - Cons: More complexity, the detailed test set-up still unclear and the feasibility from performance requirements and test aspect need to be further confirmed; bring uncertainty of completing Type II codebook test cases in Rel-16 timeframe

The purpose of this test case was to verify UE reporting accuracy of Type II codebook, and either MU-MIMO or SU-MIMO set-up can serve such test purpose. The detailed schedule of Type II is up to network scheduling. There is no restriction of Type II codebook usage scenario no matter SU-MIMO or MU-MIMO set-up agreed in RAN4.

Furthermore, based on our evaluation results, it's not feasible to introduce performance requirements under agreed MU-MIMO set-up .  There are many uncertainties need to be further evaluated/studied for MU-MIMO scenario before RAN4 start to define proper performance requirements i.e. receiver assumption, co-scheduled UE generation, test feasibility.  With these uncertainties, it’s not realistic to introduce requirements under MU-MIMO in Rel-16 timeframe. Meanwhile we also realized that evaluation performance on MU-MIMO performance was one of the work objectives in approved Rel-17 performance enhancement WI.

Also based on our simulation results, with agreed test set-up for SU-MIMO, we observed larger performance gap with Rel-16 enhanced Type II codebook compared to Rel-15 Type II and Rel-15 Type I codebook. It’s feasible to using SU-MIMO set-up to discriminate UE behaviour to ensure UE report Rel-16 Type II codebook other than Rel-15 Type II or Rel-15 Type I codebook.

Based on above analysis, we proposed to conclude the work in Rel-16:

Proposal1: Overall Test set-up:

- Introduce Rel-15 Type II PMI test cases under SU-MIMO test set-up in Rel-16 timeframe.

- Further study and define proper performance requirements if needed under MU-MIMO scenarios in Rel-17 performance enhancement WI.

Reference test point and test requirement

Based on our evaluation results, the throughput ratio among following PMI and random under Rel-16 Type II codebook is quite huge at 90% relative throughput under SU-MIMO set-up. And such test point may be not feasible to introduce performance requirements.  The major reason for such high TP ratio was the absolute throughput with random PMI under Rel-16 Type II extreme low especially under low SNR region.

In order to achieve feasible test point and requirements, below candidate option can be considered: using 95 % relative TP as test point. Meanwhile for the implementation of random PMI, we can consider to restrict random PMI follow Rel-15 Type I codebook which also a subset of Rel-16 Type II codebook.

Proposal 2:  In order to achieve test point and test requirements under SU-MIMO test set-up:

  - Using 95% relative throughput with following PMI as test point

  - Restrict random PMI to random selected within Rel-15 Type I codebook set

Table 2: SNR point and TP ratio in different cases under SU-MIMO

| Test cases Codebook Type | FDD 16x2 | FDD 16x4 | TDD 16x2 | TDD 16x4 |
| --- | --- | --- | --- | --- |
| Rel-16 Enhanced Type II (90% relative TP, full set of Rel-16 Type II codebook for random PMI) | 6.3dB/65.8 | 1.8dB/192 | 5.8dB/193 | 3.2dB/--- |
| Rel-16 Enhanced Type II (95% relative TP, Restricted set within Rel-15 Type I codebook for random PMI) | 7.7 dB/10 | 2.8 dB/10 | 7.1dB/15 | 5.5dB/6 |

- Conclusion

In this contribution, simulation results for Rel-16 Type II, Rel-15Type II and Type I codebook were provided based agreed candidate test set-up (SU-MIMO and MU-MIMO).

Observation 1: With SU-MIMO set-up, the Rel-16 Enhanced Type II could achieve significant gain over Rel-15 Type II and Rel-15 Type I codebook with following PMI.

  - The Rel-16 Enhanced Type II could achieve about 5~6dB gain over Type II codebook, while achieve around 8dB gain over Type I case.

  - It’s feasible to using SU-MIMO set-up to discriminate UE behaviour to ensure UE report Rel-16 Type II codebook other than Rel-15 Type II or Rel-15 Type I codebook.

Observation 2: With MU-MIMO Set-up, the performance of Type II codebook even worse than Type I codebook based on the agreed test parameters.

Observation 3: With SU-MIMO set-up, the throughput ratio among following PMI and random under Rel-16 Type II codebook is quite huge at 90% relative throughput.

Furthermore, following proposals given for remaining open issues:

Proposal1: Overall Test set-up:

- Introduce Rel-15 Type II PMI test cases under SU-MIMO test set-up in Rel-16 timeframe.

- Further study and define proper performance requirements if needed under MU-MIMO scenarios in Rel-17 performance enhancement WI.

Proposal 2:  In order to achieve test point and test requirements under SU-MIMO test set-up:

  - Using 95% relative throughput with following PMI as test point

  - Restrict random PMI to random selected within Rel-15 Type I codebook set

Reference

[1] R4-2012762	R4-2012761	Way forward on PMI reporting requirement for NR eMIMO, Qualcomm, Samsung

[2] R4-2012765	Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports, Ericsson

[3] R4-2010141, Views and simulation results for PMI test cases, Samsung