3GPP TSG-RAN WG4 meeting #97-e	R4-2014743

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	7.9.4.2.1

Source:	Samsung

Title:	Simulation results for Single-DCI SDM scheme

Document for:	Discussion

1	Introduction

RAN1 has introduced Multi-TRP transmission in NR eMIMO demod. Multi-DCI and Single-DCI are the two transmission schemes. In the previous meetings, RAN4 has agreed to define at least PDSCH with multi-DCI based scheme and single-DCI based SDM scheme.

In RAN4 #96-e meeting, companies agreed with the way forward [1] and simulation assumptions [2] to define the performance test cases. In this proposal, we provide our simulation results and discuss the detailed assumptions.

2	Test assumptions for Single-DCI SDM scheme

RAN4 has defined 4 test cases for FDD mode and 4 test cases for TDD mode in single-DCI SCM scheme. The main agreements are summarized as below, while more detailed assumptions are found in [2].

- Reference for timing offset/frequency offset - Using TP which carry on SSB transmission with default TCI state #0 as the reference TP (TP1) - Timing offset = time offset among TP2 and TP1 - Frequency offset = frequency offset among TP2 and TP1 - Baseline receiver assumption for FFT window timing - Assuming UE always fix FFT timing based on TCI state #0 (TP1) as baseline assumption to define RAN4 performance requirements - Timing offset among multi-panel/TRP - Timing offset values - ∆t=2-μ∆t1, ∆t1 = [-0.5, 2]μs - Frequency offset among multi-panel/TRP - 200Hz for FR1 FDD 15kHz - 300Hz for FR1 TDD 30kHz

Table 2.1 Simulation cases for Single-DCI SDM scheme

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1b: 0Hz / 2us | 70% of max Tput |
|  |  |  |  |  |  | Test 1a: 200Hz / -0.5us |  |
| 3-2 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1b: 0Hz / 2us | 70% of max Tput |
|  |  |  |  |  |  | Test 1a: 200Hz / -0.5us |  |
| 4-1 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1b: 0Hz / 1us | 70% of max Tput |
|  |  |  |  |  |  | Test 1a: 300Hz / -0.25us |  |
| 4-2 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1b: 0Hz / 1us | 70% of max Tput |
|  |  |  |  |  |  | Test 1a: 300Hz / -0.25us |  |

3	Simulation results

Fig. 1: case 3-1 and case 3-2 results for FDD

Table 3.1 SNR point with 70% of max throughput for FDD mode

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | 70% of max Throughput (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1b: 0Hz / 2us | 15.7 |
|  |  |  |  |  |  | Test 1a: 200Hz / -0.5us | 16.4 |
| 3-2 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1b: 0Hz / 2us | 9.8 |
|  |  |  |  |  |  | Test 1a: 200Hz / -0.5us | 10.7 |

Fig. 2: case 4-1 and case 4-2 results for TDD

Table 3.2  SNR point with 70% of max throughput for TDD mode

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | 70% of max Throughput (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4-1 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1b: 0Hz / 1us | 15.1 |
|  |  |  |  |  |  | Test 1a: 300Hz / -0.25us | 16.0 |
| 4-2 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1b: 0Hz / 1us | 9.6 |
|  |  |  |  |  |  | Test 1a: 300Hz / -0.25us | 10.4 |

The SNR point with case 3-1, case 3-2,case 4-1,case 4-2 are collected in table 3.1 and table 3.2.

The performance with frequency offset is degraded slightly than with no frequency offset case.

The impact with negative time offset is more significantly than with positive time offset case.

The FDD performance is similar with TDD performance.

4	Summary

Observation 1: The SNR point are summarized as below

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | 70% of max Throughput (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1b: 0Hz / 2us | 15.7 |
|  |  |  |  |  |  | Test 1a: 200Hz / -0.5us | 16.4 |
| 3-2 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1b: 0Hz / 2us | 9.8 |
|  |  |  |  |  |  | Test 1a: 200Hz / -0.5us | 10.7 |
| 4-1 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1b: 0Hz / 1us | 15.1 |
|  |  |  |  |  |  | Test 1a: 300Hz / -0.25us | 16.0 |
| 4-2 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1b: 0Hz / 1us | 9.6 |
|  |  |  |  |  |  | Test 1a: 300Hz / -0.25us | 10.4 |

Observation 2: The performance with frequency offset is degraded slightly than with no frequency offset case.

Observation 3: The impact with negative time offset is more significantly than with positive time offset case.

Observation 4: The FDD performance is similar with TDD performance.

References

- R4-2012662, “Way forward for general and PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes (eMBB)​”, Huawei, HiSilicon, RAN4 #96-e Electronic meeting.

- R4-2012664, “Simulation assumption for PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes”, Ericsson, RAN4 #96-e Electronic meeting

- R4-2010140, “Test case design for PDSCH requirements with Multi-TRP/Panel transmission”, Samsung, RAN4 #96-e Electronic Meeting