3GPP TSG-RAN WG4 meeting #97-e	R4-2014744

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	7.9.4.2.2

Source:	Samsung

Title:	Simulation results for Multi-DCI transmission scheme

Document for:	Discussion

1	Introduction

RAN1 has introduced Multi-TRP transmission in NR eMIMO demod. Multi-DCI and Single-DCI are the two transmission schemes. In multi-DCI scheme, each DCI can schedule PDSCH independently.

In the last meeting, companies agreed with the way forward [1] and simulation assumptions [2] to define the performance test cases. In this proposal, we provide our simulation results and summary the performance for each case.

2	Test assumptions for Multi-DCI scheme

RAN4 has defined 2 test cases for FDD mode and 2 test cases as shown in Table 2.1 for TDD mode in Multi-DCI based scheme. The main agreements are summarized as below, while more detailed assumptions are presented in [2].

- Reference for timing offset/frequency offset - Using TP which carry on SSB transmission with default TCI state #0 as the reference TP (TP1) - Timing offset = time offset among TP2 and TP1 - Frequency offset = frequency offset among TP2 and TP1 - Baseline receiver assumption for FFT window timing - Assuming UE always fix FFT timing based on TCI state #0 (TP1) as baseline assumption to define RAN4 performance requirements - Timing offset among multi-panel/TRP - Timing offset values - ∆t=2-μ∆t1, ∆t1 = [-0.5, 2]μs - Frequency offset among multi-panel/TRP - 200Hz for FR1 FDD 15kHz - 300Hz for FR1 TDD 30kHz - Non-overlapping case - FDD: PDSCH #1: PRB #0 to PRB #25, PDSCH #2: PRB #26 to PRB #51 - TDD: PDSCH #1: PRB #0 to PRB #52, PDSCH #2: PRB #53 to PRB #105 - Antenna ports - TCI #1: {1000,1001}, TCI #2{1002,1003}

Table 2.1 Simulation test cases for Multi-DCI scheme

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition (Note 1) | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 2a: 200Hz / -0.5us | 70% of max Tput |
| 1-2 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 2a: 200Hz / -0.5us | 70% of max Tput |
| 2-1 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 2a: 300Hz / -0.25us | 70% of max Tput |
| 2-2 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 2a: 300Hz / -0.25us | 70% of max Tput |

3	Simulation results

Fig. 1: case 1-1 and case 1-2 results for FDD

Table 3.1 SNR point with 70% of max throughput for FDD mode

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | 70% of max Throughput (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 2a: 200Hz / -0.5us | 16.7 |
| 1-2 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 2a: 200Hz / -0.5us | 11.6 |

Fig. 2: case 4-1 and case 4-2 results for TDD

Table 3.2  SNR point with 70% of max throughput for TDD mode

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | 70% of max Throughput (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 2a: 200Hz / -0.25us | 16.2 |
| 2-2 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 2a: 200Hz / -0.25us | 11.7 |

The SNR point with case 1-1, case 1-2,case 2-1,case 2-2 are collected in table 3.1 and table 3.2.

The performance of FDD is similar to TDD ones.

The performance of ’2x4’ achieves about 5dB gain than ’2x2’ case.

4	Summary

Through the above display, we conclude that:

Observation 1: The SNR point are summarized in table 3.1 and table 3.2.

Observation 2: The performance of FDD is similar to TDD ones.

Observation 3: The performance of ’antenna configuration 2x4’ achieves about 5dB gain than ’antenna configuration 2x2’ case.

References

- R4-2012662, “Way forward for general and PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes (eMBB)​”, Huawei, HiSilicon, RAN4 #96-e Electronic meeting.

- R4-2012664, “Simulation assumption for PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes”, Ericsson, RAN4 #96-e Electronic meeting

- R4-2010140, “Test case design for PDSCH requirements with Multi-TRP/Panel transmission”, Samsung, RAN4 #96-e Electronic Meeting