3GPP TSG-RAN WG4 Meeting #97-e                                                                R4-2014769

Electronic Meeting, 2 Nov. – 13 Nov., 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on V2X UE Resource Selection Tests for Re-evaluation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2020-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The re-evaluation test is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new test case for re-evaluation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.12.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

A.12.6.x	V2X UE Resource Selection Tests for Re-evaluation

A.12.6.x.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to autonomous resource re-evaluation for V2X UE in mode 2 defined in clause 12.5. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A.12.6.x.1-1and A.12.6.x.1-2 below. There are 130 active V2X sidelink UEs in this test. The first 100 active V2X sidelink UEs are scheduled with 50ms periodicity. The last 30 active V2X sidelink UEs are aperiodic service UE with retransmission reservation period equaling 15ms.

Both the UE under test and active V2X sidelink UEs select GNSS as synchronization reference source. The test system can emulate and send the GNSS signal to the test UE and active V2X sidelink UEs. The test parameters for GNSS signals are defined in B.6.1.

The test consists of three duration T0, T1, T2.

[During T0, the signal from Test Equipement are configured. The resource occupied by the active V2X sidelink UEs is expected to be excluded in the resource selection procedure such that the measured PSSCH-RSRP is above the measurement threshold. The test equipment shall just configure the resource pool for the test UE without the MAC PDU for transmission channel configuration.]

During T1, the signal from Test Equipement are configured. Some of the resource occupied by the active V2X sidelink UEs is expected to be excluded in the resource selection procedure such that the measured PSSCH-RSRP is above the measurement threshold and some of the resource occupied by the active V2X sidelink UEs is expected to be included in the resource selection procedure such that the measured PSSCH-RSRP is below the measurement threshold. The test system shall emulate the active V2X sidelink UEs to transmit PSCCH/PSSCH every [50ms] according to the RSRP level specified in the Table A.12.6.x.1-2.

At the end of T1, the test equipment shall send one message with a SL-SCH MAC PDU as specified in Clause 6.1.6 in TS 36.321, in order to make sure that the UE under test shall be scheduled to periodically transmit PSCCH/PSSCH.

During T2, the additional aperiodic active V2X sidelink UEs from Test Equipement are configured in the beginning [30] slots, and the resource occupied by these active V2X sidelink UEs is expected to be excluded in the resource re-evaluation procedure such that the measured PSSCH-RSRP is above the measurement threshold shown in Table A.12.6.x.1-2. The test system shall emulate the active V2X sidelink UEs to transmit PSCCH/PSSCH with resource reservation period equaling [15ms] with the maximum number of reserved PSCCH/PSSCH resources equalling [2].

During T2, the test UE is expeted to reselect the resources and transmit the PSCCH/PSSCH in the newly re-evaluated resources.

Table A.12.6.x.1-1: Test Parameters for V2X UE Resource Selection Tests for Re-evaluation

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | TDD carrier in Band 47 |
| Channel Bandwidth (BWchannel) |  | MHz | 20 |  |
| SCS |  | kHz | 30 |  |
| V2X sidelink communication pre-configuration |  |  | As specified in Table A.3.19.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool |  |  | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213]) |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 1 | Indicates the number of sub-channels for TX resource pool |
| Number of Active Sidelink UEs |  |  | [130] | Active Sidelink UE i = 0, .., 129 |
| SL-ThresPSSCH-RSRP-r16 |  |  | [17] | Corresponding -96 dBm as defined in Section 6.3.5 in TS38.331 |
| Active Sidelink UEs(0-99) | V2X sidelink Communication preconfiguration |  | As specified in Table A.3.24.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
|  | [sl-TimeResource-r16 included in SL-ResourcePool] |  | [10000000000000000000 00000000000000000000 00000000000000000000 00000000000000000000 00000000000000000000] | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213 [23]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize included in SL-ResourcePool |  | 50 | Indicates the size of sub-channels for TX resource pool |
|  | [sl-OffsetIndicator] |  | [i mod 100] | For Active Sidelink UE i, where i = 0, .., 99 |
|  | [sl-ResourceReservePeriod2-r16] |  | [50] | [Unit:ms] |
| Active Sidelink UEs(100-129) | V2X sidelink Communication preconfiguration |  | As specified in Table A.3.24.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
|  | [sl-TimeResource-r16 included in SL-ResourcePool] |  | 10000000000000000000 00000000000000000000 00000000000000000000 00000000000000000000 00000000000000000000 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213 [23]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize included in SL-ResourcePool |  | 50 | Indicates the size of sub-channels for TX resource pool |
|  | [sl-OffsetIndicator] |  | [i mod 100] | For Active Sidelink UE i, where i = 100, .., 129 |
|  | [sl-ResourceReservePeriod2-r16] |  | [15] | [Unit:ms] |
| Timing offset among Active Sidelink UEs |  | s | 3 | Synchronous |
| [T0] |  | s | [1] |  |
| T1 |  | ms | 50 |  |
| T2 |  | ms | 50 |  |

Table A.12.6.x.1-2: Active Sidelink UE Specific Test Parameters for V2X UE Resource Selection Tests for Re-evaluation (UE #0...99)

| Parameter | Unit | Active Sidelink UE i (i = 0, .., 99) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| PSCCH RMC (defined in A.3.24.3) | - | [CC.1A HD] |  |
| PSSCH RMC (defined in A.3.24.3) | - | [CD.1B HD] |  |
| Note1 | dBm/SCS | [-103] |  |
| PSSCH1 Note 3 | dB | [22] | [22] |
| PSSCH2 Note 4 | dB | [2] | [2] |
| PSSCH3 Note 5 | dB | [12] | [12] |
| S-RSRP1 Note 2, Note 3 | dBm/SCS | [-81] | [-81] |
| S-RSRP2 Note 2, Note 4 | dBm/SCS | [-101] | [-101] |
| S-RSRP3 Note 2, Note 5 | dBm/SCS | [-91] | [-91] |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: S-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: S-RSRP1 is the S-RSRP level measured on the slot #50-64, and slot #85-99with “SFN mod 10 = 0”. Note 4: S-RSRP2 is the S-RSRP level measured on the slot #30 – 49 with “SFN mod 10 = 0”. Note 5: S-RSRP3 is the S-RSRP level measured on the slot #0-29 and slot #65-84 with “SFN mod 10 = 0”. |  |  |  |

Table A.12.6.x.1-3: Active Sidelink UE Specific Test Parameters for V2X UE Resource Selection Tests for Re-evaluation (UE #100…129)

| Parameter | Unit | Active Sidelink UE i (i = 100, .., 129) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| PSCCH RMC (defined in A.3.24.3) | - | [CC.1A HD] |  |
| PSSCH RMC (defined in A.3.24.3) | - | [CD.1B HD] |  |
| Note1 | dBm/15 kHz | [-103] |  |
| PSSCH | dB | -infinity | [22] |
| PSSCH Note2 | dB | -infinity | [9.73] |
| S-RSRP Note 2, Note 3 | dBm/SCS | -infinity | [-81] |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: S-RSRP is the S-RSRP level measured on the slot #0 – 29 with “SFN mod 10 = 0”. |  |  |  |

A.12.6.x.2	Test Requirements

The rate of PSSCH transmissions on the resources on [slot #65 – 84 with “SFN mod 10 = 0”] shall be more than [90%] during T2.

End of Change