3GPP TSG-RAN WG4 Meeting #97-e	R4-2014789

# Electronic Meeting, Nov 2 – 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133 TC for E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Perf |  |  |  |  |  | Date: |  |  | 2020-10-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The test case for E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching is specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying the test case for E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A5.5.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1 ------------------------------------------------------------

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching

A.5.5.2.7.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE needs to transmit periodic or aperiodic SRS and/or non-contention based PRACH on a PUSCH-less carrier of SCell, the UE can perform carrier based switching to one PUSCH-less SCCs from a CC with PUSCH. The test will verify the interruption requirements on active serving cell in SCG in clause 8.2.1.2.13. Supported test configurations are shown in table A.5.5.2.7.1-1.

In the test there are three cells: cell1, cell2 and cell3. Cell1 is E-UTRAN PCell on the primary component carrier. Cell3 is E-UTRAN SCell on the TDD secondary component carrier which operates in downlink without PUCCH/PUSCH. Cell2 is NR FR2 PSCell. The UE is configured with the SRS switching between E-UTRAN PCell and E-UTRAN SCell. The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.7.1-2, A.4.5.2.7.1-3 and xxx below. And the E-UTRAN cell specific test parameters (for cell1 and cell3) can refer to Table A.3.7.2.1-1. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 LTE PCell and NR PSCell are continuously scheduled in DL. Immediately at the beginning of T2, a PDCCH with SRS-TPC-RNTI is sent to the UE to initiate SRS switching.

Table A.5.5.2.7.1-1: E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD(cell1), LTE TDD (cell3), NR 120 kHz SSB SCS, 100 MHz bandwidth,TDD duplex mode |
| 2 | LTE TDD(cell1), LTE TDD (cell3), NR 120 kHz SSB SCS, 100 MHz bandwidth,TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.5.5.2.7.1-2: General test parameters for E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is NR RF channel and the other two are E-UTRAN RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Active PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Activated SCell |  | Cell3 | SCell on E-UTRAN RF channel number 3. |
| CP length |  | Normal | Applicable to Cell1, Cell2 and Cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 | UE shall perform SRS switching during T2 |

Table A.5.5.2.7.1-3: NR cell specific test parameters for E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching

| Parameter |  | Unit | Cell 2 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR2 |
| Duplex mode | Config 1,2 |  | TDD |
| TDD configuration | Config 1,2 |  | TDDConf.3.1 |
| BWchannel | Config 1,2 | MHz | 100: NRB,c = 66 |
| Downlink initial BWP Configuration | Config 1,2 |  | DLBWP.0.1 |
| Downlink dedicated BWP Configuration | Config 1,2 |  | DLBWP.1.1 |
| Uplink initial BWP configuration | Config 1,2 |  | ULBWP.0.1 |
| Uplink dedicated BWP configuration | Config 1,2 |  | ULBWP.1.1 |
| TRS configuration | Config 1,2 |  | TRS.2.1 TDD |
| TCI state | Config 1,2 |  | TCI.State.0 |
| PDSCH Reference measurement channel | Config 1,2 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | Config 1,2 |  | CR.3.1 TDD |
| RMC CORESET Reference Channel | Config 1,2 |  | CCR.3.1 TDD |
| OCNG Patterns |  |  | OP.1 |
| SSB Configuration |  |  | SSB.1 FR2 |
| SMTC Configuration | Config 1,2 |  | SMTC.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Ês/Noc |  | dB | 17 |
| Propagation Condition |  |  | AWGN |
| Time offset to cell1 Note 2 |  | ms | 3 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Receive time difference of signals received between subframe timing boundary of E-UTRA PCell and slot timing boundary of PSCell including time alignment error between the two cells |  |  |  |

Table A.5.5.2.7.1-4: NR cell specific OTA related test parameters for E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Cell2 |
| --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |
| Assumption for UE beamsNote 6 |  | Fine |
| Note1 | dBm/15kHzNote4 | -112 |
| Note1 | dBm/SCSNote3 | -102.97 |
|  | dB | 17 |
| SS-RSRPNote2 | dBm/SCS Note4 | -85.97 |
|  | dB | 17 |
| IoNote2 | dBm/95.04 MHz Note4 | -56.90 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |

Table A.5.5.2.7.1-5: Sounding Reference Symbol Configuration for E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | Sc8 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for E-UTRAN FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 47 | SRS periodicity of 40ms. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

A.5.5.2.7.2	Test Requirements

The UE shall be continuously scheduled in NR FR2 PSCell throughout the test and during the time duration T2, each interruption on NR FR2 PSCell shall not exceed X defined in Table A.5.5.2.7.2-1.

Table A.5.5.2.7.2-1: Interruption length X (slot) E-UTRAN – NR at E-UTRA SRS carrier based switching

|  | NR Slot | Interruption length X |
| --- | --- | --- |
|  | length (ms) | (slots) |
| 2 | 0.25 | 5 |
| 3 | 0.125 | 9 |

The rate of correct events observed during repeated tests shall be at least 90%.

------------------------------------------------------------- End of change 1 ------------------------------------------------------------