3GPP TSG-RAN WG4 Meeting #97-e	R4-2014823

Electronic Meeting, August 2nd – 13th 2020

Agenda item:	7.15.3.2.3

Source: 		NTT DOCOMO, INC.

Title: 	Views on NR PUSCH for UL timing adjustment

Document for:	Approval

- Introduction

In RAN4 #96-e, WF on Rel-16 NR HST BS demodulation requirements was approved [1]. A few open issues are remaining and captured in the WF.

In this contribution, we provide views on PUSCH requirements for UL timing adjustment.

- Discussion on SCS/CBW combinations for scenario X

In RAN4 #96-e, it was approved to introduce scenario X. However, some open issues related scenario X were remained and captured in the WF:

RAN4 #96-e [1] - SCS/CBW combinations - Option 1: Have requirements for - 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz. - Option 2: Have requirements for - 15kHz: 5MHz; 30kHz: 10MHz. - Option 3: Have requirements for all Rel-15 non-HST PUSCH bandwidths. - Other options not precluded. - Applicability rules concerning SCS/CBW - Option 1: Re-use non-HST PUSCH applicability rules. - Option 2: Re-use HST PUSCH applicability rules. - Other opinions not precluded.

Firstly, the combinations of SCS and CBW should be aligned with other scenarios for UL timing adjustment. For scenario Y and scenario Z, the combinations have been agreed to define 5/10MHz for 15kHz SCS and 10/40MHz for 30kHz SCS. In addition, the last meeting, implicit test passing of scenario X was agreed if a BS declares to support HST and passes test of scenario Y or scenario Z. Thus, we believe that the combinations of SCS and CBW for scenario X should be same with scenario Y/Z in order to match test coverage among scenario X/Y/Z.

Secondly, the combinations should be aligned with PUSCH HST including single tap HST and multi-path fading with high Doppler frequency (i.e., 600Hz and 1200Hz). The combinations for NR PUSCH HST performance requirements have been already agreed to define 5/10MHz for 15kHz SCS and 10/40MHz for 30kHz SCS.

Observation 1: The requirements of PUSCH UL timing adjustment and PUSCH HST are considered with following SCS/CBW combinations:

- 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz.

From above observation, it is reasonable to define 5/10MHz for 15 kHz SCS and 10/40MHz for 30 kHz SCS as UL timing adjustment for scenario X.

Proposal 1: Introduce requirements of 5/10MHz for 15kHz SCS and requirements of 10/40MHz for 30kHz SCS (Option 1).

Basically, as mentioned above, we believe that the test for scenario X should be done under the same conditions as for scenario Y and scenario Z. Regarding applicability rule concerning CBW/SCS for scenario X, RAN4 can reuse HST PUSCH applicability rules.

Proposal 2: RAN4 reuses HST PUSCH applicability rules (Option 2).

3.	Conclusion

In this contribution, we provide our views on NR PUSCH for UL timing adjustment. The following proposals are obtained.

Observation 1: The requirements of PUSCH UL timing adjustment and PUSCH HST are considered with following SCS/CBW combinations:

- 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz.

Proposal 1: Introduce requirements of 5/10MHz for 15kHz SCS and requirements of 10/40MHz for 30kHz SCS (Option 1).

Proposal 2: RAN4 reuses HST PUSCH applicability rules (Option 2).

References

- R4-2012676, “WF on Rel-16 NR HST BS demodulation requirements”, Nokia, RAN4 #96e, August 2020