3GPP TSG-RAN WG4 Meeting #97-e                                R4-2014840

Online, 2nd Nov-13th Nov, 2020

Source:	KDDI

Title:	TP for TR 37.717-11-21: DC_1A-18A_n41A-n77A

Agenda item:			10.7.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-11-21 to include DC_1A-18A_n41A-n77A and according to the request in [1].

# 2. Reference

- RP-201477, New WID on Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 7.x	DC_3-18_n41-n77

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n41A-n77A | 1 | 1920MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 235 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.x.3	Co-existence studies

Co-existence studies of DC_1A-18A_n41A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.x.4	∆TIB and ∆RIB values

For DC_1-18_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n41-n77 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 7.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41-n77 | 1 | 0.2 |
|  | 18 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

### 7.x.5	MSD

All MSD requirements forDC_1-18_n41-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

<End of Text Proposal>