3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014854

# Electronic Meeting, 2-13 Nov., 2020

Source:	Verizon, LGE, Samsung, Nokia, Ericsson, Qualcomm

Title:	TP for TR 37.717-21-11: DC_2-48_n77

Agenda item:	10.4.2

Document for:	Approval

# Introduction

In RAN#89-e meeting, the “Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” [1] was approved. This is a text proposal to introduce DC_2A-48A_n77A into TR 37.717.21-11.

# Reference

[1] RP-201728: WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) Huawei, HiSilicon

# Text Proposal

<Start of Text Proposal>

## 5.x		DC_2-48_n77

### 5.x.1	 Operating bands for DC

Table 5.x.1-1: Inter-band DC configuration (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-48A_n77A | DC_2A_n77A DC_48A_n77A |

### 5.x.2	 Co-existence studies

For UE coexistence study of Band 2 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.x.2-1.

Table 5.x.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1390 | 2350 | 5150 | 6110 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 500 | 520 | 4690 | 6550 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 7000 | 8020 | 8450 | 10310 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1350 | 2430 | 7990 | 10750 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8850 | 9930 | 11750 | 14510 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4700 | 2780 | 10300 | 12220 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14950 | 11290 | 4340 | 3200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 18710 | 10700 | 11840 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 6080 | 870 | 2850 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 16420 | 12150 | 14130 |

For UE coexistence study of Band 48 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.x.2-2.

Table 5.x.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 14200 | 14800 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 17750 | 18500 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 400 | 650 | 6850 | 7900 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2900 | 4100 | 2900 | 4850 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 10400 | 11600 | 10150 | 12100 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 6450 | 7800 | 6200 | 9050 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 13950 | 15300 | 13450 | 16300 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1300 | 800 | 13700 | 15800 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 9500 | 11500 | 10000 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16750 | 20500 | 17500 | 19000 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5500 | 2500 | 4500 | 2250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17000 | 20000 | 17250 | 19500 |

The Rx impacts can be identified as below,

- There is no IMD issue for Rx of band 2 with UL DC_48_n77.

- 5th order harmonics generated by DC_2_n77 uplink may fall into own Rx of band 48.

- As the duplex mode for band 48 and n77 is TDD and both bands shall be synchronized to align uplink and downlink timing across and within the two bands for the U.S. C-band (R4-2008118), thus there is no impact to this TDD band 48 with the UL DC_2_n77.

### 5.x.3	 ∆TIB and ∆RIB values

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n77 | 2 | 0.3 |
|  | 48 | 0.6 |
|  | n77 | 0.5 |

Table 5.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n77 | 2 | 0 |
|  | 48 | 0.2 |
|  | n77 | 0.1 |

### 5.x.4	Reference sensitivity exceptions

There is no reference sensitivity exception (MSD) for DC_2-48_n77 combination.

<End of Text Proposal>