3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014864

# Electronic Meeting, 2-13 Nov., 2020

Source:	Verizon, LGE, Samsung, Nokia, Ericsson, Qualcomm

Title:	TP for TR 37.717-21-11: DC_48-66_n77

Agenda item:	10.4.2

Document for:	Approval

# Introduction

In RAN#89-e meeting, the “Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” [1] was approved. This is a text proposal to introduce DC_48A-66A_n77A into TR 37.717.21-11.

# Reference

[1] RP-201728: WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) Huawei, HiSilicon

# Text Proposal

<Start of Text Proposal>

## 5.x		DC_48-66_n77

### 5.x.1	 Operating bands for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_48A-66A_n77A | DC_48A-77A DC_66A-77A |

### 5.x.2	 Co-existence studies

For UE coexistence study of Band 48 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.x.2-1.

Table 5.x.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 14200 | 14800 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 17750 | 18500 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 400 | 650 | 6850 | 7900 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2900 | 4100 | 2900 | 4850 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 10400 | 11600 | 10150 | 12100 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 6450 | 7800 | 6200 | 9050 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 13950 | 15300 | 13450 | 16300 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1300 | 800 | 13700 | 15800 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 9500 | 11500 | 10000 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16750 | 20500 | 17500 | 19000 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5500 | 2500 | 4500 | 2250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17000 | 20000 | 17250 | 19500 |

For UE coexistence study of Band 66 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.x.2-2.

Table 5.x.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1520 | 2490 | 5010 | 5980 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | -780 | 260 | 4820 | 6690 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6720 | 7760 | 8310 | 10180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 930 | 2040 | 8120 | 10890 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8430 | 9540 | 11610 | 14380 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | -4980 | -3040 | 10020 | 11960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | -15090 | -11420 | -3820 | -2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 18580 | 10140 | 11320 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | -9180 | -6340 | 1260 | 3270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 16160 | 11730 | 13740 |

The Rx impacts can be identified as below.

- There is no IMD issue for Rx of band 66 with UL DC_48_n77

- 4th and 5th order IMD products generated by DC_66_n77 uplink may fall into own Rx of band 48.

### As the duplex mode for band 48 and n77 is TDD and both bands shall be synchronized to align uplink and downlink timing across and within the two bands for the U.S. C-band (R4-2008118), thus there is no impact to this TDD band 48 with the UL DC_66_n77.

### 5.x.3	 ∆TIB and ∆RIB values

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n77 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n77 | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n77 | 0.5 |

### 5.x.4	Reference sensitivity exceptions

There is no reference sensitivity exception (MSD) for DC_48-66_n77 combination.

<End of Text Proposal>