3GPP TSG-RAN4 Meeting #97-e	R4-2014903

# Online, , 2nd Nov 2020 - 13th Nov 2020

Agenda item:	7.1.2.1

Source:	Apple Inc.

Title:	PC5 NR-U MPR for NS_53 and NS_54

WI/SI:	NR_unlic-Core

Release:	Rel-16

Document for:	Decision

# 1	Introduction

This contribution presents simulation results for NR-U PC5 n96 with NS_53 and NS_54. The simulations were conducted according to the latest agreements.

# 2	Discussion

Simulations have been done for NR-U in band n96. The PA was calibrated with the agreed QPSK DFT-s-OFDM waveform in [1]. The detailed list of the setup is provided in the following:

- Power Class: PC5

- Calibration: QPSK DFT-s-OFDM 100RB3 with 1dB backoff

- ACLR: 27dB

- SEM: As agreed in [2]

- EVM: PA+Image budget used

- Image rejection: -28dBc

- Carrier leakage rejection: -28dBc

- NS_53 and NS_54 requirements

The PA was simulated with the listed impairments and the results evaluated with the given spectral requirements. The 0dB reference was obtained by finding the maximum inband power with 1MHz aggregation bandwidth. The mask was applied accordingly. Before identifying the reference level, the carrier leakage was removed to avoid an offset in the     NR-U SEM mask. Simulations were conducted for full carrier and interlace operation. The waveform setups are found in Table1. All setups were simulated with QPSK, 16QAM, 64QAM and 256QAM. The results of the simulations are provided in Figure1 for NS_53 and Figure2 for NS_54.

|  | ID | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | DFT-s-OFDM | 20 | 100RB3 | 15 |
|  | 2 | DFT-s-OFDM | 20 | 100RB0 | 15 |
|  | 3 | DFT-s-OFDM | 20 | 50RB0 | 30 |
|  | 4 | DFT-s-OFDM | 40 | 216RB0 | 15 |
|  | 5 | DFT-s-OFDM | 60 | 162RB0 | 30 |
|  | 6 | DFT-s-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 20 | 106RB0 | 15 |
|  | 8 | CP-OFDM | 20 | 51RB0 | 30 |
|  | 9 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 10 | CP-OFDM | 60 | 162RB0 | 30 |
|  | 11 | CP-OFDM | 80 | 217RB0 | 30 |
| Interlaced Allocation Single CC | 12 | DFT-s-OFDM | 20 | 1RB0 every 10RBs (10x) | 15 |
|  | 13 | DFT-s-OFDM | 40 | 1RB0 every 10RBs (20x) | 15 |
|  | 14 | DFT-s-OFDM | 60 | 1RB0 every 10RBs (30x) | 30 |
|  | 15 | DFT-s-OFDM | 80 | 1RB0 every 5RBs (40x) | 30 |
|  | 16 | CP-OFDM | 20 | 1 RB0 every 10RBs (10x) | 15 |
|  | 17 | CP-OFDM | 40 | 1RB0 every 10RBs (22x) | 15 |
|  | 18 | CP-OFDM | 60 | 1RB0 every 10RBs (33x) | 30 |
|  | 19 | CP-OFDM | 80 | 1RB0 every 5RBs (44x) | 30 |

Table1: Signal Setup

Figure1: NS_53 Simulation results

Figure2: NS_54 Simulation results

Proposal: Remove brackets for all A-MPR found in NS_53 and NS_54

# 3	Conclusions

This paper contributes to the NR-U A-MPR discussion. Several simulations have been conducted and the following proposal is made.

Proposal: Remove brackets for all A-MPR found in NS_53 and NS_54

# 4	References

- R4-2005221, “WF on NR-U UE Tx Requirement”, RAN4#94-bis-e, Skyworks Solutions Inc, Nokia, Ericsson

- R4-1915979, “WF on NR-U spectrum emission mask”, RAN4#93, Nokia, CableLabs, Charter

- R4-2008436, “WF on NR-U MPR”, RAN4#95-e, MediaTek Inc., Skyworks Solution Inc.