3GPP TSG-RAN WG4 Meeting #97-e 	R4-2014915

# Online, November 2nd – 13th, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0381 | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-3: Corrections for intra-band contiguous EN-DC configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Intra-band contiguous EN-DC configuration DC_(n)41AB was introduced in RAN4 #94bis-e meeting through a CR (R4-2003169) which was intended for introducing new BCS for the existing EN-DC combinations, but not for brand new EN-DC configuration. This combination in principle should not be approved as it did not go through the normal TP process. In addition, the EN-DC bandwidth class “AB” has never been defined which would render DC_(n)41AB as an invalid EN-DC configuration. Since the CR had been agreed, to avoid the iterative process of removing and reintroducing the combination, we can accept to add EN-DC BW class “AB” in Rel-16 specifications to validate this configuration. We also strongly encourage proponent companies to follow the regular process when proposing any new band combinations to avoid any potential errors being overlooked. - A few intra-band contiguous EN-DC combinations were specified with non-contigous UL configurations which should not be allowed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add intra-band contiguous EN-DC BW class “AB” to Table 5.3B-1 and correct table index from 5.3.B-1 to 5.3B-1. - Remove non-contiguous UL configurations from Table 5.3B.1.2-1 and Table 5.5B.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DC_(n)41AB cannot be supported and some intra-band contiguous EN-DC combinations were incorrectly specified with non-contiguous UL configurations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3B, 5.3B.1.2, 5.5B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changed sections >>>

## 5.3B	UE Channel bandwidth for EN-DC

For intra-band contiguous EN-DC, the aggregated channel bandwidth is sum of the individual NR and E-UTRA channel bandwidths assuming nominal EN-DC channel with 0 kHz offset spacing as specified in clause 5.4.

ENBW = BWNR_Channel + BWE-UTRA_Channel

In the case where the NR sub-block and/or the E-UTRA sub-block itself is composed of intra-band contiguous CA carriers, the EN-DC aggregated channel bandwidth is the sum of the aggregated channel bandwidths of the NR and E-UTRA sub-blocks assuming nominal EN-DC channel spacing between the NR sub-block and E-UTRA sub-block.

ENBW = BWNR_Channel_CA + BWE-UTRA_Channel_CA

For NR inter-band dual connectivity specified in 5.5B.7, the corresponding NR CA configurations in 5.5A.1, i.e., dual uplink inter-band carrier aggregation between FR1 and FR2 with uplink assigned to two NR bands, are applicable to Dual Connectivity.

NOTE 1:	Requirements for the dual connectivity configurations are defined in the clause corresponding NR uplink CA between FR1 and FR2 configurations, unless otherwise specified.

Intra-band contiguous EN-DC configurations are defined using intra-band contiguous EN-DC bandwidth class notation DC_(n)Xyz where the first EN-DC bandwidth class letter y indicates the number of contiguous E-UTRA carriers and the second EN-DC bandwidth class letter z indicates the number of contiguous NR carriers for the EN-DC combination of E-UTRA Band X and NR Band nX. Applicable contiguous intraband EN-DC bandwidth classes are listed in Table 5.3.B-1.

Table 5.3B-1: Intra-band contiguous EN-DC bandwidth classes

| Intra-band contiguous EN-DC bandwidth class | Number of contiguous CC |  |
| --- | --- | --- |
|  | E-UTRA | NR |
| AA | 1 | 1 |
| CA | 2 | 1 |
| DA | 3 | 1 |
| AB | 1 | 2 |

The UE channel bandwidths for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said UE channel bandwidths for band combinations with Band n90 are not listed in the tables below but are covered by this specification.

### 5.3B.1	Intra-band EN-DC in FR1

#### 5.3B.1.1	General

The requirements for intra-band EN-DC in this specification are defined for EN-DC configurations with associated bandwidth combination sets.

For each EN-DC configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

#### 5.3B.1.2	BCS for Intra-band contiguous EN-DC

For intra-band contiguous EN-DC, an EN-DC configuration is a single operating band supporting an intra-band contiguous EN-DC bandwidth class.

Bandwidth combination sets for intra-band contiguous EN-DC are specified in Table 5.3B.1.2-1.

Table 5.3B.1.2-1: EN-DC configurations and bandwidth combination sets defined for intra-band contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)5AA | DC_(n)5AA4 | 5, 10 | 5, 10, 15, 20 |  | 25 | 0 |
|  |  |  | 5, 10, 15, 20 | 5, 10 |  |  |
| DC_(n)12AA | DC_(n)12AA4 | 5, 10 | 5, 10 |  | 15 | 0 |
|  |  |  | 5, 10 | 5, 10 |  |  |
| DC_(n)38AA | DC_(n)38AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)41AA | DC_(n)41AA | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 20, 30, 40, 50, 60, 80,100 | 10 |  |  |
| DC_(n)41AB | DC_(n)41AA, | 10 | 20+20 |  | 70 | 0 |
|  |  |  | 20+20 | 10 |  |  |
|  |  | 20 | 10+20 |  |  |  |
|  |  |  | 10+20 | 20 |  |  |
|  |  | 20 | 20+30 |  |  |  |
|  |  |  | 20+30 | 20 |  |  |
| DC_(n)41CA | DC_(n)41AA1, | 20+20 | 40, 60, 80,100 |  | 140 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 40, 50, 60, 80,100 |  | 140 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 140 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20+20 |  |  |
|  |  | 10+20 | 10, 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 10+20 |  |  |
| DC_(n)41DA | DC_(n)41AA1, | 20+20+20 | 40, 60, 80,100 |  | 160 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 40, 50, 60, 80,100 |  | 160 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 30, 40, 50, 60, 80,100 |  | 160 | 2 |
|  |  |  | 30, 40, 50, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+15 | 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 30, 40, 50, 60, 80,100 | 20+20+15 |  |  |
| DC_(n)48AA5 | DC_(n)48AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)48CA5 | DC_(n)48AA4 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)48DA5 | DC_(n)48AA4 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)71AA | DC_(n)71AA3 | 15 | 5 |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |
|  |  | 5 | 5, 10, 15 |  |  |  |
|  |  |  | 5 | 15 |  |  |
|  |  |  | 5, 10 | 10 |  |  |
|  |  |  | 5, 10, 15 | 5 |  |  |
|  |  | 5 | 5,10,15,20 |  | 253 | 1 |
|  |  | 10 | 5,10,15 |  |  |  |
|  |  | 15 | 5,10 |  |  |  |
|  |  |  | 5,10,15,20 | 5 |  |  |
|  |  |  | 5,10,15 | 10 |  |  |
|  |  |  | 5,10 | 15 |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: For maximum DL aggregated bandwidth of 25 MHz the asymmetric UL and DL channel bandwidth combination of Table 5.3.6-1 in TS 38.101-1 [2] is used with a maximum UL contiguous aggregated bandwidth of 20 MHz. Furthermore, a restriction is imposed on bandwidth combinations so that only a subset of BCS1 is allowed to be used on the uplink, and this subset is equivalent to BCS0. NOTE4: Only single switched UL is supported. NOTE5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |  |  |  |

## <<< Unchanged sections omitted >>>

### 5.5B.2	Intra-band contiguous EN-DC

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)5AA | DC_(n)5AA6 | Yes6 |
| DC_(n)12AA | DC_(n)12AA6 | Yes6 |
| DC_(n)38AA5 | DC_(n)38AA6 | Yes6 |
| DC_(n)41AA5 DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_(n)41AA | Yes3 |
| DC_(n)48AA5 | DC_(n)48AA6 | Yes6 |
| DC_(n)48CA5 | DC_(n)48AA6 | Yes6 |
| DC_(n)48DA5 | DC_(n)48AA6 | Yes6 |
| DC_(n)71AA2 | DC_(n)71AA | No4 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 3: Single UL allowed due to potential emission issues, not self-interference. NOTE 4: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. NOTE 5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 6: Only single switched UL is supported |  |  |

#### <<< End of changed sections>>>