3GPP TSG-RAN WG4 Meeting #97-e                         　　　　　　　　　       R4-2014952

Online, 2nd Nov-13th Nov, 2020

Source: 	KDDI.

Title:	TP for DC_1-18_n28

Agenda item:	10.4.2

Document for:	Approval

- Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_1A-18A_n28A and according to the request in [1].

- References

[1]	RP-202060, Revised WID on Rel-17 Dual Connectivity (DC) of 2 bands LTE inter-band CA  (2DL/1UL) and 1 NR

band (1DL/1UL).

- Text Proposal

< Start of Text Proposal>

5.x	DC_1-18_n28

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-18A_n28A | DC_1A_n28A DC_18A_n28A |

5.x.2	Co-existence studies

Based on co-existence studies of DC_1_n28 and DC_18_n28, 5th order IMD generated by dual uplink of Band 18 + Band n28 may fall into own Rx of band 1. The MSD value of this configuration can follow CA_1A-18A-28A with UL CA_18A-28A case, so no additional MSD requirement need to be specified.

5.x.3	∆TIB and ∆RIB values

For DC_1-18_n28, the TIB,c and RIB,c values are given in the tables below.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | n28 | 0.5 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-18_n28 | 1 | 0 |
|  | 18 | 0 |
|  | n28 | 0 |

5.x.4	Reference sensitivity exceptions

As stated in 5.x.2, there is no additional MSD requirement for this configuration.

< End of Text Proposal>