3GPP TSG-RAN WG4 Meeting # 97-e                                                             R4-2015043

Electronic Meeting, 2-13 Nov., 2020

Source:	ZTE Corporation

Title:	Further discussion on spectrum utilization for 35MHz and 45MHz

Agenda Item:	10.22.2

Document for:	Approval

- 1	Introduction

- In last meeting, the SU (spectrum utilizaiton) for 35MHz and 45MHz have been discussed in [1][2][3], where mathematical methods were used to deduce the SU for 35MHz and 45MHz by different companies, rather than simulation evalation.

- Although there were some different values among companies, the majority views were keep single number for all the SCS but in [], as stated in the WF [4], shown as follow.

| SCS [kHz] | 35 MHz | 45 MHz |
| --- | --- | --- |
|  | NRB | NRB |
| 15 | [188] | [243] |
| 30 | [ 92] | [119] |
| 60 | [44] | [58] |

- In this contribution, we further discuss the SU for 35MHz and 45MHz.

- 2	Discussion

As we know, NRB is directly related to SU,which are the bacis parameter for the other RF requirements. For example some of the BS Rx requirements and MPR/A-MPR also Rx requirements of UE are pending on the NRB decision.

As stated in the moderator summary document [5], the reason for the values in square bracket was that the values may be needed to be further checked by some companies in this meeting, even the values highlighted in green may also need to be discussed in this meeting if needed, as commented by companies in the 1st round e-mail discussion. However, there was another agreed WF [6] on UE RF requirements, in which the generic requirements were captured based on the 15kHz SU in the table, which means the above SU for 15kHz were implicitly approved.

In addition, according to the work plan, the main work in this meeting are:

- By end of RAN4#97-e (November 2020):

- Draft CRs for BS and UE specifications for generic requirements

- To reach agreement on UE reference sensitivity

- MPR/A-MPR simulation when needed

It can be foreseen that companies will use the above SU values to continue their work although the values are in square bracket, also considering it may inefficient to discuss SU and other topics in parallel, it is crucial to agree the SU as soon as possible, so we think if there are no other techinical concerns, the values in the tables should be kept unchanged, i.e. removal of [] .

Proposal 1. Removal of [] for the SU values for 35MHz and 45MHz in the table.

- 3	Conclusion

- In this paper,  we give further discussion on spectrum utilization for 35MHz and 45MHz NR channel bandwidths.

Proposal 1. Remove [] for the SU values for 35MHz and 45MHz in the table.

- That is the spectrum utilization for 35MHz and 45MHz are:

| SCS (kHz) | 35MHz | 45MHz |
| --- | --- | --- |
|  | NRB | NRB |
| 15 | 188 | 243 |
| 30 | 92 | 119 |
| 60 | 44 | 58 |

- 4	Reference

[1] R4-2010265	Spectrum Utilization for 35MHz and 45MHz Channels in FR1 and their Support, Skyworks Solutions Inc.

[2] R4-2010503	Spectrum utilization for 35MHz and 45MHz, Huawei

[3] R4-2010635	On spectrum utilization for new channel bandwidth of 35MHz and 45MHz, ZTE

[4] R4-2011792 	WF on Spectrum utilization for 35 MHz and 45 MHz, ZTE

[5] R4-2011868   Email discussion summary for [96e][130] NR_FR1_35MHz_45MHz_BW, Huawei

[6] R4-2011793   WF on UE RF requirements, Skyworks Solutions Inc.