3GPP TSG-RAN WG4 Meeting # 97-e                                                           R4-2015047

# Electronic Meeting, 2-13 Nov., 2020

Source:	ZTE Corporation

Title:	TP for 37.717-11-21_ DC_40_n41-n258

Agenda Item:	10.7.3

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC band combination of DC_40_n41-n258 for TR37.717-11-21[1].

- Reference

- TR37.717-11-21, Dual Connectivity (EN-DC) of x bands (x=1, 2, 3, 4)  LTE inter-band CA (x DL/1 UL) and 2 bands NR Inter-band CA  (2 DL/1 UL) band combinations, v0.1.0

- Text Proposal

----- Start of TP -----

- 6.x				DC_40_n41-n258

- 6.x.1		Operating bands for DC

Table 6.x.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_40_n41-n258 | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n257 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

    - 6.x.2		Configuration for DC

Table 6.x.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_40A_n41A-n258A DC_40A_n41C-n258A | DC_40A_n41A DC_40A_n258A |

- 6.x.3		Co-existence studies

- For combination of DC_40_n41, the co-existence studies have been covered in TR37.716-11-11, where band 40 and band n41 are synchronized operaiton.

- For combination of DC_40_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

- 6.x.4	∆TIB and ∆RIB values

For the DC conbination of DC_40_n41-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 40 and n79 are the same with the DC_40_n41.

- 6.x.5	MSD

No additional MSD requirement is needed.

----- End of TP -----