3GPP TSG-RAN WG4 Meeting # 97-e                                                           R4-2015050

# Electronic Meeting, 2-13 Nov., 2020

Source:	ZTE Corporation

Title:	TP for 37.717-11-31_ DC_8A_n40A-n41A-n79A

Agenda Item:	10.13.3

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC band combination of DC_8A_n40A-n41A-n79A for TR37.717-11-31[1].

- Reference

- TR37.717-11-21, Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL, v0.1.0

- Text Proposal

----- Start of TP -----

- 6.x			DC_8A_n40A-n41A-n79A

- 6.x.1	Operating bands for DC

Table 6.x.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n40-n41-n79 | 8 | CA_n40-n41-n79 |

### 6.x.2	Inter-band DC Configurations

Table 6.x.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n40A-n41A-n79A | DC_8A_n40A DC_8A_n41A DC_8A_n79A |

- 6.x.3	Co-existence studies

For DC_8A_n40A-n41A-n79A co-existence studies, the lower order DC_8A_n40A-n41A, DC_8A_n40A-n79A and DC_8A_n41A-n79A can be applied.

- 6.x.4	∆TIB and ∆RIB values

For DC_8A_n40A-n41A-n79A , the TIB,c and RIB,c values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n40-n41-n79 | 8 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |
|  | n79 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n40-n41-n79 | 8 | 0 |
|  | n40 | 0 |
|  | n41 | 0 |
|  | n79 | 0 |

- 6.x.5	MSD

No additional MSD requirement is needed.

----- End of TP -----