3GPP TSG-RAN WG4 Meeting # 97-e                                                             R4-2015056

# Electronic Meeting, 2-13 Nov., 2020

Source:	ZTE Corporation, CMCC

Title:	TP for TR38.xxx_ PC2 CA_n40A-n41A

Agenda Item:	10.19.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce PC2 2DL/2UL CA_n40A-n41A in TR38.XXX[1].

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n40A-n41A requirements are applied.

- Reference

- TR38.xxx, High power User Equipment (UE) (power class 2) for NR inter-band Carrier Aggregation with 2 bands downlink and 2 bands uplink; (Release 17), v0.0.1

- Text Proposal

----- Start of TP -----

  - 5.x			CA_n40-n41

    - 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n40A-n41A | CA_n40A-n41A | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

    - 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n40A-n41A | CA power class | Carrier n40 power class | Carrier n41 power class |
| --- | --- | --- | --- | --- |
| CA_n40A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

    - 5.x.3	REFSENS requirements

According to the PC3 CA_n40A-n41A, band n40 and band n41 are sychronous operation. Therefore, no MSD requirements are defined for PC3 CA_n40A-n41A. The same approach can be applied to PC2 CA_n40A-n41A

      - 5.x.3.1	Power class 2 case a

No additional MSD are expected for this PC2 CA_n40A-n41A with dual uplink carrier.

      - 5.x.3.2	Power class 2 case b

No additional MSD are expected for this PC2 CA_n40A-n41A with dual uplink carrier.

      - 5.x.3.3	Power class 2 case c

No additional MSD are expected for this PC2 CA_n40A-n41A with dual uplink carrier.

      - 5.x.3.4	Power class 2 case d

No additional MSD are expected for this PC2 CA_n40A-n41A with dual uplink carrier.

    - 5.x.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n40A-n41A requirements are applied for PC2 CA_n40A-n41A.

----- End of TP -----